3GPP TSG-RAN WG4 Meeting # 116						       R4-2510846

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.5

Source: 	OPPO

Title: 	On OTA aspect

Document for:	Approval

- Introduction

The ambient-IoT topic has been fully discussed during the last RAN4 meeting. A WF [1] has been agreed and further discussion on the OTA aspect is provided in this paper.

- Discussion

During the RAN4#115 meeting, the agreed WF[1] has been captured as below:

Issue 4-2-2: Isolation between CW node and measurement antenna

Agreement in RAN4#115:

- Power difference between received backscatter signal and the CW signal leakage is at least [15dB]

  - FFS on reference point of power difference

Issue 4-2-3: Polarization of CW antenna

Agreement in RAN4#115:

- The CW signal shall be sent in two polarized sequentially, FFS on how to combine two polarization test results.

During the last meeting offline discussion of AIOT OTA test method, it has been confirmed from the TE vendors that the CW signal and the reader can be co-located. In such case, the test of transmitted output power as well as the SEM and spurious emission can all be carried out for the peak gain direction of device. In the meanwhile, because of the co-location, the power leakage of CW to the test receiver needs to be small enough that will not influent the receiver performance. The 15dB power difference should be calibrated at the measurement antenna.

Proposal 1: The reference point of power difference can be calibrated at the measurement antenna.

For the polarization of CW antenna, it has been agreed to send the two polarized signal sequentially. To combine the two polarization test results, denote the CW output power as PCW, then the vertical power is PCWV. For a PCWV, the measurement antenna needs to measure horizontal and vertical two times and add together to get the PreflectedV. Similarly the horizontal power of CW is PCWH. For a PCWH, he measurement antenna needs to measure horizontal and vertical two times and add together to get the PreflectedH. Then the reflected total power Preflected = PreflectedH + PreflectedV.

Proposal 2: To combine the two polarization of CW signal, test each polarization once with measurement antenna at horizontal and vertical polarization and add them together.

Furthermore, a question has been mentioned that if the polarization of the measurement antenna and CW antenna are directly vertical to the tag polarization which means no power can be received or reflected. This situation can be figured out during the test since no power can be received from the measurement antenna and hence to rotate the CW antenna a small angle can solve this problem. In spec usually we just need to mention that the reflected power is correctly received by the measurement antenna and it should be enough.

Proposal 3: In spec, to mention that the reflected power is correctly received by the measurement antenna to avoid the total vertical case.

For the REFSENS requirement, it has been proposed that a new performance metric will be used as the hemi-sphere will be tested. Below figure 1 has been proposed during the offline discussion. From OTA test perspective, on the plane of the theta = 60°, it is proposed to test 4 points around the circle is enough to guarantee the antenna pattern and coverage.

Figure 1 REFSENS test points

Proposal 4: For REFSENS requirement, define 4 extra test points at theta = 60° plane to verify the antenna pattern and coverage.

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Proposal 1: The reference point of power difference can be calibrated at the measurement antenna.

Proposal 2: To combine the two polarization of CW signal, test each polarization once with measurement antenna at horizontal and vertical polarization and add them together.

Proposal 3: In spec, to mention that the reflected power is correctly received by the measurement antenna to avoid the total vertical case.

Proposal 4: For REFSENS requirement, define 4 extra test points at theta = 60° plane to verify the antenna pattern and coverage.

4 References

[1] R4-2508116	WF on A-IOT device requirements, CMCC

TP to TS 38.191

8.1	General

8.1.1	Testing bands

The testing bands are based on operating bands as specified in sub-clause 5.2. The frequency ranges to be tested will be all low, middle and high frequency ranges. The detailed testing parameters as the channel bandwidth, D2R and R2D configuration for each band is defined by RAN5.