3GPP TSG-RAN WG4 Meeting # 116						        R4-2510848

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.24.3.2

Source: 	OPPO

Title: 	On LPWUS RX requirement

Document for:	Approval

- Introduction

During the last RAN4 meeting, the RX requirement sets for two sets have been discussed and still no conclusion has been drawn. Some implementation discrepancy has been observed and hence the requirement sets have not been agreed. Furthermore, during the last RAN1 meeting, there is still one open issue left for RAN4 discussion as the limitation of frequency gap of SSB and LP-WUS signal. From our point of view, this two aspects can be discussed together to further solve both problem.

- Discussion

During the RAN1#121 meeting, the agreement on the limitation of frequency gap of SSB and LP-WUS signal has been captured as below:

R1-2504724	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Tuesday session

Agreement

From RAN1 perspective, there is no restriction on the maximum frequency bandwidth covering both LP-WUS/LP-SS and SSB.

- It is up to RAN4 to introduce any restriction depending on their discussions.

- Above applies for IDLE and INACTIVE mode UEs.

From RAN1 understanding, there is no restriction on the maximum frequency bandwidth covering both LP-WUS/LP-SS and SSB. However, if the LP-WUR would like to receive the LP-WUS/LP-SS and SSB simultaneously, then the RX bandwidth of the LP-WUR should at least cover both the LP-WUS/LP-SS and SSB, otherwise the RX LO retuning is needed to change the centre frequency of the receiver from LP-WUS/LP-SS and SSB. This, from our understanding can be a UE capability based on the receiver type the UE use.

Observation 1: The ability to receive LP-WUS and SSB simultaneously with a large RX channel bandwidth can be a UE capability.

We try to link this together with the UE types since in legacy discussion, use different waveform to differentiate the LP-WUR type has been proposed while not accepted for many meetings. The most reason why is that company believe that a LP-WUR can receive both OOK-based and OFDM-based signal and hence in this case it is hard to say what type of receiver is if the criteria is the signal waveform. From our point, there will be three cases that needs to be defined.

1, OOK-based only receiver.

2, OFDM-based receiver with low capability. (can support or not support OOK-based LP-WUS)

3, OFDM-based receiver with high capability. (can support or not support OOK-based LP-WUS)

Currently there are two sets of requirements at least from REFSENS definition as set 1 and set 2 requirement. For set 1 requirement it is derived from OOK signal with worse REFSENS and for set 2 requirement it is derived from OFDM signal with better REFSENS. For the 1st case as OOK-based only receiver, it can be agreed that set 1 requirement apply. For the 3rd case as OFDM-based receiver with high capability, it can be agreed that set 2 requirement apply. The unclear part is that for the middle case as OFDM-based receiver with low capability, which set of requirement apply and further, how to define low and high capability for the receiver.

Observation 2: For 1st case as OOK-based only receiver, set 1 requirement apply.

On the above question as how to differentiate the high/low capability of the OFDM-based receiver (or OFDM and OOK both), the RX channel bandwidth can be the key factor from both power saving perspective and UE complexity perspective. If the OFDM-based receiver can support the RX frequency span as the whole channel bandwidth then mostly the case is the receiver is a high capability receiver that can fulfil the set 2 requirement. However, if the OFDM-based receiver can only support small frequency span then it means it is a low capability receiver. Still, some might argue that if a high capability UE who reports lower capability and hence it only fulfils less stringent requirement. This problem exists not only for LP-WUS feature but also other features. If this is the case, then the UE can only work in low capability with less coverage and interference tolerance but with more complex and expensive implementation which should not be the case.

Figure 1 LPWUS and SSB separation

Observation 3: For OFDM-based receiver, it can be further differentiated into two types with high and low capability.

Proposal 1: To differentiate the OFDM-based receiver with its frequency span support of the separation of SSB and LP-WUS/LP-SS.

For the UE RF requirement, it has been mentioned previously that for set 1 and set 2, two sets of requirements are defined. For the OFDM-based receiver with low capability, we propose to use set 1 requirement. One argument is that for set 1 requirement of REFSENS and also ACS/ASCS is derived based on OOK-based receiver, and hence the OFDM-based receiver should not follow the same requirement.

For the REFSENS requirement, in the last RAN4 meeting, the SNR has been agreed as captured in [1]. Further with the noise figure + IM which has been agreed in earlier RAN4 meeting, then the REFSENS can be calculated based on the thermal noise equation.

Both envelop detection and OFDM-based receiver will use -4.5dB as SNR. The IM+NF is 17.5dB and 13.5dB respectively and in total it should be 13dB and 9dB. Together with the NR OFDM receiver, the IM+NF is 11.5dB while the SNR is -1dB so in total it will be 10.5dB. In such case, the REFSENS for set 1 requirement will be 2.5dB worse than NR REFSENS requirement while the set 2 requirement will be 1.5dB better than NR REFSENS requirement. The reason here is the LPWUS signal is easier to decode and hence less SNR is required. From our point of view, the set 1 requirement with less stringent requirement can be used for both OOK-based and OFDM-based receiver. This low capability OFDM-based receiver with low cost implementation can only fulfil such requirement and hence corresponding capability has been reported and deployment of such is scheduled by network.

Proposal 2: The low capability OFDM-based receiver fulfils set 1 requirement.

Proposal 3: The high capability OFDM-based receiver fulfils set 2 requirement.

Issue 2-1-1: SNR for LP-WUR two types

Agreements:

- There is no repetition of information bits to derive OFDM-based receiver SNR.

- SNR=-4.5dB for envelope detection and OFDM-based receiver.

Issue 2-1-1: SNR, NF and IM value for FR1 OOK-based LR

Agreement:

- For the FR1 requirements targeting at bands <2.5GHz

  - The IM+NF values are

    - Set 1: [17.5~18dB]

    - Set 2: [13.5dB]

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Observation 1: The ability to receive LP-WUS and SSB simultaneously with a large RX channel bandwidth can be a UE capability.

Observation 2: For 1st case as OOK-based only receiver, set 1 requirement apply.

Observation 3: For OFDM-based receiver, it can be further differentiated into two sets with high and low capability.

Proposal 1: To differentiate the OFDM-based receiver with its frequency span support of the separation of SSB and LP-WUS/LP-SS.

Proposal 2: The low capability OFDM-based receiver fulfils set 1 requirement.

Proposal 3: The high capability OFDM-based receiver fulfils set 2 requirement.

4 References

[1] R4-2507936	WF on NR_LPWUS_UERF, VIVO