3GPP TSG-RAN WG4 Meeting #116	R4-2510864

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Discussion on NTN testing for NGSO channel model

Source: 	Huawei,HiSilicon

Agenda item:	7.6.5.1

Document for:	Discussion

Background

During RAN4#115 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model_and_demod was approved. In this contribution, we share our view on NTN testing for NGSO channel model.

Discussion

FFS to consider the Earth’s rotation during signal flight time for more accurate estimations of propagation delay

- agreement - Downlink propagation delay model at UE reception time tR with considering the Earth's rotation during signal flight time DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF - FFS whether and how to consider the propagation delay model for UL delay error for RRM uplink timing error cases - Option 1: ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT#Where SV (Satellite) : Receiver UE : Transmitter ∆tUL= tR- tT : Uplink total propagation delay tT : Time when signals are transmitted from UE tR : Time when signals are received by satellite rR : SV position vector on ECI Frame at tT. rT : UE position vector on ECI Frame at tT. VSV : Satellite velocity vector on ECI Frame RUL=rR-rT : Distance between satellite and UE at either tT or tR depending on the reference point. c = 2.99792458 x 108 [m/s] - Other options are not precluded

From our understanding, the UL delay error should be considered, since there are RRM uplink timing error cases to be defined. Based on equation as Option 1, the UL delay error is shown as following Figure 2.1-1. We can see that the UL delay error is obvious comparing the timing error limit defined in TS 38.133, i.e. 29*64*Tc = 9.44e-7s.

Figure 2.1-1 UL Delay error

Consider UL delay error since there are RRM uplink timing error cases to be defined. Option 1 should be applied for the UL delay error.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- Consider UL delay error since there are RRM uplink timing error cases to be defined. Option 1 should be applied for the UL delay error.

Reference

R4-2508625, Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod, RAN4#115, Samsung