3GPP TSG-RAN WG4 Meeting #116	R4-2510870

# Bengaluru, IN, 25 - 29 Aug, 2025

Title: 	Simulation results on IoT NTN Phase3 SAN demodulation performance requirements

Source: 	Huawei,HiSilicon

Agenda item:	7.28.6

Document for:	Information

Background

During RAN#106 meeting, Way Forward [1] for IoT_NTN_Ph3_demod was approved. In this contribution, we provide our initial simulation results on IoT NTN Phase3 SAN demodulation performance requirements.

Simulation

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC (Annex A) | Frequency offset | TxDuration | Repetition number | SNR@70% max throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | -3.9 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | -8.0 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 128 | 8 | -5.2 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 128 | 8 | -9.7 |
| Note 1: The MCS is defined in the Annex A.7.1 of TS 36.108. Note 2: The MCS is defined in the Annex A.16.1 of TS 36.104. |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, we provide our initial simulation results on IoT NTN Phase3 SAN demodulation performance requirements.

Reference

R4-2508756, Way Forward for [115][334] IoT_NTN_Ph3_demod, RAN4#115, ZTE Corporation