3GPP TSG-RAN WG4 Meeting #116	  R4-2510873

Bengaluru, India, August 25 – 29, 2025

Agenda Item:	7.17.4

Source:	Huawei, HiSilicon

Title:	Discussion on RRM core requirement for AIML beam management

Document for:	Discussion and Decision

# Introduction

According to WF in RAN4 #115 [1], we discuss the remaining issues related to the RRM core requirement for beam management.

# Prediction delay requirements

RAN4 #113 WF [1] Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

Both measurement period and inference delay need to be considered for prediction delay definition. The measurement delay is related to the number of measurement instances required for BM-Case 1 and BM-Case 2. The inference delay is related to the complexity of AI model.

Compared to legacy, the number of measurement instances could be different in AI. For example, in BM-Case 2, several measurement periods may be needed for model inputs. However, if measurement result reporting is not expected in AI prediction, there is no need to clearly distinguish the delay of measurement and the delay of inference. Instead, they can be treated as a whole prediction delay.

Proposal 1: RAN4 to define the requirement of prediction delay in AI BM, where the measurement delay and inference delay are not treated as separate requirements.

# Data collection related requirements

Data collection is involved in model inference, model monitoring and model training. The reporting of data collection is different for UE-sided model and for NW-sided model.

- NW-sided model

In RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are reached related to NW-sided model.

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

It is observed that the purpose of the beam report initiated by network will not be specified in RAN1. If the purpose will also not be specified by other WGs, RAN4 can focus on the common requirement of potential new reporting or enhanced reporting, without differentiating the purpose of the reporting.

Based on the agreements, the reporting is the beam related information, e.g., L1-RSRPs and beam index. RAN4 can use the existing core requirement as the starting point.

Proposal 2: For data collection in NW-sided model, take the existing core requirement for beam related information reporting as the starting point.

- UE-sided model

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement - For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. - where the set of beams is Set A, i.e., the beams for UE prediction. Agreement - For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement - For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption - For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement - For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

For UE-sided model, new reporting will be introduced. The content could be the beam information of the predicted beams. Since the content is still under RAN1 study, RAN4 can wait for more RAN1 progress. At current stage, potential requirements are identified for reporting delay and prediction accuracy.

# LCM related requirements

In RAN1#118 the following agreements are reached related to performance monitoring.

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In LCM procedure, the functionality/model activation/deactivation/fallback to non-AI function are to be considered.

For UE-sided model, if the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., triggered by gNB via RRC/MAC-CE/DCI signaling, the delay of activating/deactivating one function can be defined. Otherwise if these functionality/model changes are autonomously performed inside UE, there may be a need to monitor the function performance, then the requirements would be reflected by performance monitoring requirements.

For model monitoring, two options are identified in RAN1. For Option 1, UE needs to report the measurement. This can reuse the legacy requirement of measurement reporting. For Option 2, UE will calculate the performance metric(s) then report.

TR 38.843: For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Type 1 performance monitoring: - Configuration/Signalling from gNB to UE for measurement and/or reporting - UE may have different operations - Option 1 (NW-side performance monitoring): UE sends reporting to NW (e.g., for the calculation of performance metric at NW) - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s) - Indication from NW for UE to do LCM operations - Note: At least the performance and reporting overhead of model monitoring mechanism should be considered - Type 2 performance monitoring: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signalling from gNB to UE for performance monitoring measurement and/or reporting - If it is for UE side model monitoring, UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Mechanism that facilitates the UE to detect whether the functionality/model is suitable or no longer suitable RAN1 #121: Agreement - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. - Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately - The associated working assumption made in RAN1#120b will not be confirmed.

It is observed that for monitoring methods, both UE-assisted performance monitoring Type-1 option 2 and NW-side performance monitoring Type 1 option 1 are supported. RAN1 has achieved a list of agreements on specifications for supporting Type-1 option 2. However, Type 1 option 1 is more straightforward where legacy measurement and reporting scheme can be reused. RAN4 will focus on Type 1 option 1 monitoring where legacy requirements can be reused.

Proposal 3: RAN4 will focus on NW-side performance monitoring Type 1 option 1 and reuse legacy requirements for measurement reporting.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 to define the requirement of prediction delay in AI BM, where the measurement delay and inference delay are not treated as separate requirements.

Proposal 2: For data collection in NW-sided model, take the existing core requirement for beam related information reporting as the starting point.

Proposal 3: RAN4 will focus on NW-side performance monitoring Type 1 option 1 and reuse legacy requirements for measurement reporting.

# References

- R4-2508080, “WF on AI ML for air interface study”, 3GPP RAN4 #115, 2025.

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.