3GPP TSG-RAN WG4 Meeting #116	R4-25xxxxx

# Bengaluru, India, August 25th – 29th, 2025

Title: 	Simulation results on spatial channel model for demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.12.3

Document for:	Discussion

Background

In last meeting, a WF and a document on simulation assumptions for SCM were agreed. This contribution provides our related simulation results.

- PDSCH results

Figure 2-1 provides PDSCH results.

Figure 2-1: PDSCH results

Table 2-1 provides simulation results for CDL channel.

Table 2-1: Simulation summary for CDL results

| Channel model | Rank4 |  | Rank8(Random) |  |  |  | Rank8(Fixed) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CW0 |  | CW1 |  | CW0 |  | CW1 |  |
|  | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP |
| A4 | 6.2 | 15.6 | 4.7 | 16.5 | 10.0 | 24.3 | 6.7 | 16.3 | 9.2 | 20.2 |

Table 2-2 provides simulation results for enhanced TDL channel.

Table 2-2: Simulation summary for extension TDL results

| Channel model | Rank4 |  | Rank8 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CW0 |  | CW1 |  |
|  | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@30% of maxTP | SNR(dB)@70% of maxTP |
| Cluster model 1 | 5.9 | 15.1 | 1.4 | 9.7 | 10.4 | 18.7 |
| Cluster model 2 | 5.9 | 14.5 | 4.3 | 14.2 | 9.9 | 20.1 |
| Cluster model 3 | 5.3 | 13.5 | 4.0 | 14.5 | 10.8 | 21.2 |

- PMI results

Figure 3-1 provides PMI results.

Figure 3-1: PMI results

Table 3-1 and 3-2 provide simulation summary for PMI results.

Table 3-1: Simulation summary for PMI

| Chanel model | Rank2 |  |  |  |  |  | Rank4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random PMI |  | Type 1 PMI |  | eType 2 PMI |  | Random PMI |  | Type 1 PMI |  | eType 2 PMI |  |
|  | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP | SNR(dB)@70% of maxTP | SNR(dB)@90% of maxTP |
| A4 | 8.1 | 9.8 | 1.8 | 4.1 | 2.1 | 3.9 | 16.9 | 19.1 | 10.3 | 12.8 | 11.6 | 14.0 |
| Cluster model 1 | 7.3 | 8.1 | 3.8 | 5.1 | 2.5 | 3.6 | 15.6 | 16.7 | 12.1 | 13.5 | 13.6 | 15.1 |
| Cluster model 2 | 6.5 | 8.0 | 3.1 | 4.9 | 1.7 | 3.6 | 15.5 | 17.1 | 12.4 | 14.7 | 12.6 | 15.2 |

Table 3-2: Gama value for PMI

| Chanel model | Rank2 |  | Rank4 |  |
| --- | --- | --- | --- | --- |
|  | Type 1 PMI | eType 2 PMI | Type 1 PMI | eType 2 PMI |
| A4 | 1.96 | 2.02 | 1.65 | 1.78 |
| Cluster model 1 | 1.80 | 1.88 | 1.79 | 1.48 |
| Cluster model 2 | 1.73 | 1.88 | 1.48 | 1.36 |

- Conclusion

In this contribution we provide our simulation results for NR SCM for demodulation requirements.

- Reference

[1] R4-2508776 Way Forward for [115][321] NR_SCM. Nokia