3GPP TSG-RAN WG4 Meeting #116	R4-2510892

Bengaluru, India, August 25th – 29th, 2025

Title: 	On demodulation and CSI requirements for 7MHz bandwidth

Source: 	Huawei, HiSilicon

Agenda item:	7.24.6

Document for:	Discussion

Background

This contribution provides our views on demodulation and CSI requirements for 7MHz bandwidth.

- Discussions

Whether to define PDSCH CA requirments for 7MHz bandwidth

In last meeting, there was an open issue that how to define requirements for 7MHz bandwidth:

 FFS how to define requirements: - Option 1: Define new requirements for 7MHz CC based on the simulations - Option 2: Reuse existing 5MHz/10MHz requirements

Some companies express the concern that existing 5MHz and 10MHz have exact same requirements, it will cause the ambiguity if 7MHz requirements are not aligned with 5MHz and 10MHz. Based on our understanding, there is no technical reason that 7MHz requirements shall be aligned with 5MHz/10MHz.  Our proposal is that if enough results are provided in this meeting (At least 3 companies provide simulation results), the requirements shall be defined based on the new results, otherwise, existing 5MHz/10MHz requirements shall be reused.

Proposal 1: If at least three companies provide simulation results for 7MHz, the requirements shall be defined based on the new results, otherwise, reuse 5MHz/10MHz requirements

We provide our simulation results in Table 2-1:

Table 2-1: Simulation results for 7MHz bandwidth

| Number of Rx | SNR(dB) @70% of maximum TP |
| --- | --- |
| 2Rx | 10.4 |
| 4Rx | 5.2 |

Release independence for 7MHz bandwidth requirements

There is an open issue that whether to introduce release independency for 7MHz requirements:

- Proposals - Option 1: RAN4 to follow the discussion on release independent for support of 7MHz in RF session, once it is agreed, it should also apply to demodulation and CSI requirements.

RF has agreed to introduce release independency for 7MHz and RAN2 is discussing the signalling structure. There is no 7MHz requirements in Rel-15 spec, so there may be a risk that no requirements will be applied if a CC with 7MHz is selected to be tested. Therefore, we slightly prefer to introduce release independency.

Proposal 2: Introduce release independency for 7MHz requirements

- Conclusion

This contribution provides our views on 7MHz demodulation requirements. The proposals are:

Proposal 1: If at least three companies provide simulation results for 7MHz, the requirements shall be defined based on the new results, otherwise, reuse 5MHz/10MHz requirements

Proposal 2: Introduce release independency for 7MHz requirements

- Reference

[1]   R4-2508757 WF on demodulation and CSI requirements for 7MHz