3GPP TSG-RAN WG4 Meeting #116	R4-2510893

Bengaluru, India, August 25th -29th, 2025

Title: 	On performance requirements for LP-WUS

Source: 	Huawei, HiSilicon

Agenda item:	7.28.7

Document for:	Discussion

Background

In last meeting, a WF for NR_LPWUS_Demod was agreed, this contribution provides our views on related open issues.

- Discussions

One issue is whether to define a test mode for LP-WUS/WUR:

Issue 2-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement - Take RF approach as a starting point to test MDR for demodulation requirements. RF agreement Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - FFS whether test mode is recommended for MDR testing.

Counting MDR based on ACK/NACK in connected state or MSG1/3 in IDLE state may not verify LP-WUS correctly since UE has a chance to cheat. For example, a UE using MR rather than LR to monitor PDCCH can also report ACK/NACK in connected mode or transmit MSG1/3 in IDLE mode. Therefore, a better approach is to introduce a test mode that enables UE to directly report to the TE the number of detected LP-WUS signal corresponding to the group ID belongs to the UE. The correctly detected number of LP-WUS compared to the total number of LP-WUS transmitted by TE should be equal or less than 1%. The details are up to RAN5

Proposal 1: Introduce a test mode that enables UE to directly report to the TE the number of detected LP-WUS signal corresponding to the group ID belongs to the UE. The details are up to RAN5.

One issue is whether to introduce the FAR test.

Issue 2-1-2: Whether to consider shortening the test duration (if a test is introduced) - Agreement: - FFS whether to introduce FAR test and further study feasibility of introducing FAR test - FFS whether test mode is recommended for FAR testing.

It’s proposed not to introduce FAR test, high FAR means high power consumption, which is less important comparted to high MDR which means high missing data or paging occasion.  However, FAR should be specified for MDR simulation alignment, we propose 1%.

Proposal 2: Not to introduce dedicate FAR test. For the MDR simulation alignment, set FAR to 1%.

One issue is whether to define separate test cases for idle/inactive and connected modes

- WF - Option 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LP-WUR, which mode is selected for test should be up to UE declaration.

Supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa. Hence the test mode should support both connected and IDLE models. UE could detect LP-WUS and count number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Proposal 3: UE detects LP-WUS and counts number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Receiver type

One issue is receiver type:

Issue 3-1-7: Receiver type - WF - Option 1: Consider both OOK and OFDM receivers

Supporting OOK or OFDM receiver is based on UE capability, it’s proposed to consider both to cover both UE capabilities.

Proposal 5: Cover both OOK and OFDM receiver.

LP-WUS waveform

One issue is LP-WUS waveform:

- WF Option 1: Consider both OOK-1 and OOK-4

We don’t see any need to consider both OOK-1 and OOK-4 since they actually belong to one waveform type and the only difference is number of chips per OFDM symbol with no UE behaviour changes. We slightly prefer OOK-4

Proposal 4: Only consider OOK-4 waveform.

Frequency range

One issue is frequency range:

- WF - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2.

Both FR1 and FR2 are supported according to RAN1 agreements, but we see FR1 will be more popular for LP-WUS, hence, option 2 is preferred.

Proposal 6: Consider option 2. Prioritize FR1; FFS FR2

Issue 3-1-14: Channel model

One issue is channel model:

- WF - Option 1: TDLA30-10 only - Option 2: TDLC300-100 only - Option 3: Both TDLC300-100 and TDLA30-10

It’s proposed to only consider one channel model. TDLC300-100 is preferred.

Proposal 7: Only consider TDLC300-100

Time offsets

One issue is time offset:

- WF - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

According to RRM requirements, the time error is defined as following:

Time error= Residual time error + time drift (frequency offset*320ms) T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

We prefer to reuse it for demodulation test.

Proposal 8: RAN4 to use following for demodulation test:

Time error= Residual time error + time drift (frequency offset*320ms)

T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Frequency offsets

One issue is frequency offsets, the options are:

- WF - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm - Option 1: up to 20 ppm

According to [2], Option 1 is used for RAN1 and RRM simulation, which can also be used for demodulation requirements assumptions. Also, RAN4 could use centre frequency 900MHz for FDD and 2.6GHz for TDD as starting point.

Proposal 9: Confirm option 1, For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm. Use fc=900MHz for FDD and fc=2.6GHz for TDD as starting point.

Whether to define performance requirements for LP-SS

One issue is whether to define performance requirements for LP-SS:

- WF - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

LP-SS has the same function as NR PSS/SS which is used for synchronization and measurement. We propose to only test LP-WUS since RAN4 has never introduced requirements for synchronous signal.

Proposal 10: Not to introduce performance requirements for LP-SS.

Whether to define single requirement for Type 1 and Type 2 receivers

One issue is whether to define single requirements for Type 1 and 2 receivers:

- WF - Option 1: Further discuss based on simulation results if one or two sets of requirements are to be defined

We don’t see the relationship between demodulation requirements and Type1/2 receiver capability.  Type1 and 2 receivers are differentiated according to power consumption, which has nothing to do with baseband processing. We don’t support defining such capability rules for demodulation requirements.

Proposal 11: Don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

Coding/decoding algorithms for LP-WUS

- WF - Option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding

Coding/decoding algorithm depends on different number of information bits, small block coding is supported for 1 or 2 information bits and 3 or more information bits is supported for RM coding. Considering decoding algorithm is key part of baseband processing. It’s proposed to consider option 1.

Proposal 12: Support option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding.

- Conclusion

In this contribution we provide our views on performance requirements for LP-WUS, the proposals are:

Proposal 1: Introduce a test mode that enables UE to directly report to the TE the number of LP-WUS signal corresponding to the group ID belongs to the UE. The details are up to RAN5.

Proposal 2: Not to introduce dedicate FAR test. For the MDR simulation alignment, set FAR to 1%.

Proposal 3: UE detects LP-WUS and counts number of correct detected group IDs in connected or IDLE mode based on UE’s capability and report the results to TE in connected mode.

Proposal 4: Only consider OOK-4 waveform.

Proposal 5: Cover both OOK and OFDM receiver.

Proposal 6: Consider option 2. Prioritize FR1; FFS FR2

Proposal 7: Only consider TDLC300-100

Proposal 8: RAN4 to use following for demodulation test:

Time error= Residual time error + time drift (frequency offset*320ms)

T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

Proposal 9: Confirm option 1, For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm. Use fc=900MHz for FDD and fc=2.6GHz for TDD as starting point.

Proposal 10: Not to introduce performance requirements for LP-SS.

Proposal 11: Don’t define any capability rules related to Type1/2 receivers for demodulation requirements.

Proposal 12: Support option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding.

- Reference

[1] R4-2508626_WF for [115][332] NR_LPWUS_Demod. Vivo