3GPP TSG-RAN WG4 Meeting #116						R4-2510906

Banalore, India, 25th – 29th August 2025

Source:	Nokia, Ericsson

Title:	Progress update and next steps for the ETSI MCC band combination database

Agenda item:	10

Document for:	Discussion

Introduction

At RAN#102, it was highlighted that there is a need for a new tool/methodology for handling the specification of CA combinations in RAN4. It is widely recognised that tens of thousands of band combinations cannot be specified efficiently using tables in DOCX files. And the number of combinations continues to grow exponentially – a situation expected to continue in the 6G timeframe.

RAN#102 commissioned an activity to identify a new methodology with a small working party with a small number of expert delegates from RAN4 in conjunction with ETSI MCC. A solution was identified together with ETSI MCC, and work on enabling this is now ongoing.

At the joint RAN#108 and SA#108 meeting in June 2025, a joint Study Item [1] was agreed which can be seen as an extension of the work initiated by the work on the band combination database for simplifying the RAN4 specifications. The scope of the new SI spans beyond the work related to the ETSI MCC band combination database; even the developed database can be integrated into any new framework for 3GPP specifications.

Status updates on the work have previously been presented with the latest in [2], which also contains some background information. This document presents the current state of progress of this activity.

Progress Update

Since the last status update at RAN4#115 in [2], the work continues in three main areas:

- Creation of a guide on working with the JSON data and schema files.

- Updates of the Web-App

- Generation of JSON data files containing the DC Band Combination.

Scope and objective for the ETSI MCC band combination database

The scope of the work was first agreed at RAN4#113 in [3], stating that:

<Agreement>:

-	The band combination information will be contained in JSON file format

- A separate JSON file per downlink configuration shall be created to contain information sorted based on downlink configuration to maintain the current organization of the listing of band combinations.

<Agreement>:

-	The JSON files shall be stored in a GIT repository managed by ETSI

<Agreement>:

-	The band combination database is intended to replace the listing of band combinations in Tables in the RAN4 specification.

-	As a starting point;

From 38.101-1 Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3.1-1, Table 5.5A.3.2-1, Table 5.5A.3.3-1, Table 5.5A.3.4-1, Table 5.5A.3.5-1, Table 5.5B-1, Table 5.5B-2, Table 5.5B-3, Table 5.5B-4, Table 5.5C-1, Table 5.5C-2, Table 5.5C-3, Table 5.5C-4, Table 5.5C-5.

From 38.101-2 Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3-1.

From 38.101-3 Table 5.5A.1.1-1, Table 5.5A.1.2-1, Table 5.5A.1.3-1, Table 5.5A.1.4-1

<Agreement>:

-	The JSON file shall contain at least:

-	UL configurations supported per DL configuration

-	Supported Channel Bandwidths per DL band

-	Supported Bandwidth Combination Set(s)

As noted above, it was initially agreed to limit the scope of this work to focus only on NR CA.

The focus of the work has been limited to adding only NR CA combinations to the ETSI MCC band combination database as a starting point.

However, as mentioned, work has started to also include DC combinations, as this has been requested by multiple companies to fully capture all supported 5G band combinations within the database. Given that this has been agreed to start work on also enabling the inclusion of ENDC combinations to the ETSI MCC band combination database, but this work has not yet been completed.

DC combinations are also intended to be included in the ETSI MCC band combination database.

RAN4 shall endorse the inclusion of ENDC band combinations in the database.

Timeline for the ETSI MCC band combination database

At RAN4#114, a WF [4] was agreed, stating that:

<Agreement>:

-	RAN4 will plan for a transition from MS Word table representation of band combinations to the ETSI MCC provided database either by the end of Rel-19 or during Rel-20

From the above agreement and the fact that RAN4 are soon to start Rel-20 work, it is needed for RAN4 to assess whether to target a transition to fully use the database as part of the transition from Rel-19 to Rel-20, or instead, the transition should be targeted during the Rel-20 timeframe.

RAN4 shall discuss and agree on a targeted transition point to fully use the database and remove the current DOCX table-based representation of supported band combinations

Given the current state of the adaptation and development of the database, it seems likely that a transition can take place during the Rel-20 timeframe. The question is then whether there needs to be a work item in RAN4 to handle the CRs needed to make the needed specification changes, or RAN4 shall consider having these changes included as objectives in the Rel-20 band combination basket work items.

With a transition to the ETSI MCC band combination database during Rel-20, there is a need for either an independent work item or objectives for the Rel-20 band combination basket work items to facilitate the CRs needed to make the specification changes.

It would seem the most straightforward to reuse the Rel-19 WIDs on NR CA (NR_CADC_SUL_R19) and DC (DC_R19_xBLTE_yBNR) in Rel-20, as it is not expected that there will be new types of configurations introduced. These new Rel-20 WIDs can then be updated to include an objective of introducing the use of the ETSI MCC band combination database for the specifications.

RAN4 shall consider adding the objective of introducing the use of the ETSI MCC band combination database for the specifications at least in the NR CA and DC basket WIDs

Working with the ETSI MCC band combination database

Working with the ETSI MCC band combination database is new to all, and practice and learning are expected by all who want to continue involvement in this work.

As discussed in previous RAN4 meetings, there are multiple steps in the process of introducing new band combinations, and all of these can, to some extent, be part of the ETSI MCC band combination database. However, as using the database is new to RAN4, there needs to be a balance between how much is implemented and when RAN4 can start to use it.

RAN4 have agreed, as captured in the WF at RAN4#115 in [3], to adopt the so-called stage 1 implementation of the ETSI MCC band combination database as described in the following.

Stage 1 – Procedures:

- Request for new band combinations, tracking and WID listing is continued with no change. This means Word is still used for WIDs and SRs, and Excel is used for listing band combinations and their status. There may be changes to the needed information included in the Excel sheets, but otherwise, no major changes.

- When a company wants to submit a new or modify an existing band combination, they prepare a discussion paper (TP) which outlines the intended change and provides the needed technical justification/analysis. This is done in Word with a Tdoc just like the current procedure. The new step is that a JSON file shall be provided and included in the Tdoc Zip file, which includes either the new or modified JSON container for the CA configuration or single band channel bandwidth support.

- During a RAN4 meeting, the submitted Tdocs can be handled similarly as now, using the basket procedure with flagging and block approval. Any issues identified during flagging or the meeting can be corrected with a revision, just like now, where the new thing is that a revision to the JSON file has to be provided.

- After the RAN4 meeting, the Rapporteur will collect all the agreed JSON files and, together with a potential BigCR, if needed, submit this for post-meeting approval. When the batch of JSON files is approved, these are either directly by the Rapporteur or via MCC committed to the ETSI Forge (GIT repository).

- The newly committed JSON file batch (i.e. commit number/identifier) will then be added to the “CR pack” for the RAN meeting by MCC.

- When RAN approves the JSON file batch, just like previously with CR packs, these become the new version of supported band combinations attached to a new version of the specifications.

- The WebApp will always display the latest approved batch of JSON files.

The procedures described above can now be adopted by RAN4 to efficiently start utilising the ETSI MCC band combination database. It is understood that multiple optimisations, like inclusions of automated checks, etc., are already mentioned/proposed by some companies, but it is still suggested that these are implemented at a later stage.

RAN4 have agreed on the “Stage 1 process” as a starting point for further adoption of the ETSI MCC band combination database.

As captured in the WF at RAN4#115 in [5], RAN4 is already now encouraged to start working with the JSON files and database to get a more tangible understanding of what it is and can become.

To facilitate the adaptation of the ETSI MCC band combination database by RAN4, further an introduction to it and some guidance on how to use it have been added in this section of this contribution.

Access to the database and its content

ETSI Forge

All 3GPP members can gain access to a repository in 3GPP’s Git(lab) instance (also known as ETSI Forge) via:

https://forge.3gpp.org/rep/ran4/carrieraggregation

using their EOL account.

All work related to the ETSI MCC band combination database is stored in the ETSI Forge.

ETSI have its own 3GPP Git(lab) instance known as the ETSI Forge

In the ETSI Forge repository for the ETSI MCC band combination database, it is possible to access the folder containing JSON data files, channel bandwidths per NR band, as well as the corresponding schema files.

Fig.  1 Repositories in the ETSI Forge

All 3GPP members have access to the ETSI Forge, where JSON data and schema files for bands and band combinations are stored.

The ETSI Forge inherently incorporates version control of the data stored, as it uses the underlying GIT framework. It is possible to align the versions of the JSON data and schema files to the versions of the specification, meaning that one similar to now can extract and view a specific version of supported band combinations. However, currently, the ETSI Forge version of the JSON data and schema files is not necessarily in sync with the official DOCX-based specifications. This is the transformation from the DOCX band combination tables are currently not automated, but run when needed during the trial phase of the implementation of the band combination database.

The alignment between official versions of the specification and the band combination database will be implemented at a later stage.

The intention is that when RAN4 sets a transition point from the DOCX-based specification to the band combination database, then the alignment will be done beforehand to facilitate verification. But until this, there is no need for frequent updates.

ETSI Web-App

The content of the ETSI MCC band combination database can be visualised via a web app.  This can be accessed via:

https://tnm.etsi.org/ca/

Via the Web-App, it is now possible to download all the content of the repository for the ETSI MCC band combination database using the EOL account login information. This can be accessed via the link below.

All the content of the latest version (main branch) of the ETSI MCC band combination database can be downloaded using the EOL account login information.

Note that, as mentioned, the version of the available data is currently not necessarily in sync with the official DOCX-based specifications.

Currently, ETSI is working on a tool hosted on the Web-App to facilitate validation of JSON data files against the JSON schema files and also provide the possibility of generating and editing JSON data files directly in the tool.

ETSI is working on providing a simple tool for generating, editing and validating JSON data files.

The tool is intended as a simple assistance to companies, as companies for more advanced use, use as running batch generation of JSON data files, are expected to have their own tools. This is no different from now, where different companies have their work processes for providing TPs and draftCRs.

Companies are expected to create their tools for more advanced generation and/or modifications of JSON data files.

Introduction of JSON data- and schema files

The JSON data files represent a specific downlink CA configuration with all its attributes. Meaning that one can envision each row of the current DOCX band combination tables as a representation of what is included in a JSON data file.

A JSON data file represents the information we currently have in each row of the band combination tables in the current DOCX specifications.

This means that instead of adding new rows to the band combination tables in the DOCX-based specifications, as we do now, we will instead add a new JSON data file to the database.

Similar to an agreed table format in the current DOCX-based specifications, the JSON data also needs to follow a specified structure to make sure it can be represented correctly. Therefore, RAN4 have agreed on a JSON schema file which can be envisioned as an empty DOCX-based table with columns, rows and cells ready to represent the content of the JSON data files. Hence, the JSON schema file defines the structure of the corresponding JSON data files.

The JSON schema file defines the structure (i.e table-structure) of the content of the JSON data files.

In conclusion, there are two different JSON file formats that it is important to distinguish:

- JSON Data Files

- JSON Schema Files

- These two are further explained in the following.

JSON Data Files

There are currently two different JSON data files stored in the ETSI forge. One is for the individual bands and the other for the band combinations. These are stored in different repositories, as shown in Fig. 1. The content of the channelBandwidths JSON data files representing all supported channel bandwidths of the individual bands is to be aligned to what is listed in TS 38.101-1/2 Table 5.3.5-1. The other repository, bandCombinations, contains all the JSON files representing the band combination configurations currently listed in:

From 38.101-1 Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3.1-1, Table 5.5A.3.2-1, Table 5.5A.3.3-1, Table 5.5A.3.4-1, Table 5.5A.3.5-1, Table 5.5B-1, Table 5.5B-2, Table 5.5B-3, Table 5.5B-4, Table 5.5C-1, Table 5.5C-2, Table 5.5C-3, Table 5.5C-4, Table 5.5C-5.

From 38.101-2 Table 5.5A.1-1, Table 5.5A.2-1, Table 5.5A.2-2, Table 5.5A.3-1.

From 38.101-3 Table 5.5A.1.1-1, Table 5.5A.1.2-1, Table 5.5A.1.3-1, Table 5.5A.1.4-1

Both the channelBandwidths and bandCombinations JSON data files follow the structure given by the JSON schema files stored in the jsonSchemas repository. The JSON schema files are further explained in the next section 3.2.2.

An example of a JSON Data file for CA_n5A-n8A is shown in Fig. 2 with Table 5.5A.3.1-1d from TS 38.101-1 for reference below.

Fig.  2 JSON data file for CA_n5A-n8A

In Fig. 2, the following variables can be found in the JSON data file for CA_n5A-n8A:

- bcId (string, required):

  - Unique identifier for the band combination.

- bcsList (array, required):

  - List of Band Combination Sets (BCS) for this band combination. Each BCS describes a possible configuration.

- bcsId (integer, required):

  - Unique identifier for this BCS within the band combination.

  - Note that bcsId 4 is used to indicate BCS 4&5, as an integer is used for this variable.

- bandList (array, required):

  - List of bands involved in this BCS. Each entry describes one band and its carrier configuration.

- bandNumber (string, optional):

  - Band number for a single-carrier configuration.

- nonContiguousCarriers (array, optional):

  - One or more non-contiguous carriers. Each item is an array of integers, where each integer is a carrier bandwidth in MHz.

- notes (object, optional):

  - Additional requirements, constraints, or informative notes applicable to selected bands. Contains boolean flags for specific technical notes (see section 3.2.3).

Note that some of the possible components/attributes of the JSON data files are not used in the example in Fig. 2. The full list is part of the JSON schema files described in the next section, and a summary of the components/attributes is included in Annexe A.

JSON Schema Files

JSON schema files are templates which define the structure of corresponding JSON data files. JSON editors can use the schema files to assist by auto-completion when writing or editing new data files and to highlight syntax errors when a data file does not comply with the schema.

JSON editors can use the schema files to assist by auto-completion when writing or editing new data.

Currently, there are two different JSON schema files, as shown in Fig. 3.

Fig.  3 Current JSON schema files in the ETSI Forge

The two different schema files give the structure for the channel bandwidths supported per band in the Band.json file and the individual NR CA band combinations in the BandCombination.json file. In relation to this work, when referring to the JSON data file, it is generally the band combinations files meant unless it is wanted to present a single band and its supported channel bandwidths. The reason there are two separate files is that the structure and information per band are different from that of the band combinations.

There are currently two different JSON schema files in the ETSI Forge.

When DC combinations are added to the ETSI MCC band combination database, there may be a need to define a specific JSON schema file for this, as the current BandCombination.json file is tailored to the NR CA combinations.

There may be a need for a new JSON schema file for DC combinations, as the ones used now are specific to NR CA combinations.

Typically, the schema files don’t need to be modified. A modification of a schema file corresponds to a change in today’s DOCX table layout (adding/removing columns; using different content in existing cells; …). If there is a need to modify a schema, RAN4 should review those changes with great care to ensure that existing data files are still compatible with the updated schema.

When schema files are modified, there will typically be a need to modify the Web-App, too. This is to ensure that all necessary data is also displayed in the Web-App as needed.

Finally, a modification of a JSON schema will typically also require updating tools that are used to validate consistency of the tables or to extract information, e.g. for use in product development.

It should be noted that modifications of the traditional DOCX table formats had the same consequences, but those were much more difficult to detect and account for than with the new JSON format. Nevertheless, it has also been an aim for RAN4 not to modify the DOCX tables unless completely justified. The same should apply to the JSON schema.

Frequent changes to the JSON schema should be avoided by RAN4.

See Annex A for a full list of all attributes of the JSON schema file.

See Annex A for JSON Schema File component reference of all attributes

(Foot-)Notes

In the DOCX version of TS 38.101, the note numbering and sometimes the wording for bands and band combinations were not aligned across tables and files. The same footnote numbers had different meanings in different tables and vice versa. Furthermore, definitions of those footnotes tended to deviate slightly in wording among tables, even though they were meant to be the same.

In the ETSI MCC band combination database, each band and each band combination is defined in a separate JSON data file. To ensure consistency among those files and to simplify creating and utilising those files, notes are now defined in the JSON schema files and associated with a short but descriptive identifier.

The JSON schema file includes the possible additional attributes of band combinations, as we are now capturing via notes in the DOCX-based specifications. This includes, e.g. HPUE UL support and/or DL channel bandwidth restrictions.

The JSON schema file includes the possible additional attributes of a band combination as e.g. additional UL power class support and/or DL channel bandwidth restrictions.

A clear advantage with this approach is that there will inherently need to be alignment/agreement on wording for these notes, defining additional attributes of band combinations. Further, as an extension to ensure unambiguous representation of the JSON data, the JSON schema file templates that the JSON editors (i.e proponents of a new or modified band combination) can use the schema files to assist by auto-completion when writing or editing new data files and to highlight syntax errors when a data file does not comply with the schema

By having the “master” JSON schema file as a starting point for all new band combinations, now including all the attributes (including notes), the task for those proposing a new combination will then become only to specify the required attributes. With this approach, each JSON file becomes independently readable and as lean as possible.

The use of the JSON schema files as a basis for adding notes to the JSON files is further described in [2].

Examples of creation/modification of JSON data files

There are two different recommended approaches to either introduce a new band combination or modify/amend an existing band combination within the ETSI MCC band combination database.

Note that the below is aligned to the Stage 1 procedure.

- Edit an existing JSON data file:

  - Navigate to the JSON data file located in the ETSI Forge bandCombinations repository (e.g. /38.101-1/) of the band combination which is to be modified/amended.

  - Download the JSON data file and save it as a local copy

    - It is suggested to append the Tdoc number of the contribution in front of the file name to associate the modified JSON data file.

  - Open the JSON data file in your preferred code editor (e.g. Notepad++ or VS Code).

  - Perform the needed additions or modifications to the JSON data file and save it

  - Add the modified JSON data file to the Zip folder together with other content, i.e. draftCR or TP.

  - Submit as a contribution for RAN4

- Add a JSON data file:

  - Copy an existing JSON data file as a template, or create a new file with a .json extension.

  - Save the new JSON data files as a local copy using the naming convention of the included bands

    - It is suggested to append the Tdoc number of the contribution in front of the file name to associate the new JSON data file.

  - Open the JSON data file in your preferred code editor (e.g. Notepad++ or VS Code).

    - If using a code editor as VS Code, one can use the autocompletion features provided by the JSON schema file to fill in required fields according to the schema.

  - Perform the needed additions or modifications to the JSON data file and save it

  - Add the modified JSON data file to the Zip folder together with other content, i.e. draftCR or TP.

  - Submit as a contribution for RAN4

The following shows an example with explanatory comments to the rows in the JSON structure.

{

"bcId": "n3A-n20A", // Unique identifier for the band combination

"bcsList": [

{

"bcsId": 0, // Unique identifier for this BCS

"ulConfigList": [

{

"bandNumber": "n3", // Uplink band number

"notes": {

"pc2_2tx": true // Uplink-specific note: Power Class 2, 2Tx

}

},

{

"bcId": "n3A-n20A" // Reference to another band combination for uplink

}

],

"bandList": [

{

"bandNumber": "n3", // Band number

"nonContiguousCarriers": [

[5, 10, 15, 20, 25, 30, 40] // List of supported non-contiguous carrier bandwidths (MHz)

]

},

{

"bandNumber": "n20",

"nonContiguousCarriers": [

[5, 10, 15, 20] // List of supported non-contiguous carrier bandwidths (MHz)

]

}

]

},

{

"bcsId": 4,

"ulConfigList": [

{

"bandNumber": "n3",

"notes": {

"pc2_2tx": true

}

},

{

"bcId": "n3A-n20A"

}

],

"bandList": [

{

"bandNumber": "n3",

"referencedComponents": [

{

"bandNumber": "n3" // Reference to a single-carrier band

}

]

},

{

"bandNumber": "n20",

"referencedComponents": [

{

"bandNumber": "n20"

}

]

}

]

}

]

}

RAN4 encourages companies to bring JSON files together with TPs and draftCR for NR CA combinations.

Next steps

Already now, multiple companies have started to work with the JSON data files and are getting themselves accustomed to them.

Multiple companies have already started working with the JSON data files.

The next step is to formalise how RAN4 will handle the JSON files. The agreed stage-1 adaptation states that:

- After the RAN4 meeting, the Rapporteur will collect all the agreed JSON files and, together with a potential BigCR, if needed, submit this for post-meeting approval. When the batch of JSON files is approved, these are either directly by the Rapporteur or via MCC committed to the ETSI Forge (GIT repository).

- The newly committed JSON file batch (i.e. commit number/identifier) will then be added to the “CR pack” for the RAN meeting by MCC.

- When RAN approves the JSON file batch, just like previously with CR packs, these become the new version of supported band combinations attached to a new version of the specifications.

The steps above need to be practised and also aligned with ETSI MCC working procedures ETSI MCC since they are hosting it and are responsible for its version control, etc.

Any process for updating and/or adding to the band combination database needs to be aligned and agreed with ETSI MCC, since they are hosting it and are responsible for its version control.

How this shall work in practice is a focus point for the task group in the coming time.

Future aspects

Currently, the focus of the work is on NR CA/DC combination tables listed in TS 38.101-1/2/3, as in the channel bandwidth and CA configuration tables. This means that configuration-specific requirements as delta values and potential MSD, are not within the scope of the ETSI MCC band combination database. This sis also reflected in the agreement below [5]:

<Agreement>:

-	RAN4 will focus on inclusion of the CA configuration tables to the database as a first step. Other aspects as delta values and MSD requirements can be considered at a later stage.

However, there have been multiple suggestions to also consider moving the long tables listing band combination-specific requirements to the ETSI MCC band combination database. It is therefore proposed that RAN4 discuss how this can be facilitated.

RAN4 shall discuss the inclusion of band combination-specific requirements in the ETSI MCC band combination database.

Conclusion

In this contribution, we present the status of the band combination database and what steps are planned next. The following observations have been made:

Observation 1: The focus of the work has been limited to adding only NR CA combinations to the ETSI MCC band combination database as a starting point.

Observation 2: DC combinations are also intended to be included in the ETSI MCC band combination database.

Proposal 1: RAN4 shall endorse the inclusion of ENDC band combinations in the database.

Proposal 2: RAN4 shall discuss and agree on a targeted transition point to fully use the database and remove the current DOCX table-based representation of supported band combinations

Observation 3: With a transition to the ETSI MCC band combination database during Rel-20, there is a need for either an independent work item or objectives for the Rel-20 band combination basket work items to facilitate the CRs needed to make the specification changes.

Proposal 3: RAN4 shall consider adding the objective of introducing the use of the ETSI MCC band combination database for the specifications at least in the NR CA and DC basket WIDs

Observation 4: RAN4 have agreed on the “Stage 1 process” as a starting point for further adoption of the ETSI MCC band combination database.

Observation 5: ETSI have its own 3GPP Git(lab) instance known as the ETSI Forge

Observation 6: All 3GPP members have access to the ETSI Forge, where JSON data and schema files for bands and band combinations are stored.

Observation 7: The alignment between official versions of the specification and the band combination database will be implemented at a later stage.

Observation 8: All the content of the latest version (main branch) of the ETSI MCC band combination database can be downloaded using the EOL account login information.

Observation 9: ETSI is working on providing a simple tool for generating, editing and validating JSON data files.

Observation 10: Companies are expected to create their tools for more advanced generation and/or modifications of JSON data files.

Observation 11: A JSON data file represents the information we currently have in each row of the band combination tables in the current DOCX specifications.

Observation 12: The JSON schema file defines the structure (i.e table-structure) of the content of the JSON data files.

These two are further explained in the following.

Observation 13: JSON editors can use the schema files to assist by auto-completion when writing or editing new data.

Observation 14: There are currently two different JSON schema files in the ETSI Forge.

Observation 15: There may be a need for a new JSON schema file for DC combinations, as the ones used now are specific to NR CA combinations.

Observation 16: Frequent changes to the JSON schema should be avoided by RAN4.

Observation 17: See Annex A for JSON Schema File component reference of all attributes

Observation 18: The JSON schema file includes the possible additional attributes of a band combination as e.g. additional UL power class support and/or DL channel bandwidth restrictions.

Proposal 4: RAN4 encourages companies to bring JSON files together with TPs and draftCR for NR CA combinations.

Observation 19: Multiple companies have already started working with the JSON data files.

Observation 20: Any process for updating and/or adding to the band combination database needs to be aligned and agreed with ETSI MCC, since they are hosting it and are responsible for its version control.

Proposal 5: RAN4 shall discuss the inclusion of band combination-specific requirements in the ETSI MCC band combination database.

- References

- SP-250802, Study on Modernization of Specification Format and Procedures for 6G, Nokia et. al.

- R4-2507377, Progress update and next steps for the ETSI MCC band combination database, Nokia, Ericsson

- R4-2420341, WF on next step for CA framework database, Nokia

- R4-2502976, WF on next step for CA framework database, Nokia

- R4-2508014, WF for the ETSI MCC band combination database, Nokia, Ericsson

- Annex A – JSON Schema File component reference

The BandCombination.json schema defines the structure for each band combination file. Below is a detailed description of its components to guide you in creating or editing band combinations.

### Top-Level Properties

- bcId (string, required):

  - Unique identifier for the band combination (e.g., n1(2A)-n3A-n7A).

- bcsList (array, required):

  - List of Band Combination Sets (BCS) for this band combination. Each BCS describes a possible configuration.

- notes (object, optional):

  - Additional requirements, constraints, or informative notes applicable to selected bands. Contains boolean flags for specific technical notes (See Notes Objects for examples).

### Band Combination Set (bcsList items)

- bcsId (integer, required):

  - Unique identifier for this BCS within the band combination.

- ulConfigList (array, optional):

  - List of uplink configuration entries.

- bandList (array, required):

  - List of bands involved in this BCS. Each entry describes one band and its carrier configuration.

### Uplink Configuration (ulConfigList items)

- bcId (string, optional):

  - Band combination ID for uplink (for referencing another band combination).

- bandNumber (string, optional):

  - Band number for a single-carrier configuration.

- notes (object, optional):

  - Uplink-specific notes (See Notes Objects for examples).

### Band (bandList items)

- bandNumber (string, required):

  - The band number (e.g., n1, n3).

- nonContiguousCarriers (array, optional):

  - One or more non-contiguous carriers. Each item is an array of integers, where each integer is a carrier bandwidth in MHz.

- contiguousCarriers (array, optional):

  - One or more variants of two or more contiguous carriers. Each item is an array (variant) of arrays (carriers), where each carrier is an array of integers (bandwidths in MHz) or a single string (reference to all bandwidths for a band).

- referencedComponents (array, optional):

  - References to other bands or band combinations are inherited into this band combination. Each reference can be:

  - An object with bcId (string) and bcsId (integer) for referencing another band combination set.

  - An object with bandNumber (string) for referencing a single-carrier band.

- maxAggBwPerBand (integer, optional):

  - The maximum aggregated bandwidth (in MHz) across all carriers in this band.

### Notes Objects

Both the top-level notes and ulConfigList[].notes can contain boolean flags for special requirements or constraints, such as:

- pc2, pc2_2tx, pc2_3tx, pc1p5, pc1p5_2tx, pc1p5_3tx, n26_DualPA, ul_n26_opt, fLim3450_3700, Rel-18_800MHzUL (uplink-specific)

- psdi_6dB, ul_n5, intraReq, ul_n28, n77_RxTx, noSimRxTx, n7_n38, Rel-18_1600MHzDL (downlink-specific)

Refer to the JSON schema file for full descriptions of each flag.