3GPP TSG-RAN WG4 Meeting #116						R4-2510908

Bangalore, IN. 25th – 29th, 2025

Source:	Nokia

Title:	TP to TR 38.793 on applicability of mandatory support of simultaneous Rx-Tx

Agenda item:	6.11.2

Document for:	Approval

Introduction

Simultaneous Rx/Tx UE Capability

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory, and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

At RAN4#115s a discussion was raised in [1] explaining the uncertainty on when the “conditional mandatory” applies and when it is needed to defined requirements and/or indicate in the specification the mandatory support of simultaneous Rx/Tx capability. A WF as included in section 1.1. was agreed and the clarification was also incuraged to be included to the TR via TP submitted at this meeting. Hence, we in this contribution provide a TP for TR 38.793 on applicability of mandatory support of simultaneous Rx-Tx

RAN4 shall agree the TP presented in Clause 2 to TR 38.793 on applicability of mandatory support of simultaneous Rx-Tx.

WF from previous meeting

At RAN4#115 a WF for simultaneous Rx-Tx [2] includes the following:

RAN4 has concluded simultaneous Rx/Tx requirements for different types of inter-band CA and SUL configurations:

- TDD + TDD -> Conditional mandatory support for simultaneous RxTx with signaling

- FDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- TDD + SDL -> Mandatory support for simultaneous RxTx with signaling

- FDD/TDD + SUL -> Mandatory support for simultaneous RxTx with signaling

- FDD + FDD -> Mandatory support for simultaneous RxTx without signaling

- FDD + SDL -> Mandatory support for simultaneous RxTx without signaling

TP to TR 38.793

***************************** Start of TP ************************************

## 4.2	WI Objective in Rel-19

1.	Identify the unnecessary Notes for simultaneous Rx-Tx applied to band combinations for NR CA/DC, NR SUL and LTE/NR DC. Focus on principles and specific methods of simplification on Notes to simultaneous Rx-Tx case by case.

2.	Clarify in the specification on the simultaneous Rx-Tx feature for DC band combinations with same band but different configurations of carrier number.

3.	Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values. If needed, specify the MSD requirements for the identified band combinations.

Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary.

Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

Note 3: Applicability of simultaneous Rx/Tx capability to FDD-TDD band pairs should be reviewed

4.	Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

## 4.3	Applicability of mandatory support of simultaneous Rx-Tx

During Rel-19 RAN4 has agreed the following applicability of mandatory support of simultaneous Rx-Tx which may aid as a guide to when requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC as captured in this TR is needed defined.

RAN4 has concluded simultaneous Rx/Tx requirements for different types of inter-band CA and SUL configurations:

- TDD + TDD -> Conditional mandatory support for simultaneous RxTx with signaling

- FDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- TDD + SDL -> Mandatory support for simultaneous RxTx with signaling

- FDD/TDD + SUL -> Mandatory support for simultaneous RxTx with signaling

- FDD + FDD -> Mandatory support for simultaneous RxTx without signaling

- FDD + SDL -> Mandatory support for simultaneous RxTx without signaling

Since the simultaneous Rx/Tx capability in one case is conditional mandatory, it means that requirement for related requirements in some cases is dependent on whether signalling is introduced for the specific combination. Signalling is present when indication is included for the specific combination e.g. as in TS 38.101-1 Table 5.2A.2.1 by Note 1.

***************************** End of TP ************************************

Conclusion

In this contribution, we present a TP and the following proposals and observations:

Observation 1: The Simultaneous Rx/Tx UE capability is conditional mandatory and therefore it needs to be accurately noted in the RAN4 specification whether it is expected supported by the UE for a given band combination.

Proposal 1: RAN4 shall agree the TP presented in Clause 2 to TR 38.793 on applicability of mandatory support of simultaneous Rx-Tx.

- References

- R4-2507374, On simultaneous RxTx and capability for overlapping FDD bands, Nokia, RAN4#115

- R4-2507910, WF on simultaneous Rx-Tx, Huawei