# 3GPP TSG-RAN WG4 Meeting #116	R4-2510909

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	7.24.5.2

Source: 	Ericsson

Title:	Remaining issues on LP-WUS RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1].

Issue 1-1-10: Higher priority frequency layer measurement requirements Agreement: For case 1: - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. For case 3: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers: - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied Issue 1-1-17 Reply RAN1 LS R1-2503103 Agreement on the reply LS to RAN1: - Confirm to RAN1 that for LP-SSS-RSRP/RSSI measurement performed by OFDM-based LP-WUR for the serving cell, SMTC window is not applicable. - RAN4 agreed that: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state Issue 1-2-4-1: On LR measurement filtering requirement Agreement: No need to define filtering requirement for LP-RSRP and LP-RSRQ measurements. Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity Agreement: This issue is closed. Issue 1-2-6-1: On the number of SSBs (for RAN1 LS R1-2501624) Agreement: For the 400ms and 800ms ramping up time: {5, 5} samples For the 10ms ramping up time: 3 samples Agreement For the side conditions, RAN4 assume the side conditions is the side conditions of the LP-WUS monitoring used by RAN1

In this contribution, we continue the discussion on the remaining issues of LP-WUS.

- General aspects

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In RAN4 #114 meeting, RAN4 already made the agreement.

RAN4 114 Agreement: - At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

Before UE entering LR-based RRM relaxation or offloading, LR may already wake up since UE needs to evaluates LR criteria if NW configured. There is no issue from UE side to wake up LR earlier and earlier monitoring LR clearly shows the benefits to avoid the ping-pong between the LR and MR. However, when to wake up the LR receiver to monitor the LR status is up to UE implementation. We expect no LR measurement and evaluation requirements apply at the legacy state, i.e., from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). The requirements is applied when the LP-WUR is in ON state.

Proposal 1: The LR measurement and evaluation requirement is applied when the LP-WUR is in ON state.

Issue 1-1-10: Higher priority frequency layer measurement requirements

In last meeting, RAN4 agreed the UE measurement requirement for higher priority frequency layer as follow:

RAN4 #115 Issue 1-1-10: Higher priority frequency layer measurement requirements Agreement: For case 1: - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. For case 3: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchP is NOT met, the same requirement for higher priority, equal priority and lower priority carriers: - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

For case 3, the condition is only related to Rel-15 SnonIntraSearchP and SnonIntraSearchQ. It may result in some misunderstanding about the intra-frequency behaviour in RRM relaxation. For example, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE may only perform relaxed higher priority measurement. However, if Srxlev > SIntraSearchP and Squal > SIntraSearchQ is not fulfilled, UE still needs to perform intra-frequency measurements. It’s independent with non-intra-frequency measurement thresholds.

Proposal 2: In LP-WUS Case 3, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, but Srxlev < SIntraSearchP or Squal < SIntraSearchQ, UE follows RRM relaxation measurement requirement for intra-frequency with scaling factor 16.

Issue 1-1-11: RRM requirements for FR2

In current state, there is no any concrete proposal to identify the FR2 issue. Therefore, we suggest to focus on FR1 first to complete the WI on time.

Issue 1-1-11: RRM requirements for FR2 Proposals - P1: Prioritize defining RRM requirements for FR1 in this WI - P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI

Proposal 3: RAN4 to focus on LP-WUS FR1 RRM requirement in Rel-19.

- Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

RF margin

The remaining issue is about the RF impairment margin for LR measurement accuracy requirement. Generally, in RAN4, we use 2.5dB RF impairment margin for measurement. Thus, we suggest to reuse the 2.5dB margin.

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Proposals - P1: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. - P2: Consider 3.5 dB RF impairment margin for LR

Proposal 4: RAN4 to use 2.5dB RF impairment margin for LR accuracy requirements.

Transition period

In Rel-16 RRM relaxation, RAN4 also defined the transition requirement among not in UE fulfilling cell edge or low mobility to UE fulfilling both not in cell edge and low mobility. When UE detects to fulfil Rel-19 RRM measurement relaxation or offloading crietia, UE will switch between normal measurement, no neighbour cell measurement and Rel-19 MR RRM measurement relaxation or offloading. RAN4 shall also discuss and define the transition requirements among different status.

Figure 1. Transition status for Rel-19 MR RRM relaxation, offloading

Proposal 5: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading.

Low mobility

In Rel-16 RRM relaxation, except not in cell edge criteria, RAN4 also defined the low mobility criteria. In Rel-19 LP-WUS, RAN2 is also discussing the low mobility criteria based on LR. From our understanding, whether to introduce new LR-based low mobility criteria is up to RAN2. RAN4 shall focus on the existing criteria about Rel-16 low mobility. In Rel-16, when UE fulfils both not in cell edge and low mobility, the requirements can be further relaxed, such as measure the inter-frequency layer of higher priority at least every 1 hour. However, in Rel-19, when UE fulfils the RRM relaxation criteria, the requirements are agreed with scaling factor 16. Therefore, RAN4 needs to discuss the UE behaviour when UE fulfils both Rel-16 low mobility and Rel-19 RRM relaxation criteira.

Proposal 6: RAN4 to discuss the UE behaviour when UE detects to meet Rel-16 low mobility criteria and Rel-19 RRM relaxation criteira.

- LP-WUR requirements at RRC-CONNECTED mode

In last RAN4 meeting, one of the remaining issues is whether and which CONNECTED mode requirement needs to be discussed.

Issue 1-4-1: LP-WUR at CONNECTED mode Proposals - P1: No RRM objectives is expected for connected mode in this WI. (oppo ZTE) - P2: Postponed until more progress in other groups (xiaomi vivo Ericsson) - P3: For LP-WUR at CONNECTED mode, discuss the interruption requirement for LP-WUS monitoring after the DRX related discussion is concluded in RAN1/2.(Apple) - P4: Prioritise idle-mode requirements over connected mode requirements. If needed, start the connected mode requirements from LR PSS/SSS and LP-WUS monitoring requirements. (Nokia) - P5: In the Connected mode, interruptions on the serving cell due to MGs as defined in Clause 9.1.2 also apply to WUS monitoring by the WUR. In the Connected mode, interruption requirements specified in Clause 8.2 also apply to WUS monitoring by WUR. (Qualcomm)

In RAN2 #129bis meeting, RAN2 had sent a LS to RAN1 to ask the LP-WUS collision in RRC_CONNECTED mode.

RAN2 discussed whether there are any scenarios on when the UE would not be able to monitor LP-WUS and made the following working assumption: Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Based on this working assumption, RAN2 would like to ask RAN1: 1. Whether there are any cases/scenarios on when the UE is not able to monitor LP-WUS? - RAN2 would like to note that for DCP, such cases/scenarios included collision with Active Time, measurement gap, interruption caused by BWP switching, and RAR window monitoring for BFR. 2. Is there any case when the UE is not able to monitor LR and MR simultaneously?

In last RAN1 meeting, RAN1 had already made the agreement about the LP-WUS collision with MG as follow. When LP-WUS monitoring is colliding with MG, UE is not able to monitor LP-WUS. Thus, RAN4 can follow the same conclusion. The detail procedure can be captured in RAN2 spec. and no impact is identified in RAN4 requirement.

RAN1 #121 Agreement As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS.

Proposal 7: No RAN4 RRM requirement related to LP-WUS is idenfitied in RRC_CONNECTED mode.

- Others

EMR

In last meeting, some companies mentioned to discuss UE’s behaviour in EMR. In legacy IDLE/Inactive mode RRM requirement, the relaxation of neighbour cell measurements is not allowed when EMR is configured on that frequency layer and T331 is running. However, RAN4 never discussed the UE’s behaviour when NW configures EMR in MR relaxation and offloading scenarios. From our understanding, both Rel-16 AND Rel-18 EMR should be considered. For Rel-16 EMR, when UE is in RRM relaxation mode, the Rel-19 RRM measurement relaxation shall not apply to the carriers configured for Rel-16 EMR when Srxlev <= SnonIntraSearchP or Squal <= SnonIntraSearchQ. The UE shall follow the higher priority measurement requirement for the carriers configured for Rel-16 EMR when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. When UE is in RRM offloading mode, UE shall follow the higher priority measurement requirement for the carriers configured for Rel-16 EMR.

Issue 1-5-1: LR based RRM with EMR in IDLE/Inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple) - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo) - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC) - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements. (Nokia) - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode) - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Proposal 8: For a UE which supports Rel-16 EMR,

- If UE fulfils MR RRM relaxation condition and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE shall at least search for carriers configured for EMR (and the serving cell) based on higher priority layer search requirement Thigher_priority_search;

- If UE fulfils MR RRM relaxation condition and Srxlev <= SnonIntraSearchP or Squal <= SnonIntraSearchQ, the RRM measurement relaxation is NOT applied to the carriers configured for EMR measurement;

- If UE fulfils MR RRM offloading condition(UE also fulfils Rel-15 neighour cell measurement stopping criteira), UE shall at least search for carriers configured for EMR (and the serving cell) based on higher priority layer search requirement Thigher_priority_search.

Proposal 9: For a UE which supports Rel-18 EMR, FFS UE’s measurement behaviour for the carriers configured for EMR when UE filfils RRM relaxation/offloading thresholds.

RedCap

In Rel-19 LP-WUS WID[2], it clearly explained the use case for LP-WUS as below: “Energy efficiency is even more critical for UEs without a continuous energy source. Among vertical use cases, sensors and actuators are deployed extensively. Wearables include smart watches, rings, eHealth related devices, and medical monitoring devices. With typical battery capacity, it is challenging to sustain up to 1-2 weeks as required. In our understanding, RedCap UE is a typical use case for LP-WUS.”

In last RAN1 meeting, it clearly mentioned RedCap UE can support LP-WUS if reported. Furthermore, LP-WUR is assumed as a new engine. All the related requirements and simulations can be independent with UE type. No additional evaluation efforts is needed. Therefore, when RAN4 defines the LP-WUS related requirement, RAN4 shall consider both normal UE and RedCap UE. RedCap UE shall also meet LP-WUS related requirement.

RAN1 #120bis meeting Agreement For RRC idle/inactive, support Alt 2 for both Redcap and non-redcap UEs: - Alt 2: LP-WUS/LP-SS frequency resource can be out of initial DL BWP where UE receives paging but within the same carrier. - FFS whether to define the maximum frequency gap between LP-WUS/LP-SS and SSB

Proposal 10: RedCap UE shall meet LP-WUS related requirement based on UE’s LP-WUS capability.

- Summary

In this contribution, we discussed the remaining issues on LP-WUS operation. Based on the discussions, we have made the following proposals and observations:

Proposal 1: The LR measurement and evaluation requirement is applied when the LP-WUR is in ON state.

Proposal 2: In LP-WUS Case 3, when Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, but Srxlev < SIntraSearchP or Squal < SIntraSearchQ, UE follows RRM relaxation measurement requirement for intra-frequency with scaling factor 16.

Proposal 3: RAN4 to focus on LP-WUS FR1 RRM requirement in Rel-19.

Proposal 4: RAN4 to use 2.5dB RF impairment margin for LR accuracy requirements.

Proposal 5: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading.

Proposal 6: RAN4 to discuss the UE behaviour when UE detects to meet Rel-16 low mobility criteria and Rel-19 RRM relaxation criteira.

Proposal 7: No RAN4 RRM requirement related to LP-WUS is idenfitied in RRC_CONNECTED mode.

Proposal 8: For a UE which supports Rel-16 EMR,

- If UE fulfils MR RRM relaxation condition and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE shall at least search for carriers configured for EMR (and the serving cell) based on higher priority layer search requirement Thigher_priority_search;

- If UE fulfils MR RRM relaxation condition and Srxlev <= SnonIntraSearchP or Squal <= SnonIntraSearchQ, the RRM measurement relaxation is NOT applied to the carriers configured for EMR measurement;

- If UE fulfils MR RRM offloading condition(UE also fulfils Rel-15 neighour cell measurement stopping criteira), UE shall at least search for carriers configured for EMR (and the serving cell) based on higher priority layer search requirement Thigher_priority_search.

Proposal 9: For a UE which supports Rel-18 EMR, FFS UE’s measurement behaviour for the carriers configured for EMR when UE filfils RRM relaxation/offloading thresholds.

Proposal 10: RedCap UE shall meet LP-WUS related requirement based on UE’s LP-WUS capability.

- References

[1]. R4-2508280, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. RP-240773, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony