# 3GPP TSG-RAN WG4 Meeting #116	R4-2510913

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	7.23.3.1

Source: 	Ericsson

Title:	Remaining issues on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, the following OD-SSB related issues were agreed.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 1: Lower bound for measurement in FMW Agreement: Introduce UE capability on whether a lower bound is needed, i.e., - No signalling is needed if UE doesn’t need the lower bound (default value), or, - The lower bound for measurement/cell identification period in FMW is derived based on 10ms periodicity in case 1 and case 2 Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1: The start point of T2 Agreement: - The time point of OD-SSB deactivation. Sub-issue 2: The minimum interval(T2) between two FMWs Agreement: - For FR1, 5*max (measCycleSCell, DRX cycles) - For FR2, 4s for power class 1/5 and 3s for power class 2/3/4 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Agreement: - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. Issue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation. Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion. Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies) Sub issue 1: RAN2 LS for scenario 3 Agreement: - RAN4 to confirm RAN2’s WS to introduce a new servingCellMO. Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion. Issue 1-2-5: Scheduling restriction for L1 measurement after SCell activation Agreement - In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement. - The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them. Issue 1-3-1: The known cell condition in Case 2 Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. Issue 1-3-2: The known cell condition – period of known condition Agreement - Reuse the legacy known/unknown cell condition period for FR1 and FR2, i.e. max(5*measCycleSCell, 5*DRX cycles) in FR1 or 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 in FR2. Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.] Issue 1-4-3: Additional Processing time for OD-SSB activation Agreement - RAN4 should introduce UE capability with values of 2 ms and 5 ms. Issue 1-4-7: L1 measurement in scenario 3B Agreement - When MAC-CE indicates OD-SSB updating in scenario 3B, - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB. - When DRX is configured, follow legacy principle to define requirements - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4. Issue 1-4-8: Neighbor cell measurement in the carrier of OD-SSB serving cell before OD-SSB activation in Case 1 Agreement - UE to perform neighbour cell measurement based on legacy requirement.

In this contribution, we continue to discuss the remaining issues on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- OD-SSB based deactivated SCell measurement

## 2.1 Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the agreements about OD-SSB based deactivated SCell measurement requirement in Case 1, but some details related to OD-SSB activation requirement and procedure are still FFS.

OD-SSB in FMW collided with MG

RAN4 never discussed the collision between OD-SSB in FMW with MG. The wayforward is shown as follow.

Issue 1-1-1: OD-SSB based deactivated SCell measurement – Requirement Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In legacy deactivated SCell measurement, generally, the measCycleSCell for deactivated SCell and MGRP are larger than the SSB periodicity. UE can find the SSB occasion for deactivated SCell outside MG. However, when UE performs the fast measurement for OD-SSB in FMW, UE cannot find spare OD-SSB occasion if colliding with MG since the requirement is based on OD-SSB periodicity. In legacy release, MG always has higher priority than the measurement outside the gap in legacy measurement design. Therefore, we support to skip the OD-SSB if OD-SSB collides with MG occasion in FMW. The OD-SSB measurement requirement can be updated to include Kp.

Proposal 1: UE to skip the OD-SSB measurement if the OD-SSB collides with MG occasion in FMW.The OD-SSB measurement period and the FMW shall be extended by Kp due to OD-SSB occasion in FMW colliding with MG occasion.

UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2

RAN4 spent a long time discussion in several meetings to achieve the agreements related T2 in last meeting. From our understanding, the UE behaviour is clear based on the agreements in last meeting. When OD-SSB is further activated within T2 after OD-SSB deactivation, the UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. On the contrary, when NW activates the OD-SSB after T2, UE performs the fast measurement within FMW. Current solution is a good trade-off between NES and UES. We propose that UE can follow legacy deactivated SCell measurement after T2 when OD-SSB is further activated within T2 after OD-SSB deactivation.

Proposal 2: UE can follow legacy deactivated SCell measurement after T2 when OD-SSB is further activated within T2 after OD-SSB deactivation.

## 2.2 Case 2: With always-on SSB on the cell

Alt Time-C2

At the same time, RAN1 also introduced a new scenario Alt Time-C2 where the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern. In our understanding, some UEs may have issues to combine both OD-SSB and AO-SSB to perform measurement and SCell activation in this case. However, the UE at least can follow the basic princinple to use OD-SSB to perform measurement in FMW. During the meeting, one concern from UE vendors is about how to handle the scheduling restriction for AO-SSB. From our understanding, no scheduling restriction is expected for the AO-SSB occasions since the requirement is defined based on OD-SSBs.

Figure 1. OD-SSB and AO-SSB transmission in scenario Alt Time-C2

Proposal 3: RAN4 to define the requirement based on the OD-SSB periodicity if UE supports Case 2 Alt Time C2.

- No scheduling restriction is expected for the AO-SSB occasions.

Scenario 3

In RAN1 #120 meeting, RAN1 agreed to introduce a new OD-SSB scenario in which the frequency location of OD-SSB is different with the frequency location of AO-SSB. In last RAN2 meeting, RAN2 made the agreement to introduce a new servingCellMO to indicate MO of OD-SSB.

RAN2 #130 agreement - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

Figure 2. OD-SSB and AO-SSB transmission in scenario 3

When NW indicates the OD-SSB transmission, UE shall at least monitor OD-SSB for serving cell measurement and/or SCell activation. In addition, UE needs to perform messurement without gap on both AO-SSB and OD-SSB frequencies since NW has configured frequency layers for both AO-SSB and OD-SSB in the same BWP. When both AO-SSB and OD-SSB are configured, a potential open issue from RAN4 is how to define intra-frequency which will be difficult and result in a long debate to change current intra-frequency definition. Another issue is how to define the UE behaviour in this scenario 3. One possible solution is to follow OD-SSB activation/deactivation command. UE performs intra-frequency in AO-SSB frequency before and after OD-SSB activation, but UE follows intra-frequency in OD-SSB when NW activates the OD-SSB. However, this solution will result in dynamically switch intra-frequency between two frequencies. In RedCap, NW can indicate the NCD-SSB frequency as the intra-frequency based on serving cell MO. However, based on current RAN2 design, a new signalling to further indicate the frequency is needed if RAN4 follows the similar pricinple. We think it’s too late to introduce a new signalling in current stage. Therefore, we suggest to not define requirements for this scenario 3.

Proposal 4: In Rel-19, RAN4 NOT to define requirement if OD-SSB and AO-SSB are in the different frequencies and only define the requirement when OD-SSB is in the same frequency as AO-SSB.

- OD-SSB based SCell activation

In RAN4 #115 meeting, RAN4 agreed the OD-SSB based known/unknown Scell activation in Case 2 Alt Time-C1 as follow.

Issue 1-3-1: The known cell condition in Case 2 Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. - FFS whether apply to Alt Time-C2, if the requirement is defined. - FFS whether apply to scenario 3, if the requirement is defined. - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

Known cell condition in Case 2

From our understanding, the agreement of known cell condition for Alt Time-C1 mostly can be reused for Alt Time-C2. The known condition needs a little update as follow since the Alt Time-C2 SCell activation requirement is only based on OD-SSB. The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2 with the following combination: When the SSB measured for known cell condition is AO-SSB or OD-SSB, and the SSB used in SCell activation procedure is AO-SSB or OD-SSB.

Proposal 5: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2.

- OD-SSB based L3 measurement

Alt Time-C1

In last RAN2 meeting, RAN2 made the agreement about OD-SMTC and sent the LS to RAN4 as follow.

R2-2504981 LS For the case where AO-SSB and OD-SSB have the same center frequency, RAN2 had made the following agreement: - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

In last RAN4 meeting, RAN4 agreed to apply effective measurement periodicity to L1-RSRP measurement for an active SCell.

RAN4 #115 Issue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation.

From our understanding, the UE’s behaviour for serving cell measurement and neighbour cell measurement for L3 measurement is similar as SMTC1 and SMTC2. UE shall follow OD-SMTC to perform serving cell measurement, and follow legacy SMTC(s) to perform neighbour cell measurement.

Proposal 6: When SCell is activated, RAN4 to define L3 measurement requirement based on the OD-SMTC for serving cell and legacy SMTC(s) for neighbour cells in Alt Time-C1.

- General issues

Scenario

In last meetings, RAN4 agreed to define requirements for OD-SSB based multiple SCell activation procedures. However, the detail scenario hasn’t been agreed. To simplify the scenario, all SCells being activated shall be the OD-SSB. Both legacy known and unknown cases in FR1 and FR2 should be supported.

Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

Proposal 7: RAN4 to define OD-SSB based multiple SCells activation delay requirements.

- All Scells being activated are OD-SSB based Scells.

- Both known and unknown SCell activation in FR1 and FR2 should be supported.

Additional processing time for OD-SSB parameter update

In last RAN4 meeting, RAN4 discussed the additional processing time for OD-SSB parameter update. After furthering thinking the possible two solutions on the table, we’re fine to go with option 1 to merge the OD-SSB parameter update processing time together with OD-SSB activation capability.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

Proposal 8: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The additional processing time is 5ms or 2ms based on UE capability reporting.

- The additional processing time is only applied to L1 and L3 measurement transition period after the SCell is activated.

Additional processing time for OD-SSB deactivation

In last RAN4 meeting, RAN4 confirms RAN1’s working assumption and also discuss whether additional UE processing time is needed. From our understanding, the UE behaviour is similar as SCell deactivation. The timeline should be the same without additional processing time. We also agreed UE’s observation that the deactivated OD-SSB may be included in the measurement filtering and result in an error measurement reporting. This error case should also be considered.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Proposal 9: No UE additional processing time is allowed for OD-SSB deactivation.

Cell identification component

In last meetings, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indentification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than Xs and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 10: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary in the following scenario.

  - UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5]s and then the OD-SSB is activated again with the same spatial reception parameter.

Interruption

In RAN4 #114bis meeting, RAN4 agreed to define the OD-SSB based interruption requirements as follow. RAN4 needs to further discuss the interruption duration and position.

Issue 1-4-9: Interruption Agreement - New interruption is allowed for scenario 2 due to OD-SSB activation/deactivation. - New interruption is also allowed for scenario 2 at the end of the FMW if UE does not receive SCell activation command. - The interruption can be applied in both Case 1 and Case 2. - The detail condition, duration and position of the interruption is FFS. - The interruption is allowed for scenario 2a due to OD-SSB activation/deactivation. - No new interruption requirement is defined. - The interruption requirement follows the same principle of legacy SCell activation.

From our understanding, the interruption duration shall be similar as RF retuning in legacy SCell activation which is 0.5ms in FR1 and FR2. OD-SSB can be activated/deactivated at the same time of RRC-based and MAC-based SCell activation/deactivation and together with deactivated SCell measurement. Thus, the OD-SSB based activation/deactivation interruption can also apply to both SCell addition/release, SCell activation/deactivation and deactivated SCell measurement. During previous discussion, RAN4 also agreed the OD-SSB based SCell activation is based on OD-SSB periodicity other than SMTC. In other words, the OD-SSB based SCell activation is purely up to its OD-SSB and the RF retuning is not related with other serving cells’ SSB. Therefore, the interruption should be only applied based on OD-SSB occasion other than the SMTC duration due to other active serving cells which are contiguous in FR1 or in the same band in FR2.

Proposal 11: The interruption duration of OD-SSB activation/deactivation shall be 0.5ms.

Proposal 12: The The interruption due to OD-SSB activation/deactivation is applied to SCell addition/release, SCell activation/deactivation and deactivated SCell measurement.

Proposal 13: The interruption length due to OD-SSB activation/deactivation is independent with other active serving cells which are contiguous in FR1 or in the same band in FR2.

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: UE to skip the OD-SSB measurement if the OD-SSB collides with MG occasion in FMW.The OD-SSB measurement period and the FMW shall be extended by Kp due to OD-SSB occasion in FMW colliding with MG occasion.

Proposal 2: UE can follow legacy deactivated SCell measurement after T2 when OD-SSB is further activated within T2 after OD-SSB deactivation.

Proposal 3: RAN4 to define the requirement based on the OD-SSB periodicity if UE supports Case 2 Alt Time C2.

- No scheduling restriction is expected for the AO-SSB occasions.

Proposal 4: In Rel-19, RAN4 NOT to define requirement if OD-SSB and AO-SSB are in the different frequencies and only define the requirement when OD-SSB is in the same frequency as AO-SSB.

Proposal 5: The current known SCell activation delay methodology can be reused in Case 2 Alt Time-C2

Proposal 6: When SCell is activated, RAN4 to define L3 measurement requirement based on the OD-SMTC for serving cell and legacy SMTC(s) for neighbour cells in Alt Time-C1.

Proposal 7: RAN4 to define OD-SSB based multiple SCells activation delay requirements.

- All Scells being activated are OD-SSB based Scells.

- Both known and unknown SCell activation in FR1 and FR2 should be supported

Proposal 8: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates.

- The additional processing time is 5ms or 2ms based on UE capability reporting.

- The additional processing time is only applied to L1 and L3 measurement transition period after the SCell is activated.

Proposal 9: No UE additional processing time is allowed for OD-SSB deactivation.

Proposal 10: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary in the following scenario.

- UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5]s and then the OD-SSB is activated again with the same spatial reception parameter.

Proposal 11: The interruption duration of OD-SSB activation/deactivation shall be 0.5ms.

Proposal 12: The The interruption due to OD-SSB activation/deactivation is applied to SCell addition/release, SCell activation/deactivation and deactivated SCell measurement.

Proposal 13: The interruption length due to OD-SSB activation/deactivation is independent with other active serving cells which are contiguous in FR1 or in the same band in FR2.

- References

- R4-2508277, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R2-2504981, “LS on L3 measurement of OD-SSB”, Apple