# 3GPP TSG-RAN WG4 Meeting #116	R4-2510917

# Bengaluru, India, Aug 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on OD-SSB based intra-frequency measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 NES enh. WI is nearly completed. The requirements need to be added to spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The general introduction about OD-SSB based measurement of an SCell is added. - OD-SSB based intra-frequency measurement section is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 8.3.x4, 9.x, 9.x.1, 9.x.2, 9.x.3, 9.x.4, 9.x.5, 9.x.5.1, 9.x.5.2, 9.x.5.3, 9.x.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------- CHANGE-------------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [11] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [11].

1 Rx RedCap: RedCap UE for which requirements are derived assuming 1 Rx branch.

2 Rx RedCap: RedCap UE for which requirements are derived assuming 2 Rx branches.

Active DL BWP: Active DL bandwidth part as defined in TS 38.213 [3].

Blackbox Approach: Testing methodology, in which the UE internal implementation of certain specific UE functionality involved in the test, is unknown.

CD-SSB: Cell defining SSB as defined in TS 38.300 [10].

Control Resource Set: As defined in TS 38.213 [3].

DL BWP: DL bandwidth part as defined in TS 38.213 [3].

EN-DC: E-UTRA-NR Dual Connectivity as defined in clause 4.1.2 of TS 37.340 [17].

en-gNB: As defined in TS 37.340 [17].

FH: As defined in TS 38.214 [26].

First SSB: SS/PBCH block as defined in clause 4.4 of TS 38.213 [3].

FR1: Frequency range 1 as defined in clause 5.1 of TS 38.104 [13].

FR2: Frequency range 2 as defined in clause 5.1 of TS 38.104 [13].

gNB: as defined in TS 38.300 [10].

IBM (Independent Beam Management): As defined in TS 38.101-2 [19].

IDC solution: As described in TS 36.300 [24] and TS 38.300 [10].

LMF: as defined in TS 38.305 [22].

Master Cell Group: As defined in TS 38.331 [2].

Multi-Radio Dual Connectivity: Dual Connectivity between E-UTRA and NR nodes, or between two NR nodes, as defined in TS 37.340 [17].

NCD-SSB: Non cell defining SSB as defined in TS 38.300 [10].

ng-eNB: As defined in TS 38.300 [10].

NE-DC: NR-E-UTRA Dual Connectivity as defined in clause 4.1.3.2 of TS 37.340 [17].

NGEN-DC: NG-RAN E-UTRA-NR Dual Connectivity as defined in clause 4.1.3.1 of TS 37.340 [17].

NR-DC: NR-NR Dual Connectivity as defined in clause 4.1.3.3 of TS 37.340 [17].

Primary Cell: As defined in TS 38.331 [2].

PRS resource instance: An instance in time of a configured PRS resource as defined in TS 38.331 [2], which may or not overlap with a measurement gap occasion.

Quasi Co-Location: As defined in TS 38.214 [26].

RedCap UE: A UE with reduced capabilities as defined in clause 4.2 in TS 38.306 [14].

RLM-RS resource: A resource out of the set of resources configured for RLM by higher layer parameter RLM-RS-List [2] as defined in TS 38.213 [3].

SA operation mode: Operation mode when the UE is configured with at least PCell and not any MR-DC.

Secondary Cell: As defined in TS 38.331 [2].

Secondary Cell Group: As defined in TS 38.331 [2].

Serving Cell: As defined in TS 38.331 [2].

SMTC: An SSB-based measurement timing configuration configured by SSB-MeasurementTimingConfiguration as specified in TS 38.331 [2].

Special Cell: As defined in TS 38.331 [2].

SSB: SS/PBCH block as defined in clause 7.8.3 of TS 38.211 [6].

Timing Advance Group: As defined in TS 38.331 [2].

--------------------------------- END OF CHANGES ----------------------------

--------------------------------------- CHANGE-------------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

AWGN	Additive White Gaussian Nouse

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CCE	Control Channel Element

CG-SDT	Configured Grant Small Data Transmission

CHO	Conditional Handover

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CN	Core Network

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DAPS	Dual Active Protocol Stack

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DPC	Delta Power Class

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMP	Effective measurement periodicity

EMR	Early measurement reporting

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

FMW	Fast measurement window

GEO	Geostationary Earth Orbit

GNSS	Global Navigation Satellite System

GSO	Geosynchronous Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

HST	High Speed Train

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

kHz	Kilo Hertz

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MHz	Mega Hertz

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NGSO	Non-Geosynchronous Orbit

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OD-SSB	On-demand SSB

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PCI	Physical Cell Identity

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRB	Physical Resource Block

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PPWL	PRS Processing Window Length

PPWRP	PRS Processing Window Repetition Period

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAN	Radio Access Network

RAT	Radio Access Technology

RF	Radio Frequency

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RRT	RF Retuning Time

RS	Reference Signal

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTD	Receive Timing Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCH	Synchronization Channel

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRP	SL-PRS Received Power

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDCP	Time Domain Channel Properties

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TE	Test Equipment

TN	Terrestrial Network

TRP	Transmission-Reception Point

TRS	Tracking Reference Signal

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

--------------------------------- END OF CHANGES ----------------------------

--------------------------------------- CHANGE-------------------------------------

### 8.3.x4	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells

The requirements in this clause shall apply for the UE configured with more than one downlink OD-SSB based SCells.

In EN-DC, NE-DC, standalone NR carrier aggregation, or in one CG of NR-DC, the requirements in this clause shall apply when the following conditions are met:

-	UE only receives one single MAC command for multiple SCell activation within the activation period defined in this clause

-	in each single CG, there are no other SCell activation, deactivation, addition or release before activation is completed for all the SCells activated by the single MAC CE in this clause, and

-	in EN-DC and NE-DC, there are no E-UTRAN SCell activation, deactivation, addition or release before multiple SCell activation is completed in this clause, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

In two CGs of NR-DC, the requirements in this clause shall apply when the following conditions are met:

-	UE receives one MAC command per CG for multiple SCell activation within the activation period defined in this clause, and

-	UE supports per-FR measurement gap capability, and

-	any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band

The applicability of OD-SSB based SCell activation defined in clause 8.3.x1 is also applied in this clause.

The transmited SSB defined in clause 8.3.x1 is also applied in this clause for OD-SSB based multiple downlink SCells activation.

The delay within which the UE shall be able to activate the deactivated SCell with other downlink to-be-activated SCell(s) depends upon the specified conditions. The deactivated SCell and other downlink to-be-activated SCell(s) are OD-SSB based SCells.

Upon receiving SCell activation command in slot n for more than one SCell, for each of the to-be-activated SCell, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot  n+THARQ+Tactivation_time_multiple_scells+TCSI_ReportingNR slot length, where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time_multiple_scells is the target SCell activation delay in milliseconds in multiple SCell activation scenario.

If the SCell is known and belongs to FR1 and the measurement period of the SCell being activated is equal to or smaller than 2400ms, Tactivation_time_multiple_scells is:

-	TFirst, OD-SSB_multiple_scells + Trs, OD-SSB + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 known SCell with the measurement period larger than 2400ms but does not have any parallel to-be-activated SCell which is FR1 unknown SCell.

-	TFirst, OD-SSB_multiple_scells + 2*Trs, OD-SSB + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell

-	otherwise, TFirst, OD-SSB_multiple_scells + 5ms.

If the SCell is known and belongs to FR1 and the measurement period of the SCell being activated is larger than 2400 ms, Tactivation_time_multiple_scells is:

-	TFirst, OD-SSB_multiple_scells + 2*Trs, OD-SSB + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell

-	otherwise, TFirst, OD-SSB_multiple_scells + Trs, OD-SSB + 5 ms

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time_multiple_scells is:

-	TFirst, OD-SSB_multiple_scells + 2*Trs, OD-SSB +5 ms, if the SCell is not counted in N1

-	The activation delay may be longer if SSB is not in the same half-frame on the SCell and the contiguous FR1 known cell or contiguous FR1 active serving cell

otherwise

-	if the following conditions are met

-	its od-ssb-PositionInBurst if provided; otherwise, by ssb-PositionsInBurst indicates only one SSB is being actually transmitted, or

-	its od-ssb-PositionInBurst if provided; otherwise, by ssb-PositionsInBurst indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

-	Tactivation_time_multiple_scells is:

-	TFirst, OD-SSB_multiple_scells + Trs, OD-SSB +Trs, OD-SSB*N1 +Trs, OD-SSB +5 ms,

-	Otherwise:

Tactivation_time_multiple_scells is:

-	6 ms + TFirst, OD-SSB_multiple_scells + Trs, OD-SSB + Trs, OD-SSB*N1 + TL1-RSRP, measure, OD-SSB + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC_multiple_scells + TFineTiming, OD-SSB + 2 ms, Tuncertainty_SP_multiple_scells), if semi-persistent CSI-RS is used for CSI reporting,

-	3 ms + TFirst, OD-SSB_multiple_scells + Trs, OD-SSB + Trs, OD-SSB*N1 + TL1-RSRP, measure, OD-SSB + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC_multiple_scells + 5 ms + TFineTiming, OD-SSB, Tuncertainty_RRC_multiple_scells + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.x1.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.x1.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.x1.

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.x1.

If the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_multiple_scells is:

-	3 ms + max(Tuncertainty_MAC_multiple_scells +TFineTiming, OD-SSB + 2ms, Tuncertainty_SP_multiple_scells), if on the same band UE also has at least one parallel to-be-activated SCell which is FR2 known SCell. Tuncertainty_MAC_multiple_scells =0 and Tuncertainty_SP_multiple_scells =0 if

-	UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	TCI-ActivatedConfig is configured for the SCell, and UE receives the SCell activation command and semi-persistent CSI-RS activation command at the same time.

If the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_multiple_scells is:

-	max(Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, OD-SSB, Tuncertainty_RRC_multiple_scells + TRRC_delay-THARQ), if on the same band UE also has at least one parallel to-be-activated SCell which is FR2 known SCell . Tuncertainty_MAC_multiple_scells =0 if:

-	UE receives the SCell activation command and TCI state activation commands at the same time, or

-	Provided this is the initial activation after SCell addition, and

-	UE receives the SCell activation command and TCI-ActivatedConfig is configured for the SCell.

The requirements for FR2 unknown SCells apply provided that the parameter od-ssb-PositionsInBurst if provided; otherwise, by ssb-PositionsInBurst is same for the SCell and the known serving cell on the same FR2 band. The activation delay FR2 unknown SCell may be longer if SSB is not in the same half-frame on the SCell and the FR2 known cell on the same band.

Where,

N1 is the number counting for parallel FR1 unknown to-be-activated SCell(s) only except the ones which fulfilled the following conditions:

-	contiguous to an active serving cell in the same band, or to a known SCell in the same band being activated by the same MAC PDU, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its od-ssb-PositionsInBurst if provided; otherwise, by ssb-PositionInBurst is same as the one of contiguous FR1 known cell or contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 known cell or contiguous FR1 active serving cell is smaller than or equal to 260 ns and its reception power difference with contiguous FR1 known cell or contiguous FR1 active serving cell is smaller than or equal to 6 dB, and

-	its transmitted SSB offset is same as the one of contiguous FR1 known cell or contiguous FR1 active serving cell

However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell and other SCells being activated and counted in N1:

-	contiguous to an active serving cell in the same band, or to a known SCell in the same band being activated by the same MAC PDU, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the unknown SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its od-ssb-PositionsInBurst if provided; otherwise, by ssb-PositionInBurst is same as the one of FR1 known cell or FR1 active serving cell, and

-	its RTD with contiguous FR1 known cell or contiguous FR1 active serving cell is larger than 260 ns or its reception power difference with contiguous FR1 known cell or contiguous FR1 active serving cell is larger than 6 dB, and

-	its transmitted SSB offset is same as the one of FR1 known cell or FR1 active serving cell

TFirst, OD-SSB_multiple_scells is the time to the end of the first complete transmitted SSB burst after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCells being activated are transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

Tuncertainty_MAC_multiple_scells is the time period between reception of the activation command for PDCCH TCI, PDSCH TCI (when applicable) and SCell activation command of this unknown SCell.

Tuncertainty_SP_multiple_scells is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting and SCell activation command of this unknown SCell.

Tuncertainty_RRC_multiple_scells is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) and SCell activation command of this unknown SCell.

Trs, OD-SSB, TFineTiming, OD-SSB, and TRRC_delay is defined in clause 8.3.x1.

Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

The condition of known SCell in FR1 or FR2 is defined in clause 8.3.x1.

The starting point and the end-point of an interruption window on PCell or any activated SCell in MCG for NR standalone mode, or on PSCell or any activated SCell in SCG for EN-DC mode is same as single SCell activation requirement in clause 8.3.2.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Upon receiving SCell activation command in slot n, if the start of the first complete transmitted SSB used in the TX in the different bands which have SCells being activated after n+THARQ+3msNR slot length are not aligned on time domain among

-	SCells in different bands being activated by the same MAC CE if UE does not support per FR gap, or

-	SCells in different FR1 bands being activated by the same MAC CE if UE supports per FR gap,

additional interruptions may be expected for the activated serving cells, where

-	The number of additional interruptions is no more than the number of FR1 bands which have both SCell being activated for which the activation requirements involve TFirst, OD-SSB_multiple_scells with Trs, OD-SSB and the active serving cell, and

-	In each interruption occasion, the interruption length is defined in clause 8.2.2.2.2, and

-	Longer activation delay may be expected for multiple SCell activation under one MAC CE with multiple interruptions, and

-	TX is:

-	TFirst, OD-SSB, for any scenario where Tactivation_time multiple_scells includes TFirst, OD-SSB;

-	TFirst, OD-SSB_multiple_scells, for any scenario where Tactivation_time multiple_scells includes TFirst, OD-SSB_multiple_scells;

-	Tuncertainty_MAC+ TFineTiming, OD-SSB or Tuncertainty_MAC multiple_scells+ TFineTiming, OD-SSB, for any scenario where Tactivation_time multiple_scells includes TFineTiming.

Otherwise, no additional interruption is expected due to activation of multiple SCells.

Starting from slot n + THARQ + 3 ms where slot n is the slot where SCell activation command is received (as specified in clause 4.3 of TS 38.213 [3]) and until the SCell activation completion at UE, after at least one CSI-RS transmission occasion for the channel measurement and reporting (specified in clause 5.2.2.5 of TS 38.214 [26]), the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

--------------------------------- END OF CHANGES ----------------------------

--------------------------------------- CHANGE-------------------------------------

## 9.x	OD-SSB based L3 measurement for an SCell

### 9.x.1	Introduction

A UE can be indicated, by od-ssb-config TS 38.331[2] or by a MAC CE TS 38.321[7], activation of transmission for OD-SSB in a configured DL BWP of an SCell TS 38.300[10] if UE supports On-demand SSB operation.

The requirements in clause 9.x apply, provided the following conditions are met:

-	an OD-SSB transmission in a configured DL BWP without SSB transmission, or

-	an OD-SSB transmission in a configured DL BWP with a first SSB transmission which cannot be used to obtain SIB1 have the following relations:

-	are with the same Cell ID, and

-	are in the same frequency location, and

-	are with same sub-carrier spacing, and

-	are with the same downlink transmit power, and

-	are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SSB index.

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for OD-SSB based intra-frequency measurement are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated.

### 9.x.2	Requirements Applicability

The accuracy requirements in clause 9.2.2 shall be applied to OD-SSB based intra-frequency measurement.

### 9.x.3	Number of cells and number of SSB

The requirements of number of cells and number of SSB in clause 9.2.3 shall be applied to OD-SSB based intra-frequency measurement.

### 9.x.4	Measurement Reporting Requirements

The measurement reporting requirements in clause 9.2.4 shall be applied to OD-SSB based intra-frequency measurement.

### 9.x.5	OD-SSB based Intra-frequency measurements without measurement gaps

#### 9.x.5.1	Intra-frequency cell identification for active SCell

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index_ODSSB if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise the UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_ODSSB. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_ODSSB. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index_ODSSB = (TPSS/SSS_sync_intra_ODSSB + TSSB_measurement_period_intra_ODSSB) ms

Tidentify_intra_with_index_ODSSB = (TPSS/SSS_sync_intra_ODSSB + T SSB_measurement_period_intra_ODSSB + TSSB_time_index_intra_ODSSB) ms

Where:

TPSS/SSS_sync_intra_ODSSB: it is the time period used in PSS/SSS detection

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_intra_ODSSB is given in table 9.x.5.1-4; otherwise, TPSS/SSS_sync_intra_ODSSB is given in table 9.x.5.1-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra_ODSSB is given in table 9.2.5.1-1 for FR1 and table 9.2.5.1-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra_ODSSB is given in table 9.x.5.1-5 for FR1 and table 9.x.5.1-6 for FR2.

-	Otherwise, TPSS/SSS_sync_intra_ODSSB is given in tables 9.x.5.1-1, 9. x.5.1-2, or 9.x.5.1-3.

TSSB_time_index_intra_ODSSB: it is the time period used to acquire the index of the SSB being measured

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra_ODSSB is given in table 9.2.x.1-3 for FR1. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra_ODSSB is given in table 9.x.5.1-7 for FR1.

-	Otherwise, TSSB_time_index_intra_ODSSB is given in table 9.x.5.1-3.

TSSB_measurement_period_intra_ODSSB: equal to a measurement period of SSB based measurement

-	For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-4; otherwise, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-2.

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-1 for FR1 and table 9.x.5.2-2 for FR2. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-5 for FR1 and table 9.x.5.2-6 for FR2.

-	For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2 is configured, the TSSB_measurement_period_intra_ODSSB given in table 9.x.5.2-4 (if SMTC ≤ 40ms) and table 9.x.5.2-2 (if SMTC > 40ms) shall apply for SCC.

-	Otherwise, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-1, 9.x.5.2-2, 9.x.5.2-3 or 9.x.5.2-4.

CSSFintra: it is a carrier specific scaling factor and is determined	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating no-gap-with-interruption, or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating no-gap-no-interruption

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24.  For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync_w/o_gaps =24.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting power class 6, Mmeas_period_w/o_gaps =24.

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

When UE supports concurrent GAPs and UE concurrent GAPs, i.e., supports the following capability or capabilities’ combination:

-	concurrentMeasGap-r17, or

-	concurrentMeasGapsPreMG-r18, or

-	concurrentMeasGapsNCSG-r18,

Or when UE supports musim-GapPreference-r17 or both concurrent measurement GAPs and musim-GapPreference-r17 and UE concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

Otherwise, when the UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with periodic MUSIM gaps or UE does not support MUSIM gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

For FR2,

Klayer1_measurement =1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement =1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

When UE supports MUSIM-GapConfig-17 and the SMTC occasion of the target frequency layer is overlapping with the configured aperiodic MUSIM gap, longer cell identification period for the target frequency layer is expected.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

If the higher layer signaling in TS 38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.x.5.1-1, Table 9.x.5.1-2, Table 9.x.5.1-3, and Table 9.x.5.1-4 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.x.5.1-1, Table 9.x.5.1-2, Table 9.x.5.1-3 and Table 9.x.5.1-4 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement shall be conducted without gap and without interruption regardless of the NeedForGaps’ status reporting.

Table 9.x.5.1-1: Time period for OD-SSB based PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | max( 600 ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 9.x.5.1-2: Time period for OD-SSB based PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.x.5.1-3: Time period for OD-SSB based time index detection (FR1)

| DRX cycle | TSSB_time_index_intra_ODSSB |
| --- | --- |
| No DRX | max(120 ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 9.x.5.1-4: Time period for OD-SSB based PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2) when SMTC period ≤ 40ms

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.x.5.1-5: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | max( 600 ms, 5 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max( 600 ms, ceil(M2 Note 2x 5) x max(80 ms, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.x.5.1-6: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | max(600 ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max(80 ms, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.x.5.1-7: Time period for time index detection for UE indicating no-gap-with-interruption (FR1)

| DRX cycle | TSSB_time_index_intra_ODSSB |
| --- | --- |
| No DRX | max(120 ms, 3 x max (80 ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(120 ms, ceil (M2 Note 2 x 3) x max(80 ms, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption. |  |

Table 9.x.5.1-8: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil(7 x Kp) x SMTC period) x CSSFintra |
| DRX cycle≤ 320 ms | max(120ms, ceil(M2 x 7 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(7 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |

#### 9.x.5.2	Measurement period for active SCell

The measurement period for intra-frequency measurements without gaps is as shown in table 9.x.5.2-1, 9.x.5.2-2, 9.x.5.2-3, 9.x.5.2-4.

When UE highSpeedMeasFlagFR2-r17 is configured, if SMTC ≤ 40ms, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-3; otherwise, TSSB_measurement_period_intra_ODSSB is given in table 9.x.5.2-2. For power class 6 UE supporting measEnhCAInterFreqFR2-r18 when highSpeedMeasFlagFR2-r17 is configured, the TSSB_measurement_period_intra_ODSSB is given in table 9.2.5.2-3 (if SMTC ≤ 40ms) and table 9.2.5.2-2 (if SMTC > 40ms) shall apply for SCC.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

If MCG DRX is in use, measurement period requirements for intra-frequency measurement in MCG specified in table 9.x.5.2-1, table 9.x.5.2-2, table 9.x.5.2-3 and table 9.x.5.2-4 shall depend on the MCG DRX cycle. If SCG DRX is in use, measurement period requirements for intra-frequency measurement in SCG specified in table 9.x.5.2-1, table 9.x.5.2-2, table 9.x.5.2-3 and table 9.x.5.2-4, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 9.x.5.2-1: Measurement period for OD-SSB based intra-frequency measurements without gaps (FR1)

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX | max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.x.5.2-2: Measurement period for OD-SSB based intra-frequency measurements without gaps (FR2)

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX | max(400 ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(400 ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.x.5.2-3: TSSB_measurement_period_intra_ODSSB when highSpeedMeasCA-Scell-r17 is configured (Frequency range FR1)

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX Note 2 | max(200 ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 160 ms | max(200 ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| 160 ms < DRX cycle≤ 320 ms | ceil(4 x M2 Note 2 x Kp) x DRX cycle x CSSFintra |
| DRX cycle>320 ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC period > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC period ≤ 40ms, Y=5 when SMTC period > 40ms NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to measurements of secondary component carrier with active SCell. |  |

Table 9.x.5.2-4: Measurement period for OD-SSB based intra-frequency measurements without gaps when highSpeedMeasFlagFR2-r17 is configured (FR2) when SMTC period <= 40 ms

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX | max(400 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 80 ms | max(400 ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| 80 ms< DRX cycle≤ 320 ms | ceil(1.5 x Mmeas_period_w/o_gaps Note 3 x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle) x CSSFintra |
| DRX cycle>320 ms | ceil(Mmeas_period_w/o_gaps Note 3 xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.x.5.2-5: Measurement period for intra-frequency measurements without gaps for UE indicating no-gap-with-interruption (FR1)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200 ms, 5 x (80 ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(200 ms, ceil(1.5x 5) x max(80 ms, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | 5 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates ‘no-gap-with-interruption’ via NeedForInterruptionInfoNR-r18. |  |

Table 9.x.5.2-6: Measurement period for intra-frequency measurements without gaps for UE indicating no-gap-with-interruption (FR2)

| DRX cycle | TSSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400 ms, ceil(Mmeas_period_w/o_gaps x Klayer1_measurement) x (80 ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320 ms | max(400 ms, ceil(1.5x Mmeas_period_w/o_gaps x Klayer1_measurement) x max(80, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | ceil(Mmeas_period_w/o_gaps x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates no-gap-with-interruption via NeedForInterruptionInfoNR-r18. |  |

#### 9.x.5.3	Intra-frequency cell identification for deactivated SCell

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell without a first SSB transmission before SCell activation command,

-	After slot n+THARQ+3ms+TprocessingNR slot length, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length, where:

-	TFMW equals min(Treport, Tidentify_intra), which is the duration of the fast measurement window(FMW).

-	Tidentify_intra equals Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB if the cell is a newly detectable cell or TSSB_measurement_period_intra_ODSSB if the cell has been detected for a period less than 5 seconds.

-	Tprocessing is the additional processing time as 2ms or 5ms depending on OD-SSB post processing time.

-	Treport is the duration after slot n+THARQ+3ms+TprocessingNR slot length to the earliest OD-SSB based L3 measurement reporting.

-	CSSFintra: it is a carrier specific scaling factor and is determined	according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps. When single OD-SSB carrier is configured with multiple SCCs without OD-SSB,  CSSFintra equals 1 for OD-SSB carrier during the FMW.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell within the T2 after the previous OD-SSB deactivation and before the SCell activation command, the UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if OD-SSB is transmitted, where T2 is 5*max (measCycleSCell, DRX cycles) in FR1 and 4s for power class 1/5 and 3s for power class 2/3/4 in FR2.

- When UE is indicated the activation of another OD-SSB transmission in a configured DL BWP of an SCell after previous OD-SSB deactivation plus T2 and before the SCell activation command, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length.

-	After the FMW, UE shall follow the legacy deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5 9.x.5.4-2 and 9.x.5.4-3 if the OD-SSB transmission is not deactivated.

.

No OD-SSB based deactivated SCell requirement applies if NW deactivates the OD-SSB earlier than the time slot n+THARQ+3ms+Tprocessing+min(Treport, Tidentify_intra)NR slot length.

When UE is indicated the activation of OD-SSB transmission in slot n in a configured DL BWP of an SCell with a first SSB transmission before SCell activation command, after slot n+THARQ+3ms+TprocessingNR slot length,

- UE shall perform the deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if UE has sent a valid measurement report before receiving the activation of OD-SSB transmission during the period equal to max(5*measCycleSCell,  5*DRX cycles) in FR1 or 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2.

- Otherwise, UE shall perform the deactivated SCell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-1, 9.x.5.3-2 and 9.x.5.4-1 regardless of configured DRX and measCycleSCell till slot n+THARQ+3ms+Tprocessing+TFMWNR slot length..

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, UE shall perform deactivated SCC measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.4-2 and 9.x.5.4-3 if OD-SSB is transmitted.

- Before UE receiving the activation of OD-SSB transmission or after OD-SSB deactivation in a deactivated SCell, for UE indicating support3MHz-ChannelBW-Symmetric-r18 and configured to perform measurements on a deactivated SCell with 12PRB SSB in FR1, TSSB_time_index_intra_less_than_5MHz_ODSSB is given in table 9.x.5.3-6.

UE shall perform the deactivated SCC neighbour cell measurement with TPSS/SSS_sync_intra_ODSSB, TSSB_time_index_intra_ODSSB, TSSB_measurement_period_intra_ODSSB given in table 9.x.5.3-3, 9.x.5.3-4, 9.x.5.3-5, 9.x.5.3-6, 9.x.5.4-2 and 9.x.5.4-3 regardless of OD-SSB activation/deactivation.

If MCG DRX is in use, cell identification requirements for intra-frequency measurement in MCG specified in Table 9.x.5.3-2, Table 9.x.5.3-3, Table 9.x.5.3-4, 9.x.5.3-5 and Table 9.x.5.3-6 shall depend on the MCG DRX cycle. If SCG DRX is in use, cell identification requirements for intra-frequency measurement in SCG specified in Table 9.x.5.3-2, Table 9.x.5.3-3, Table 9.x.5.3-4, 9.x.5.3-5 and Table 9.x.5.3-6 shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.x.5.3-1: Time period for OD-SSB based PSS/SSS detection in FMW, deactivated SCell

| Frequency range | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| FR1 | ceil(M Note 2 x Kp) x max(Tlower Note 1, EMP) x CSSFintra |
| FR2 | ceil(M Note 2 x KFR x Kp x Klayer1_measurement) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. NOTE 2: When multiple frequency layers are configured, M equals 6 in FR1 and Mpss/sss_sync_w/o_gaps+1 in FR2. Otherwise, M equals 5 in FR1 and Mpss/sss_sync_w/o_gaps in FR2. |  |

Table 9.x.5.3-2: Time period for OD-SSB based time index detection in FMW, deactivated SCell (FR1)

| FR | TSSB_time_index_intra_ODSSB |
| --- | --- |
| FR1 | ceil( 3 x Kp ) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. |  |

Table 9.x.5.3-3: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR1)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-4: Time period for OD-SSB based PSS/SSS detection, deactivated SCC (FR2)

| DRX cycle | TPSS/SSS_sync_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mpss/sss_sync_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-5: Time period for OD-SSB based time index detection, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(3 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(3 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

Table 9.x.5.3-6: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCC (FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5MHz |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements apply to CC with deactivated SCG SCell. |  |

#### 9.x.5.4	Measurement period for deactivated SCell

The measurement period for deactivated SCell measurements is as shown in table 9.x.5.4-1, 9.x.5.4-2, 9.x.5.4-3.

If MCG DRX is in use, measurement period requirements for intra-frequency measurement in MCG specified in Table 9.x.5.4-2, and Table 9.x.5.4-3 shall depend on the MCG DRX cycle. If SCG DRX is in use, measurement period requirements for intra-frequency measurement in SCG specified in 9.x.5.4-2, and Table 9.x.5.4-3, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

For FR2, a longer measurement period is allowed, if aperiodic CSI-RS resource is measured for L1-RSRP measurement on any FR2 serving frequency in the same band, and the CSI-RS resource is outside measurement gap and overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols. If SSB-ToMeasure or SS-RSSI-Measurement is configured, the SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same band which can be merged and the RSSI symbols are indicated by SS-RSSI-Measurement.

Table 9.x.5.4-1: Measurement period for OD-SSB based intra-frequency measurements without gaps (deactivated SCC)

| FR | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| FR1 | ceil(M Note 2 x Kp) x max(Tlower Note 1, EMP) x CSSFintra |
| FR2 | ceil(M Note 2 x KFR x Kp x Klayer1_measurement) x max(Tlower Note 1, EMP) x CSSFintra |
| NOTE 1: When UE reports not supporting lower bound capability, the Tlower equals 10ms; Otherwise, Tlower equals 0ms. NOTE 2: When multiple frequency layers are configured, M equals 6 in FR1 and Mpss/sss_sync_w/o_gaps+1 in FR2 if the deactivated SCell is a detected Scell. Otherwise, M equals 5 in FR1 and Mpss/sss_sync_w/o_gaps in FR2. |  |

Table 9.x.5.4-2: Measurement period for OD-SSB based intra-frequency measurements without gaps (deactivated SCC) (FR1)

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG |  |

Table 9.x.5.4-3: Measurement period for OD-SSB based intra-frequency measurements without gaps (deactivated SCC) (FR2)

| DRX cycle | TSSB_measurement_period_intra_ODSSB |
| --- | --- |
| No DRX | Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320 ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320 ms | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to CC with deactivated SCell in SCG. |  |

--------------------------------- END OF CHANGES ----------------------------