3GPP TSG-RAN4 Meeting # 116	R4-2510935

Bangalore, IN. 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1: Introduction of requirements for 6Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, Huawei, HiSilicon, AT&T, vivo, Apple, OPPO, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ∆TRxSRS requirements for 6Rx are agreed in the WF R4-2503014, and the draft CR for the Rx related part have already been endorsed in R4-2417094. This CR is to introduce it in the R19 specification, based on the |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the RF requirements for 6Rx. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 6Rx feature is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | [3.1], 3.2, [3.3], 6.2.4, 7.2, 7.3.1, 7.3.2, 7.3G, 7.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change >

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial UE: A UE supporting UAS (Uncrewed Aircraft Systems) as indicated by the capability aerialUE-Capability-r18 [15] and that has an aerial subscription as described in TS 23.256 [17]. The UE is considered to have access to UAS services after the UE ha performed a successful authentication and authorization with the USS as described in TS 23.256 [17].

Aggregated Allocation Bandwidth: Total bandwidth of all allocated RBs in a transmission occasion. Can be calculated for two aggregated CCs as LCRB, 1* 12* SCS1 + LCRB,2 * 12 * SCS2 .

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

ATG UE: The terminals or user equipments which are mounted in aircraft and support ATG feature (i.e. UE capability airToGroundNetwork-r18) as defined in clause 4.2.2 from TS38.306[15].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Concurrent operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

Enhanced channel raster: channel raster with a 10 kHz granularity in bands with a 100 kHz channel raster.

eRedCap UE: The UE with enhanced reduced capabilities as defined in clause 4.2.22.1 from TS38.306 [15].

[FWA CPE: A CPE device mounted either inside or outside the home or business premises and used for FWA.]

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Inter-band concurrent operation: Operation of NR Uu carrier and NR Sidelink carrier in different operating bands.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Intra-band SL CA UE: UE that supports NR SL CA operation in a single band

NR SL CA: Aggregation of two or more NR Sidelink component carriers in order to support wider transmission bandwidths

NR SL inter-band concurrent operating Band：Band combinations of NR Uu carrier and NR Sidelink carrier in different operating bands.

NR SL-U UE: UE that supports NR Sidelink operation in unlicensed bands (e.g. n46, n96, n102).

Railway Mobile Radio: railway operations encompassing GSM-R and its successor(s), including the Future Railway Mobile Communication System (FRMCS); in the context of this specification the Railway Mobile Radio is limited to NR operation in band n100, or n101.

RedCap UE: The UE with reduced capabilities as defined in clause 4.2.21.1 from TS38.306 [15].

Sub-band:  For a UE that supports shared spectrum channel access in wideband operation, a sub-band is the set of RBs within an approximately 20 MHz segment of the channel where the wideband channel is uniformly divided into an integer number of 20 MHz sub-bands.  Sub-bands may be separately allocated in uplink and downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands.

NOTE:	Vehicular UE does not refer to other UE form factors placed inside the vehicle.

Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets

<Next change >

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔFTX-RX	Maximum deviation to the Tx-Rx carrier center frequency separation for asymmetric uplink/downlink channel bandwidth operation

∆MPRc	Allowed Maximum Power Reduction relaxation for serving cell c

ΔPPowerClass	Adjustment to maximum output power for a given power class

RB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔRIBC	Allowed reference sensitivity relaxation due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed reference sensitivity relaxation due to support for intra-band non-contiguous CA operation

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports

ΔRIB,6R	Reference sensitivity adjustment due to support for 6 antenna ports

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports

ΔR1R	Reference sensitivity adjustment due to support for 1 antenna ports

ΔRXR,2R	Reference sensitivity adjustment for two antenna ports XR UEs on bands defined in Table 7.3.2-2b

ΔShift	Channel raster offset

TC	Allowed operating band edge transmission power relaxation

TC,c	Allowed operating band edge transmission power relaxation for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

BWChannel	Channel bandwidth

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWChannel,max	Maximum channel bandwidth supported among all bands in a release

BWGB	max(GBChannel,low, GBChannel,high)

BWDL	Channel bandwidth for DL

BWUL	Channel bandwidth for UL

BWinterferer	Bandwidth of the interferer

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC,low	The Fc of the lowest carrier, expressed in MHz

FC,high	The Fc of the highest carrier, expressed in MHz

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge , low	The lower edge of aggregated channel bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge, high	The higher edge of aggregated channel bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured). For intra-band contiguous CA, the FInterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

<Next change >

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

ATG	Air-To-Ground

BS	Base Station

BW	Bandwidth

BWP	Bandwidth Part

CA	Carrier Aggregation

CA_nX-nY	Inter-band CA of component carrier(s) in one sub-block within Band nX and component carrier(s) in one sub-block within Band nY where nX and nY are the applicable NR operating bands

CC	Component Carriers

CG	Carrier Group

CP-OFDM	Cyclic Prefix-OFDM

[CPE	Customer Premises Equipment]

CW	Continuous Wave

DC	Dual Connectivity

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

E-UTRA	Evolved UTRA

EIRP	Equivalent Isotropically Radiated Power

(e)RedCap	Redcap or eRedCap

eRedCap	enhanced Reduced Capability

EVM	Error Vector Magnitude

FR	Frequency Range

FRC	Fixed Reference Channel

FRMCS	Future Railway Mobile Communication System

FWA	Fixed Wireless Access

GSCN	Global Synchronization Channel Number

HD	Half Duplex

IBB	In-band Blocking

IDFT	Inverse Discrete Fourier Transformation

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

PS	Public Safety

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSFCH	Physical Sidelink Feedback CHannel

PSSCH	Physical Sidelink Shared CHannel

QAM	Quadrature Amplitude Modulation

RE	Resource Element

REFSENS	Reference Sensitivity

RedCap	Reduced Capability

RF	Radio Frequency

RMR	Railway Mobile Radio

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving PowerRx	Receiver

Rx	Receiver

SC	Single Carrier

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SDL	Supplementary Downlink

SEM	Spectrum Emission Mask

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna transmission

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

S-SSB	Sidelink Synchronization Signal Block

SUL	Supplementary uplink

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TAG	Timing Advance Group

Tx	Transmitter

TxD	Tx Diversity

UAS	Uncrewed Aircraft Systems

UAV	Uncrewed Aerial Vehicle

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

USS	UAS Service Supplier

V2X	Vehicle to Everything

XR	eXtended Reality

<Next change >

6.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

where

PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PPowerClass is the maximum UE power specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation, without taking into account the tolerance specified in the Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation;

When  the IE powerBoostPi2BPSK is set to 1, PEMAX,c is increased by +3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with PI/2 BPSK modulation with Rel-15 DMRS and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less symbols in certain evaluation period are used for UL transmission when PEMAX,c ≥ 20 dBm (The exact evaluation period is no less than one radio frame).

When the IE powerBoostPi2BPSK is set to 1, ΔPPowerClass = -3 dB for a power class 3 UE operating in TDD bands n40, n41, n77, n78, and n79 with Pi/2 BPSK modulation with Rel-15 DMRS and UE indicates support for UE capability powerBoosting-pi2BPSK and 40% or less slots in radio frame are used for UL transmission.

ΔPPowerClass =

-	3 dB for a power class 2 UE or 6 dB for a power class 1.5 UE when P-max of 23 dBm or lower is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3 dB for a power class 1.5 UE when P-max of between 23 dBm and 26 dB is indicated; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1/2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame).

-	3dB when the UE is configured with SUL configurations and the requirements of default power class are applied as specified in sub-clause 6.2C.1 on the band where UE indicates power class 2;

-	3dB is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ with configured SRS resources in each SRS resource set(s) consisting of one SRS port when PC2 UE with txDiversity-r16 or txDiversity2Tx-r18 capability or PC1.5 UE further indicates SRS-TxSwitch capability ‘t1r2’ or ‘t1r4’ or ‘t1r1-t1r2’ or ‘t1r1-t1r2-t1r4’ or further indicates srs-AntennaSwitchingBeyond4RX-r17 as ‘t1r6’ or ‘t1r8’;

-	0 dB otherwise;

NOTE:	UE reports ∆PPowerClass when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the ue-PowerClass after the duty cycle exceedance.

∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise; In case the UE supports more than one of band combinations for V2X operating bands for concurrent operation, CA, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ∆TIB,c shall be the average value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ∆TIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ∆TIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.101-3 [3] for the applicable operating bands.

ΔPPowerBoost is defined as 1dB for power class 3 and 0.5dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

-	0 dB otherwise;

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

MPRc and A-MPRc for serving cell c are specified in clause 6.2.2 and clause 6.2.3, respectively and in clause 6.2F.2 and clause 6.2F.3 respectively for shared spectrum access operation;

∆MPRc for serving cell c is specified in clause 6.2.2 and in clause 6.2F.2 for shared spectrum access operation.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2' or 't1r1-t1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4' or 't1r1-t1r2-t1r4' or, 't1r1-t1r2-t2r2-t1r4-t2r4'

c)	UE transmits SRS from the SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or 't1r1-t1r2-t2r2-t2r4' or 't1r1-t1r2-t2r2-t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

e)	UE transmits SRS on the second, third, fourth, fifth, sixth, seventh and eighth SRS resources of the total 8 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r8', or

f)	UE transmits SRS from the SRS port pair on the second, third and fourth SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r8', or

g) 	UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of four SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't4r8',

h)  UE transmits SRS on the second, third, fourth, fifth and sixth SRS resources of the total 6 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r6', or

i) UE transmits SRS from the SRS port pair on the second and third SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r6', or

[j) UE transmits SRS from the set of SRS ports on the second SRS resource in every configured SRS resource set consisting of the SRS ports as indicated by fourPortSRS-3Tx when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't3r6'],

The following ∆TRxSRS applies according to the indicated SRS-TxSwitch or srs-AntennaSwitchingBeyond4RX-r17 capabilities:

if  't1r8' and 't4r8' are indicated:

-	The value of ∆TRxSRS is 7.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 5.8 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 10.3 dB for bands whose FUL_high is higher than the FUL_low of n79 and 8.8 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' and 't2r8' are indicated:

-	The value of ∆TRxSRS is 6.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.5 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating txDiversity-r16.

-	The value of ∆TRxSRS is 9.0 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if 't1r8' or 't2r8', but not both is indicated:

-	The value of ∆TRxSRS is 5.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.5 dB for bands whose FUL_high is higher than the FUL_low of n79 and 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  't1r2', 't1r1-t1r2', 't1r4', 't1r4-t2r4', 't1r1-t1r2-t1r4', 't2r4', 't1r1-t1r2-t2r2-t2r4', 't1r1-t1r2-t2r2-t1r4-t2r4' or 't4r8' is indicated:

-	The value of ∆TRxSRS is 4.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 7.5dB for bands whose FUL_high is higher than the FUL_low of n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  't1r6' ,‘t2r6’and 't3r6' are all indicated:

-	The value of ∆TRxSRS is 8.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 5.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 7.0dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 11.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 8.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 10.0dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  ‘t2r6’and 't3r6' are indicated:

-	The value of ∆TRxSRS is 6.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 5.5dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 9.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 8.5dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  ‘t1r6’and 't3r6' are indicated or ‘t1r6’and 't2r6' are indicated:

-	The value of ∆TRxSRS is 7.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 4.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 6.0 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 only for ‘t1r6’and 't2r6' in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability only for ‘t1r6’and 't2r6'.

-	The value of ∆TRxSRS is 10.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 9.0 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  ‘t3r6’ is indicated:

-	The value of ∆TRxSRS is 5.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 3.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 4.5 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.0 dB for bands whose FUL_high is higher than the FUL_low of n104, 6.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 7.5 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  ‘t2r6’ is indicated:

-	The value of ∆TRxSRS is 5.5 dB for bands whose FUL_high is higher than the FUL_low of n104, 3.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 5.0 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 8.5 dB for bands whose FUL_high is higher than the FUL_low of n104, 7.5 dB for bands whose FUL_high is lower than the FUL_low of n79 and 8.0 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

else, if  ‘t1r6’ is indicated:

-	The value of ∆TRxSRS is 6.5 dB for bands whose FUL_high is higher than the FUL_low of n104, 4.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 5.5 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

-	The value of ∆TRxSRS is 9.5 dB for bands whose FUL_high is higher than the FUL_low of n104, 7.0 dB for bands whose FUL_high is lower than the FUL_low of n79 and 8.5 dB for bands whose FUL_high is lower than the FUL_low of n104 and higher than FUL_low of n79 during SRS transmission occasions with configured SRS resources consisting of one SRS port when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability.

For other SRS transmissions ∆TRxSRS is zero;

P-MPRc is the power management maximum power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions.

NOTE 2:	P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.4-1. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum  value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2.4-1: Evaluation and reference periods for Pcmax

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4-1. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1-1 and in Table 6.2F.1-1 for shared spectrum access operation.

Table 6.2.4-1: PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

<Next change >

7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.

Unless otherwise stated, the following applicability rules apply,

-	For the single carrier REFSENS requirements in Clause 7:

-	the UE shall be verified with two Rx antenna ports in all supported frequency bands,

-	additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and

-	additional requirements for four and six Rx ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports, and

-	additional requirements for four, six and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports.

-	For Rx requirements other than single carrier REFSENS in Clause 7:

-	the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports,

-	the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports unless the UE does not support six Rx ports for band(s) in which case those band(s) shall be verified with four Rx antenna ports,

-	the UE shall be verified with eight Rx antenna ports and skip two, four and six Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless the UE does not support eight Rx ports for band(s) in a band combination in which case those band(s) shall be verified with six Rx antenna ports, unless the UE does not support six Rx ports for band(s) in which case those band(s) shall be verified with four Rx antenna ports,

-	otherwise, the UE shall be verified with two Rx antenna ports.

-	The above rules apply for all clauses except for clause 7.9.

A (e)Redcap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a (e)RedCap UE.

If aUE indicates intraBandNR-CA-non-collocated-r18, both Rx power imbalance requirements as specified in clause 7.10A and Rx requirements in clauses 7.3 – 7.9 shall be verified.

<Next change >

## 7.3	Reference sensitivity

### 7.3.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In later clauses of Clause 7 where the value of REFSENS is used as a reference to set the corresponding requirement:

-	when the UE is verified with 2 Rx antenna ports, it shall be verified against those requirements by applying the REFSENS value in Table 7.3.2-1a,  Table 7.3.2-1b and Table 7.3.2-1c or Table 7.3.2-1d with 2 Rx antenna ports tested;

-	when the UE is verified with 4 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2 with 4 Rx antenna ports tested.

-	when the UE is verified with 6 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2aa with 6 Rx antenna ports tested.

-	when the UE is verified with 8 Rx antenna ports, it shall be verified against those requirements by applying the resulting REFSENS value derived from the requirement in Table 7.3.2-2a with 8 Rx antenna ports tested.

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d , Table 7.3.2-2, and Table 7.3.2-2a and Table 7.3.2-2aa.

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 | -83.6 | -81.5 |  |  |
|  | 30 |  |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 | -83.7 | -81.6 |  |  |
|  | 60 |  |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 | -83.8 | -81.7 |  |  |
| n3 | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  | 30 |  |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  | 60 |  |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 | -89.2 | -88.6 |  | -81.5 |
|  | 30 |  |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 | -89.3 | -88.7 |  | -81.5 |
|  | 60 |  |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 | -89.4 | -88.9 |  | -81.5 |
| n8 | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
|  | 30 |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |  |  |  |
| n12 | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n18 | 15 |  | -100.0 | -96.8 | -95.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -97.1 | -95.1 |  |  |  |  |  |  |  |
| n20 | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -97.1 |  |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
|  | 30 |  |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -81.8 | -79.6 | -77.7 |  |
|  | 60 |  |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -81.9 | -79.7 | -77.8 |  |
| n26 | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 | -84.5 | -81.7 |  |  |  |  |
|  | 30 |  |  | -94.86 | -92.76 | -87.7 | -84.6 | -81.8 |  |  |  |  |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  |  |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  |  |  |  |
| n30 | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  | 30 |  |  | -96.1 |  |  |  |  |  |  |  |  |
| n31 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 | -90.7 | -90.1 | -89.6 |  |
|  | 30 |  |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 | -90.8 | -90.2 | -89.7 |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 | -90.9 | -90.4 | -89.8 |  |
|  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.19 -74.810 | -82.59 -67.110 | -80.79 -64.010 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.29 -74.910 | -82.69 -67.210 | -80.89 -64.110 |  |  |  |
| n72 | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  | 30 |  |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  | 60 |  |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n100 | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n105 | 15 |  | -97.28 | -94.0 | -91.6 | -86.9 | -85.1 | -83.8 | -82.5 |  |  |  |
|  | 30 |  |  | -94.3 | -91.9 | -87.9 | -85.5 | -84.3 | -82.6 |  |  |  |
| n106 | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS NOTE 9: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 10: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 35, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411, n901 | 15 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 50 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n677 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20,25,30,40,50 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -95.3 + 10log10(NRB/52) | SDL |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| n1041,10 | 15 | 20, 30, 40, 50 | -90.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -90.8 + 10log10(NRB/51) |  |
|  | 60 | 20, 30, 40, 50, 60, 70, 80, 90, 100 | -91.1 + 10log10(NRB/24) |  |
| n10911 | 15 | 5,10,15,20,25,30,40,50 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10,15,20,25,30,40,50 | -97.1 + 10log10(NRB/24) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and two Rx antenna port XR UEs indicating UE capability supportOf2RxXR-r18. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: Void. NOTE 10: A UE may implement two RX antenna ports for band n104 when conditions are met. The exact conditions are FFS. NOTE 11: Applies for DL channels for which channels edges are > 15 MHz away from 2xFUL at 15 kHz SCS and > 30 MHz away from 2xFUL at 30 kHz SCS. In case of UL second harmonic direct hit, the value is modified to -71.9 dBm for all channel bandwidths. |  |  |  |  |

For power class 2 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15].

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 |  | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n5 | 0.5 | 0.5 | 0.7 | 0.8 | 1.5 | 2.6 | 2.8 |  |  |  |  |
| n7 |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |  | 2.0 |
| n8 |  | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n13 |  | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 |  | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 |  | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n26 | 0.5 | 0.5 | 0.7 | 0.8 | 2.2 | 2.6 | 2.8 |  |  |  |  |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  | 3 |  |  |
| n66 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 |  | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 |  | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 0.6 | 0.6 | 1.0 | 2.6 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 |  | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |
| n3 |  | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n5 | 1.1 | 1.1 | 1.2 | 1.4 | 4.5 | 6.1 | 6.5 |  |  |  |  |
| n7 |  | 0.9 | 1.0 | 1.0 | 1.0 | 1.1 | 1.1 | 1.1 | 1.1 |  | 5.3 |
| n8 |  | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n13 |  | 1.2 | 1.3 |  |  |  |  |  |  |  |  |
| n14 |  | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n25 |  | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| n26 | 1.1 | 1.1 | 1.2 | 1.4 | 5.2 | 6.1 | 6.5 |  |  |  |  |
| n28 | 1.1 | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  | 8.2 |  |  |
| n66 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 |  | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| n70 |  | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 1.2 | 1.2 | 1.4 | 6.4 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n13, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by [FWA CPE] NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n1, n3, n7, n25 | -4.5 |
| n34, n39, n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for[FWA CPE]/Vehicle/Industrial devices. |  |

For UE(s) equipped with 6 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,6R in Table 7.3.2-2aa for the applicable operating bands.

Table 7.3.2-2aa: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.41 -3.72 |
| n77, n78, n79, n104 | -3.01 -3.32 |
| NOTE 1: When 6 Rx operation is supported by handheld UE. NOTE 2: When 6 Rx operation is supported by [FWA CPE]. |  |

For two Rx antenna port XR UE(s) indicating UE capability supportOf2RxXR-r18, reference sensitivity for two Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRXR,2R in Table 7.3.2-2b for the applicable operating bands.

Table 7.3.2-2b: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-2aa and Table 7.3.2-2b shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

<Next change >

7.3G	Reference sensitivity for Tx Diversity

For UE supporting Tx diversity, the minimum requirements specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1d, Table 7.3.2-2, Table 7.3.2-2aa and Table 7.3.2-2a  shall be met with Tx diversity configuration described in clause 6.2G.1. For Tx diversity, the parameter PUMAX is defined in clause 6.2G.4 with the sum of the output power from all UE antenna connectors.

<Next change >

7.6.4	Narrow band blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The relative throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.4-1. For operating bands with an unpaired DL part (as noted in Table 5.2-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n68, n70, n71, n72, n74, n75, n76, n85, n87, n88, n100, n101, n106, n110 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm |  | -55 |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWCℎannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWCℎannel2+BWGB,CℎannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWCℎannel2+BWGB,CℎannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2aa and Table 7.3.2-2a for two, four, six and eight antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |

<End of change >