3GPP TSG-RAN4 Meeting # 116	R4-2510938

Bangalore, IN. 25th – 29th, 2025

# Source:	ZTE Corporation, Sanechips

# Title:	TP for PRD 01 to IMD MSD test point for single UL band with two CCs for two band DL CA

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

This TP for PRD 02[1] is to introduce IMD MSD test point for single UL with two CCs for three-band  based on the WF [2] and [3].

- Reference

- [1] Permanent Reference Document (PRD);Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, v0.3.0

- [2] R4-2507991, WF on UL CA_n41C IMD MSD test point, ZTE, Skyworks

- [3] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

- Text Proposal

----- Start of TP -----

### 5.5.2	Inter-band CA

#### 5.5.2.1	Two-band Inter-band Downlink CA

#### 5.5.2.1.1	One Uplink Band

#### 5.5.2.1.1.1	One Uplink CC

#### <texts are omitted>

#### 5.5.2.1.1.2	Two UL CCs

For 2 band inter-band combinations with simultaneous Tx/Rx operation with at least 1UL band with intra-band UL CA in the UL configuration, MSD related to IMDs of intra-band UL CA part (contiguous or non-contiguous) needs to be analysed.

Guideline on co-existence studies for one UL band with 2CC intra-band UL CA:

Table 5.5.2.1.1.2-1 summarizes the frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.5.2.1.1.2-1: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| nX fUL |  |  | Order | flow | fhigh |
| nY fDL |  |  |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  |  |  |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| Analysis | Editor’s Note: The analysis conclusion for the UL CA IMD shall be included here, even if no issues are identified. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

Guidelines for IMD MSD test points

When the two UL CCs, a.k.a  intra-band contiguous or non-contiguous CA configuration, is supported in the UL, it is decided to capture the two test points, which are: 1RB+1RB and full+full RB test points in the current IMD table in section 7.3A.5 of TS 38.101-1 used for the mandatory test point as long as some definition/explanation of the source of IMD is provided for the Full+Full case.

- The mandatory 1RB+1RB test point and optional full+full RB test points are captured in the current MSD table

- Include as a note for the optional full+full RB test points in IMD MSD table in the related band combination 	TP to TR and TS38.101-1.

- The IMD order captured in the table is the lowest IMD order of the two UL CCs that dominates the interfering power in the victim channel.

- This source of IMD are the same for the mandatory 1RB+1RB test point and the optional full+full RB test points.

- The optional full+full RB test point for the inter-band UL CA configurations with two contiguous CC in one band relies on the operator request.

Guidelines for IMD MSD order

The lowest order IMD is recommended as worst case to represent single band UL transmission with UL configured intra-band CA.

○   If the DL band may be affected by a mix of even and odd order IMD products, the MSD value of the lowest even and the lowest odd order IMD, if any, shall be defined in the specifications.

○   A footnote shall be attached to the DL band to indicate that MSD may occur for higher order IMD products, and these orders shall be specified in the footnote.

○   As an exception to this rule, a second MSD test point may be specified to capture the MSD that may occur due to the next highest odd order or due to the next highest even order IMD product. This exception ensures regional frequency restrictions are accounted for.

○   A footnote shall be attached to the UL band that is configured intra-band UL CA to distinguish the case of intra-band contiguous vs intra-band non-contiguous CA.

Guidelines for general requirements assumptions:

If the UL configuration is contiguous UL CA, the following assumptions can be used:

IMD3 can reach up to 200MHz OOB with -13dBm/MHz.

IMD5 can reach up to 400MHz OOB with -30dBm/MHz for NS_01 and -25dBm/MHz for NS_04.

IMD7 can reach up to 600MHz OOB with -45dBm/MHz for NS_01 and -35dBm/MHz for NS_04.

If the UL configuration is non-contiguous UL CA, the following assumptions can be used:

IMD 3 can reach up to 600MHz OOB with -30dBm/MHz.

IMD 5 can reach up to 1200MHz OOB with -45dBm/MHz.

The above levels are further attenuated by the UL band filter and any diplexer.

There is only an MSD or band protection issue with band that have simultaneous Tx/Rx with the UL band.

----- End of TP -----