3GPP TSG-RAN WG4 Meeting #116		R4-2510950

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.8

Source: 	CMCC

Title: 			 Discussion on Demodulation performance requirements for Device 1

Document for:	Discussion

- Introduction

In RAN plenary #107 meeting, the revised WID of Solutions for Ambient IoT (Internet of Things) in NR [1] has been approved. The RAN4 performance related scope of this WI is updated as follow:

- Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

Based on the time budget in RP-243326, the performance part of R19 A-IoT should start on August’s meeting. In this contribution, we provide our initial thinking about test scope and simulation parameters.

- Discussion

Test scope

PRDCH (Physical Reader-to-Device Channel) shall be introduced for Device 1 demodulation requirements.

Proposal 1: Introduce new PRDCH channel for Device 1 demodulation requirements.

Antenna configuration and Propagation condition

We propose 2T1R and TDLA30-10 for initial simulation.

Proposal 2: Use 2T1R and TDLA30-10 for initial simulation.

FRC

In RAN4 RF session, for reference sensitivity requirement, the WF for FRC is as follows;

- FFS for the FRC, consider following as starting point:

| SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- |
| PRB | 1 | 2 | 3 | 4 |
| TBS | 20 | 20 | 20 | 20 |
| CRC | 6 | 6 | 6 | 6 |
| M | 6 | 12 | 24 | 24 |

For demodulation, we prefer to keep it align with the reference sensitivity test configuration in RF session, and wait for the conclusion from RF first.

Proposal 3: Reuse the same FRC reference sensitivity simulation in RF session, and wait their conclusion.

Test metric

For now, at least SNR at 10% BLER shall be verified. Whether to introduce false alarm requirement, we prefer to keep it align with the reference sensitivity simulation in RF session, and wait for the conclusion from RF first.

Proposal 4: At least define SNR at 10% BLER requirement.

- Conclusion

Proposal 1: Introduce new PRDCH channel for Device 1 demodulation requirements.

Proposal 2: Use 2T1R and TDLA30-10 for initial simulation.

Proposal 3: Reuse the same FRC reference sensitivity simulation in RF session, and wait their conclusion.

Proposal 4: At least define SNR at 10% BLER requirement.

- Reference

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc.