3GPP TSG-RAN WG4 Meeting #116	R4-2510964

Bengaluru, IN, August 25th – 29th 2025

Agenda Item:	7.21.3.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF RX requirements for SBFD. Discussion on existing BS RF TX requirements for SBFD, new requirements for SBFD, and SBFD general aspects are presented in separate submissions [1-3].

# 2	Co-existence simulation for in-band blocking

2.1	Simulation assumption

## Issue 2-2: In-band blocking for FR1 WA BS - Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered - Issue 2-3: In-band blocking for FR2-1 WA BS - Way forward - Encourage companies to have offline discussion for the simulation assumptions before next meeting. - Issue 2-4: In-band blocking for FR1 Micro BS - Way forward - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point - Companies to check the blocking level in the range of [-20, -32] dBm

In RAN4#112bis and RAN4#113 meetings, it was agreed that for FR1 and FR2 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. However, the in-band blocking requirements for FR1 WA and MR, and FR2-1 WA still require further study. In RAN4#115, it was agreed that the in-band blocking requirements for FR1 WA and MR will be derived based on simulation results with 100% grid-shift, meanwhile, deployment restrictions shall be captured in the specification.

2.2	Discussion of simulation results for FR1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR1 scenarios: scenarios 1 (urban macro) and 5 (micro). The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858 [4] and are also summarized in the TR Annex.

2.2.1 FR1 Urban Macro

We consider a coexistence scenario where FR1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%. The grid-shift examples are depicted in Figure 2.2.1-1.

Figure 2.2.1-1: Grid-shift 10% and 100% between two UMa network deployments

Two different values of BS TRP are considered 1) 49 dBm, which is the default value in the assumptions of the SI; 2) 53 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.1-2.

Figure 2.2.1-2: FR1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.1-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases. In addition, it can be observed that when considering TRP 49 dBm and grid-shift 100% (best case assumptions), we are already around 20 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS.

- Considering TRP 53 dBm:

- When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

- Considering TRP 49 dBm:

- When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

- For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

It is important to note that deployment restrictions must be implemented when the in-band blocking requirement is defined based on simulation results with a 100% grid-shift.

In the WF [5], there are three options for further study, where Option 2 is the most straightforward for operators when deploying their networks. As to the relationship between distance and frequency bands, since the main application scenarios for SBFD is 4 GHz and 6 GHz, and not the low frequencies down to hundreds of MHz, we believe the impact is not significant. Specifically, a 100% grid-shift corresponds to 288 meters.

- Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive

- Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values

- Option 3: One note can be added to generally describe the deployment restriction.

- A 100% grid-shift corresponds to 288 meters. This indicates the need to introduce deployment restrictions among operators in both the RAN4 specification and real-world deployments.

Define in-band blocking interferer signal power to -20.4 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations. Meanwhile, deployment restriction of 288 meters among operators must be implemented and written into TS38.104.

In addition to the interfering signal power level, to define the in-band blocking requirement properly, the interfering signal frequency offset, and type of interfering signal also need to be discussed. Considering the agreed minimum channel bandwidth (CBW) for SBFD operation is no less than 20MHz for band n39 and no less than 40MHz otherwise, by following the existing values defined in TS 38.104 [6], ±7.5 MHz should be used when CBW is 20 MHz, and ±30 MHz should be used for all other CBW higher than 20 MHz. Since the dominant interference for in-band blocking for SBFD-capable BS comes from neighboring BSs rather than UEs, the interfering signal type should be NR signal instead of DFT-s-OFDM, and SCS can be 15 kHz when CBW is 20 MHz, and SCS should be 30 kHz for all other CBW higher than 20 MHz.

In the WF [5], companies are encouraged to investigate a new idea of interfering signal placement, which is configuration dependent and requires that the interfering signal needs to be placed next to the DL subband, and far away from the UL subband. To our understanding, it has never been discussed before in previous meetings in Rel-19 for the co-existence of SBFD with legacy TDD, and it will introduce additional deployment restrictions in frequency domain, in addition to the spatial deployment restrictions. Therefore, we could not support the idea.

- For SBFD in-band blocking requirement, configuration dependent placement of interfering signal is new and has never been discussed in previous meetings in Rel-19 for the co-existence of SBFD with legacy TDD, and it will introduce additional deployment restrictions in frequency domain, in addition to the spatial deployment restrictions.

In RAN4#115, it was also discussed that “For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered”, for which we believe clarification is necessary, since this is only applicable to OTA requirements, for conducted requirements, no additional REFSENS degradation of 0.7 dB is expected.

For OTA ACS/blocking requirement and test, SBFD-capable BS own transmitter is ON and the corresponding REFSENS degradation will be considered. For conducted ACS/blocking requirement and test, no additional REFSENS degradation of 0.7 dB is expected.

2.2.2 FR1 Urban Micro

We consider a coexistence scenario where FR1 urban micro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 5% and 10%. The grid-shift examples are depicted in Figure 2.2.2-1.

Figure 2.2.2-1: Grid-shift 10% and 100% between two UMi network deployments

Two different values of BS TRP are considered 1) 46 dBm, which is the default value in the assumptions of the SI; 2) 38 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.2-2.

Figure 2.2.2-2: FR1 UMi: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.2-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases.

- Considering TRP 46 dBm:

- When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

- Considering TRP 38 dBm:

- When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

It is important to note that deployment restrictions must be implemented when the in-band blocking requirement is defined based on simulation results with a 100% grid-shift. Specifically, a 100% grid-shift corresponds to 167 meters.

- A 100% grid-shift corresponds to 167 meters. This indicates the need to introduce deployment restrictions among operators in both the RAN4 specification and real-world deployments.

Define in-band blocking interferer signal power to -31.5 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations. Meanwhile, deployment restriction of 167 meters among operators must be implemented and written into TS38.104.

Table 2.2.1-1 summarize our proposals for defining in-band blocking requirements for FR1 WA and MR SBFD-capable BS.

Table 2.2.1-1: FR1 WA and MR SBFD-capable BS in-band blocking requirements

| SBFD CBW (MHz) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- |
| 20 | Wide Area SBFD-capable BS: -20.4 Medium Range BS: -31.5 | ±7.5 | 5 MHz NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wide Area SBFD-capable BS: -20.4 Medium Range BS: -31.5 | ±30 | 20 MHz NR signal 30 kHz SCS, 51 RBs |
| NOTE 1: To avoid blocking of an SBFD-capable BS, for Wide Area SBFD-capable BSs, an inter-site distance of at least 288 m between different operators’ BSs shall be implemented. For Medium Range SBFD-capable BSs, an inter-site distance of at least 167 m between different operators’ BSs shall be implemented. |  |  |  |

Use Table 2.2.1-1 as baseline for defining in-band blocking requirements for FR1 WA and MR SBFD-capable BS.

2.3	Discussion of simulation results for FR2-1

In this section we focus on FR2-1 urban macro scenario. The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858 [4] and are also summarized in the TR Annex.

2.3.1 FR2-1 Urban Macro

We consider a coexistence scenario where FR2-1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. Similarly to the other discussed scenarios, we are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%.

Two different values of BS TRP are considered 1) 30 dBm, which is the default value in the assumptions of the SI; 2) 40 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.3.1-1.

Figure 2.3.1-1: FR2-1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.3.1-1 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases. It can be observed that when considering TRP 40 dBm and grid-shift 5%, we are around 35 dB above the currently specified in-band blocking requirement (-58 dBm) for FR2-1 Wide Area BS.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

It is worth mentioning that depending on the hypothesis on the beamforming architecture, whether analog or digital/hybrid (no subarray is considered in simulation assumptions from SI Rel.18), the assumption of whether the Rx beamforming gain should be included or not in the in-band blocking interference received at the BS may change. The results in Figure 2.3.1-1 are obtained without including the Rx beamforming gain in the interference received from the adjacent channel.

The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

The above methodology considers the absolute value of the total wideband power received in the UL of the SBFD BS, from the adjacent channel, to derive the in-band blocking power requirement. In TS 38.104 [6], the OTA blocking requirement for FR2 is derived based on a relative value of the in-band blocking power, compared to the declared EISrefsens and considering 6 dB relaxation. The procedure to follow this methodology should be better clarified, by identifying the declared EISrefsens value to be considered in the evaluation. An approach based on simulations to derive the in-band blocking power/wanted signal, where the wanted signal is derived based on simulations, would lack of a fixed declared reference value, which is instead required by the specification.

To further discuss the methodology to derive the in-band blocking requirement and decide on if it should be based on an absolute value of the in-band blocking power, or on a relative value of the in-band blocking power compared to a declared EISrefsense + 6 dB, which should be further discussed and agreed.

Regarding the ratio-based approach adopted for FR2 OTA in-band blocking requirements, more background information can be found in TR 38.817-02 [7]. The blocking results are derived from co-existence simulations where UEs are the interfering source, and the blocking offset of 33 dB considers interference originating from UEs, rather than from neighbouring BSs, and is therefore not applicable to BS-to-BS blocking scenarios. Even if the EISrefsense could be declared in a range, when interference from neighbouring BSs is 35 dB higher, the ratio between interfering and wanted signal power could not be maintained at 33 dB.

10.5.3.3 FR2 BS type 2-O has a number of differences when considering the OTA blocking levels. - There are no conducted requirements, so simulation of conducted interferer power levels do not give a final OTA power level. - Beam forming is necessary in order to overcome the path loss. - A wide range of implementations with varying antenna maximum beam forming gain are envisaged. - Different beam forming architectures result in different statistical spread of interferer power at the active Rx input (i.e. the LNA). Traditionally the in-band blocking level has been analysis on a statistical basis based on the 99.99% probability of an interferer being possible. This has been used since UTRA where WCDMA modulation was susceptible to blocking and the entire system would be blocked if such an event occurred. The NR OFDMA scheme does not suffer so greatly due to a blocking event hence such a high probability is not required, probabilities between 99% and 99.9% have also been considered. In the past the wanted signal has not been considered when studying the interferer level, however with an OTA requirement and a beam forming system when considering a statistical worst case, it is important to consider both the wanted and the interfering signal. This is due to the fact that blocking arises when a low power wanted signal and a high-power blocking signal occur simultaneously (the likelihood of this occurring depends on instantaneous power control, scheduling and beam directions) Hence the difference between the wanted signal and the interferer is also important. Initially the probability of the interferer alone was simulated looking at the same scenarios identified in the co-existence simulation in 3GPP TR 38.803 [24]. The probability of the blocking signal level and also the probability of simultaneous low wanted signal and high blocking signal were both considered. Simulations of both of these aspects suggested that the blocking signal is typically 33dB above the reference sensitivity level. This result was consistent when considering different architectures. It was agreed that the specification for the interferer power level will be 33dB higher than the OTA REFSENS power level. As for FR2 there is only a single sensitivity requirement, a 6dB offset from OTA reference sensitivity is used for the wanted signal and a 33dB offset from reference sensitivity is used for the interferer.

# 3	Impact on BS RX requirements

ACS

Agreement from RAN4#112bis: - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. Content from TR 38.858: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Agreement from RAN4#115: ## Issue 2-1: ACS requirement - Way forward - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

Regarding ACS, there is one agreement reached in RAN4#112bis, as shown above. By looking at the content from TR 38.858 [4] regarding conducted ACS, we would like to further refine the agreement to the following wording, which is more precise:

The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

In the WF [8] agreed in RAN4#113, we need to further study if the interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

In our understanding, ACS and in-band blocking are derived based on different methods, and 9 dB offset may not be valid for SBFD.

- ACS is derived from co-existence simulations that evaluate ACIR versus the 5th percentile and mean throughput degradation, identify an ACIR and then decide a split of ACLR and ACS. The ACS is applied in the adjacent channel.

- The in-band blocking is derived from the 99.9th percentile RX power and is applied in the 2nd adjacent channel.

For SBFD, the ACS should be decided based on BS-BS co-existence simulations of mean throughput degradation and ACIR.

The requirement level of ACS should be derived from the simulation results obtained in SI, and RAN4 need to decide which scenario to use to extract the results.

The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

Derive ACS requirement level based on the simulation results using the same GS as for in-band blocking.

ICS

- Issue 2-6: In-channel selectivity - Way forward - Check whether existing ICS requirements can be reused

Considering ICS requirement for SBFD-capable BS during SBFD period and non-SBFD period are essentially the same, it should be OK to reuse the existing ICS requirement for SBFD.

Reuse existing ICS requirement level for SBFD-capable BS during SBFD period.

Receiver intermodulation

- Issue 2-5: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal - Way forward - Down-select to one of the below two options in the next meeting - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case - Interfering signal 1: CW - Interfering signal 2: TBD MHz NR - Power level: the final agreed in-band blocking level -9 dB

The RX intermodulation requirement for conventional TDD consists of a CW signal and a modulated signal that are located in frequency such that their intermodulation produce falls within the receiver frequency range. The RX intermodulation sets a minimum requirement on receiver linearity.

For SBFD, it is possible that a single signal can intermodulate in the receiver with the TX sub-band TX signal. It is necessary to supress such intermodulation sufficiently that the receiver is not desensitized. In this case, transmission in DL sub-band needs to be activated when testing RX IM, and the offsets are chosen in a way that intermodulation product between DL sub-band and interferer falls in the UL sub-band.

The scenarios for 2 input signals and 1 input signal + TX sub-band may need different levels of receiver linearity to be met, depending on the level of the TX sub-band signal in the receiver.

Since receiver linearity should meet both scenarios, in our view, the requirement should be set based on the tougher case.

As defined in Table 7.7.2-1 in TS 38.104 [6], the general intermodulation requirement for WA BS is -52 dBm, which is derived by the corresponding in-band blocking level (-43 dBm) – 9 dB. Hence, for scenario with two input signals, the interfering level for both signals should be equivalent to [the final agreed in-band blocking level] – 9 dB.

Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS +6 dB | -52 |  |
| Medium Range BS | PREFSENS +6 dB | -47 | See Table 7.7.2-2 |
| Local Area BS | PREFSENS +6 dB | -44 |  |
| NOTE 1: PREFSENS depends on the RAT and the BS class. For NR, PREFSENS depends also on the BS channel bandwidth, see clause 7.2. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |  |

Use FR1 WA as example for comparison of the two considered scenarios,

- considering the scenario of 2 inputs signals, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

- considering the scenario of 1 input signal + TX sub-band, the CW interfering level is [the final agreed in-band blocking level] – 9 dB, and the signal level of TX sub-band is -27 dBm, which equals to 53 dBm (TRP) – 80 dB (isolation).

Other factors that need to be considered, e.g., the signal level of TX sub-band is based on declaration, whereas RAN4 needs to define a RX IMD requirement with fixed interfering signals, and for SBFD DU pattern, whether the intermodulated product will fall into the UL subband depends on the placement of the CW interfering signal.

The requirement should be set based on the tougher case between the scenario of 2 inputs signals and the scenario of 1 input signal + TX sub-band.

Terminology

- Issue 2-7: One guidance for CR drafting - Way forward - Check and align the terminology for “SBFD slot”, “SBFD symbol”, “SBFD symbol/slot” or “SBFD period” to be used in specification drafting. - The decided terminology should be clarified in clause 3.

Regarding the terminology to be used in RAN4 specification, our concern is that the terminology “SBFD symbol” originates from RAN1, whereas RAN4 BS RF requirements are defined at the slot level. It is uncertain whether the terminology “SBFD symbol” can be reused directly in RAN4 specification.

Referring to the definition of “transmitter ON period”: time period during which the BS transmitter is transmitting data and/or reference symbols, which is widely used in transmitter RF requirements, here is our proposal on “SBFD period”: time period during which subband non-overlapping full duplex (SBFD) operation is active, allowing simultaneous transmission and reception in separate, non-overlapping subbands.

Use the terminology “SBFD period” for CR drafting of SBFD-capable BS RF requirements.

“SBFD period”: time period during which subband non-overlapping full duplex (SBFD) operation is active, allowing simultaneous transmission and reception in separate, non-overlapping subbands.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	Considering TRP 53 dBm:

When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

Observation 2	Considering TRP 49 dBm:

When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

Observation 3	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

Observation 4	For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 5	A 100% grid-shift corresponds to 288 meters. This indicates the need to introduce deployment restrictions among operators in both the RAN4 specification and real-world deployments.

Observation 6	For SBFD in-band blocking requirement, configuration dependent placement of interfering signal is new and has never been discussed in previous meetings in Rel-19 for the co-existence of SBFD with legacy TDD, and it will introduce additional deployment restrictions in frequency domain, in addition to the spatial deployment restrictions.

Observation 7	Considering TRP 46 dBm:

When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

Observation 8	Considering TRP 38 dBm:

When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

Observation 9	A 100% grid-shift corresponds to 167 meters. This indicates the need to introduce deployment restrictions among operators in both the RAN4 specification and real-world deployments.

Observation 10	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

Observation 11	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 12	For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define in-band blocking interferer signal power to -20.4 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations. Meanwhile, deployment restriction of 288 meters among operators must be implemented and written into TS38.104.

Proposal 2	For OTA ACS/blocking requirement and test, SBFD-capable BS own transmitter is ON and the corresponding REFSENS degradation will be considered. For conducted ACS/blocking requirement and test, no additional REFSENS degradation of 0.7 dB is expected.

Proposal 3	Define in-band blocking interferer signal power to -31.5 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations. Meanwhile, deployment restriction of 167 meters among operators must be implemented and written into TS38.104.

Proposal 4	Use Table 2.2.1-1 as baseline for defining in-band blocking requirements for FR1 WA and MR SBFD-capable BS.

Proposal 5	Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

Proposal 6	The methodology to derive the in-band blocking requirements should be clearly described, including the details about where the received in-band blocking power is measured and if the Rx beamforming gain is included or not in that measurement.

Proposal 7	To further discuss the methodology to derive the in-band blocking requirement and decide on if it should be based on an absolute value of the in-band blocking power, or on a relative value of the in-band blocking power compared to a declared EISrefsense + 6 dB, which should be further discussed and agreed.

Proposal 8	The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Proposal 9	The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD.

Proposal 10	Derive ACS requirement level based on the simulation results using the same GS as for in-band blocking.

Proposal 11	Reuse existing ICS requirement level for SBFD-capable BS during SBFD period.

Proposal 12	The requirement should be set based on the tougher case between the scenario of 2 inputs signals and the scenario of 1 input signal + TX sub-band.

Proposal 13	Use the terminology “SBFD period” for CR drafting of SBFD-capable BS RF requirements.

Proposal 14	“SBFD period”: time period during which subband non-overlapping full duplex (SBFD) operation is active, allowing simultaneous transmission and reception in separate, non-overlapping subbands.

# References

R4-2510963, Impact on SBFD BS RF TX requirements, Ericsson

R4-2510962, New SBFD BS RF requirements, Ericsson

R4-2510961, SBFD general aspects, Ericsson

TR 38.858, Study on evolution of NR duplex operation

R4-2508713, Way Forward for [115][307] NR_duplex_evo_BSRF

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

TR 38.817-02, General aspects for Base Station (BS) Radio Frequency (RF) for NR

R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF