3GPP TSG-RAN WG4 Meeting #116	R4-2510965

Bengaluru, IN, August 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | draftCR | rev | -- | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.104 on adding receiver in-channel requirements for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding receiver in-channel requirements for SBFD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 12.3.8, 12.3.9, 12.6.9 and 12.6.10 are introduced to capture conducted and radiated receiver in-channel selectivity and in-channel adjacent subband blocking requirements for SBFD-capable BS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RF requirements for SBFD-capable BS are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 12.3.8, 12.3.9, 12.6.9, 12.6.10 and Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------Start of change-----------------------------------------------------

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees. Applicable for FR1 only.

BeWφ,REFSENS	Beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees. Applicable for FR1 only.

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS Channel Bandwidth, expressed in MHz. BWChannel_CA = Fedge,high- Fedge,low.

BWChannel,block	Sub-block bandwidth, expressed in MHz. BWChannel,block = Fedge,block,high- Fedge,block,low.

BWConfig	Transmission bandwidth configuration, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or Aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWGB,low	The minimum guard band defined in clause 5.3.3 for lowest assigned component carrier

BWGB,high	The minimum guard band defined in clause 5.3.3 for highest assigned component carrier

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔfBE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the nominal -3 dB point of the measuring filter closest to the carrier frequency

ΔFGlobal	Global frequency raster granularity

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔFR2_REFSENS	Offset applied to the FR2 OTA REFSENS depending on the AoA

ΔminSENS	Difference between conducted reference sensitivity and minSENS

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster	Channel raster granularity

Δshift	Channel raster offset for SUL

EISminSENS	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster, given in table 5.4.2.2-1

FC,SBFD,UL	RF reference frequency on the channel raster for SBFD UL subband

FC,SBFD,DL	RF reference frequency on the channel raster for SBFD DL subband in DU or UD case

FC,SBFD,DL,1	RF reference frequency on the channel raster for the lower assigned SBFD DL subband in DUD case

FC,SBFD,DL,2	RF reference frequency on the channel raster for the upper assigned SBFD DL subband in DUD case

FC,block,high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block,low	Fc of the lowest transmitted/received carrier in a sub-block.

FC,low	The Fc of the lowest carrier, expressed in MHz.

FC,high	The Fc of the highest carrier, expressed in MHz.

FDL,low	The lowest frequency of the downlink operating band

FDL,high	The highest frequency of the downlink operating band

Fedge,low	The lower edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,low = FC,low - Foffset,low.

Fedge,high	The upper edge of Aggregated BS Channel Bandwidth, expressed in MHz. Fedge,high = FC,high + Foffset,high.

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset,low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset,high.

Ffilter	Filter centre frequency

Foffset,high	Frequency offset from FC,high to the upper Base Station RF Bandwidth edge, or from F C,block,high to the upper sub-block edge

Foffset,low	Frequency offset from FC,low to the lower Base Station RF Bandwidth edge, or from FC,block,low to the lower sub-block edge.

f_BE_offset	Separation between the edge of the last transmitted channel of the channels assigned for NR-U channel bandwidth and the centre of the measuring

f_offset	Separation between the channel edge frequency and the centre of the measuring

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FREF,shift	RF reference frequency for Supplementary Uplink (SUL) bands

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FUL,low	The lowest frequency of the uplink operating band

FUL,high	The highest frequency of the uplink operating band

GBChannel	Minimum guard band defined in clause 5.3.3

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

Physical resource block number

NRB	Transmission bandwidth configuration, expressed in resource blocks

NRB,high	Transmission bandwidth configuration for the highest assigned component carrier within a sub-block in CA

NRB,low	Transmission bandwidth configuration for the lowest assigned component carrier within a sub-block in CA

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

NRXU,counted	The number of active receiver units that are taken into account for conducted Rx spurious emission scaling, as calculated in clause 7.6.1

NRXU,countedpercell	The number of active receiver units that are taken into account for conducted RX spurious emissions scaling per cell, as calculated in clause 7.6.1

NTXU,counted	The number of active transmitter units as calculated in clause 6.1, that are taken into account for conducted TX output power limit in clause 6.2.1, and for unwanted TX emissions scaling

NTXU,countedpercell	The number of active transmitter units that are taken into account for conducted TX emissions scaling per cell, as calculated in clause 6.1

PEM,n50/n75,ind	Declared emission level for Band n50/n75; ind = a, b

PEM,n54,ind	Declared emission level for Band n54; ind = a, b, c, d, e, f

PEIRP,N	EIRP level for channel N

Pmax,c,AC	Maximum carrier output power measured per antenna connector

Pmax,c,cell	The maximum carrier output power per TAB connector TX min cell group

Pmax,c,TABC	The maximum carrier output power per TAB connector

Pmax,c,TRP	Maximum carrier TRP output power measured at the RIB(s), and corresponding to the declared rated carrier TRP output power (Prated,c,TRP)

Pmax,c,EIRP	The maximum carrier EIRP when the NR BS is configured at the maximum rated carrier output TRP (Prated,c,TRP)

Prated,c,AC	The rated carrier output power per antenna connector

Prated,c,cell	The rated carrier output power per TAB connector TX min cell group

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,sys	The sum of Prated,c,TABC for all TAB connectors for a single carrier

Prated,c,TABC	The rated carrier output power per TAB connector

Prated,c,TRP	Rated carrier TRP output power declared per RIB

Prated,t,AC	The rated total output power declared at the antenna connector

Prated,t,TABC	The rated total output power declared at TAB connector

Prated,t,TRP	Rated total TRP output power declared per RIB

PREFSENS	Conducted Reference Sensitivity power level

SCSlow	Sub-Carrier Spacing for the lowest assigned component carrier within a sub-block in CA

SCShigh	Sub-Carrier Spacing for the highest assigned component carrier within a sub-block in CA

SSREF	SS block reference frequency position

Wgap	Sub-block gap or Inter RF Bandwidth gap size

## 3.3	Abbreviations

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

### 12.3.8	In-channel selectivity for SBFD

The same requirements as specified in clause 7.8 shall be applied to SBFD-capable BS type 1-H during SBFD period, except for the note in table 7.8.2-1, table 7.8.2-1b,  table 7.8.2-2, table 7.8.2-2b, table 7.8.2-2d, table 7.8.2-3 and table 7.8.2-3d, which shall be adjusted to: Wanted and interfering signal are placed adjacently around FC,SBFD,UL, where the FC,SBFD,UL is defined for SBFD UL subband bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SBFD UL subband bandwidth of the wanted signal. For DU/UD case, place the wanted signal closer to the DL subband.

### 12.3.9	In-channel adjacent subband blocking for SBFD

#### 12.3.9.1	General

The in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency at the TAB connector for SBFD-capable BS type 1-H in the presence of an interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

#### 12.3.9.2	Minimum requirement for SBFD-capable BS type 1-H

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted signal and an interfering signal coupled to the SBFD-capable BS type 1-H TAB connector using the parameters specified in table 12.3.9.2-1. The reference measurement channel for the wanted signal is identified in table 7.2.2-1, 7.2.2-2 and 7.2.2-3 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal is further specified in annex D.

The in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Minimum conducted requirement is defined at the TAB connector for BS type 1-H.

Table 12.3.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-H

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) |
| --- | --- | --- | --- | --- | --- |
| 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | NR signal 30 kHz SCS NRB,SBFD,DL |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL2 | NR signal 30 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. |  |  |  |  |  |

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

### 12.6.9	OTA in-channel selectivity for SBFD

The same requirements as specified in clause 10.9 shall be applied to SBFD-capable BS type 1-O and BS type 2-O during SBFD period, except for the note in table 10.9.2-1, table 10.9.2-1a, table 10.9.2-2, table 10.9.2-2a, table 10.9.2-3 and table 10.9.2-3a, which shall be adjusted to: Wanted and interfering signal are placed adjacently around FC,SBFD,UL, where the FC,SBFD,UL is defined for SBFD UL subband bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SBFD UL subband bandwidth of the wanted signal. For DU/UD pattern, place the wanted signal closer to the DL subband.

### 12.6.10	OTA in-channel adjacent subband blocking for SBFD

#### 12.6.10.1	General

OTA in-channel adjacent subband blocking requirement is a measure of the receiver’s ability to receive an OTA wanted signal at its assigned channel frequency in the presence of an OTA interfering signal from other sector(s) and/or site(s) within the same operator’s network, which is an NR signal.

#### 12.6.10.2	Minimum requirement for SBFD-capable BS type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and:

-	when the wanted signal is based on EISREFSENS: the AoA of the incident wave of a received signal and the interfering signal are within the OTA REFSENS RoAoA.

-	when the wanted signal is based on EISminSENS: the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.10.2-1. The reference measurement channel for the wanted signal is identified in clause 10.3.2 for each SBFD UL subband bandwidth and are further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.10.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-O

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) |
| --- | --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25 ,30, 35, 40, 45, 50, 60, 70, 80, 90 | EISREFSENS + 6 dB | Wide Area BS: -28 – ΔOTAREFSENS Medium Range BS: -32 – ΔOTAREFSENS Local Area BS: -34 – ΔOTAREFSENS | DU or UD | FC,SBFD,DL | NR signal 30 kHz SCS NRB,SBFD,DL |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | NR signal 30 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
|  | EISminSENS + 6 dB | Wide Area BS: -28 – ΔminSENS Medium Range BS: -32 – ΔminSENS Local Area BS: -34 – ΔminSENS | DU or UD | FC,SBFD,DL | NR signal 30 kHz SCS NRB,SBFD,DL |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | NR signal 30 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
| NOTE 1: EISREFSENS and EISminSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. |  |  |  |  |  |

#### 12.6.10.3	Minimum requirement for SBFD-capable BS type 2-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction and are within the OTA REFSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with an OTA wanted signal and OTA interfering signal(s) using the parameters specified in table 12.6.10.3-1. The reference measurement channel for the wanted signal is further specified in annex A.1. The characteristic of the interfering signal(s) is further specified in annex D.

The OTA in-channel adjacent subband blocking requirement shall apply to SBFD UL subband bandwidth. The interfering signal(s) shall be positioned at the centre frequency of the SBFD DL subband bandwidth(s).

Table 12.6.10.3-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 2-O

| Frequency Range | SBFD UL subband bandwidth (MHz) | OTA wanted signal mean power (dBm) (Note 1) | OTA interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) |
| --- | --- | --- | --- | --- | --- | --- |
| FR2-1 | 50, 100, 200 | EISREFSENS + 6 dB | Wide Area BS: -47 – ΔOTAREFSENS Local Area BS: [TBC] – ΔOTAREFSENS | DU or UD | FC,SBFD,DL | NR signal 60 kHz SCS NRB,SBFD,DL |
|  |  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL,2 | NR signal 60 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
| NOTE 1: EISREFSENS depends on the SBFD BS UL subband bandwidth as specified in clause 12.6.3. |  |  |  |  |  |  |

------------------------------------------End of change------------------------------------------------------

------------------------------------------Start of change-----------------------------------------------------

Annex D (normative): 
Characteristics of the interfering signals

The interfering signal shall be a PUSCH containing data and DM-RS symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 6 of TS38.211 [9]. Mapping of PUSCH modulation to receiver requirement are specified in table D-1.

Table D-1: Modulation of the interfering signal

| Receiver requirement | Modulation |
| --- | --- |
| In-channel selectivity | 16QAM |
| Adjacent channel selectivity and narrow-band blocking | QPSK |
| General blocking | QPSK |
| Receiver intermodulation | QPSK |
| In-channel adjacent subband blocking for SBFD | QPSK |

------------------------------------------End of change------------------------------------------------------