3GPP TSG-RAN WG4 Meeting #116	R4-2510973

# Bengaluru, India, August 25th – 29th, 2025

Title:	Phase noise specification for CW

Source:	Qualcomm Incorporated

Agenda item:	7.22.3.3

Release:	Rel-19

WI: 	Ambient_IoT_Solutions-Core

Lead WG: 	RAN1

Document for:	Approval

# 1.	Introduction

RAN4#115 approved a WF [17] for BS and CW requirements. WF agrees in Issue 3-4 to define a phase noise for the implementation that has no phase noise cancellation capability. It is true that if reference is shared between the reader and CW source, there is correlation between phase noise and implementation can be such that the phase noise of the CW can be cancelled in the receiver. This paper focuses on discussing how the phase should be specified when there is no share reference.

# 2. 	Discussion on CW phase noise

WF [17] agreement

### Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

In the WF, the agreement is mildly confusing since it discusses the offset for the phase noise. This literary means that the phase noise spec is point frequency at a certain offset. While phase noise could be specified like this and especially for the offset for the R=1 and longest Tc, the BW is only +/- 7.5 kHz (15 kHz) and this would lie on the 1/f profile of the VCO and therefore tests RF HW of the CW source. However, this it is not a representative of the phase noise impact to system, i.e. receive band interference. Better spec would be integrated phase noise (IPN) where the noise is integrated over some bandwidth. In this case, the hardest spec would be the R=1 with longest Tc.

Observation 1: Phase noise need to be specified only for the closest D2R bandwidth.

It is also possible to specify either single sided or double sides phase noise.

In [14] we proposed to adopt the integrated phase noise requirement of -90 dBc/15 kHz that would guarantee only small system degradation. Some small offset should be set from where the measured phase noise is integrated. It is not practical to measure the noise from 0 Hz offset. Observing the D2R spectrum, the energy at the edge of the spectrum is very low and likely the 1/f noise is very high at the CW source.

Proposal 1: Ignore first 500 Hz in the phase noise requirement.

Similarly the SFO of 10 % should be included i.e. the phase noise should be integrated up to 7.5 kHz + 10% i.e. up to +/- 8.25 kHz.

From system simulations in the TR we can conclude that the 90 dBc is a good value for the requirement regardless of the integration bandwidth.

Proposal 2: For CW without shared reference, the phase noise of CW is 90 dBc for +/- 8.25 kHz.

In Appendix we provide specification language for this requirement and for clarity, we provide picture to show the regions.

Figure 1. Illustration of phase noise requirement for the CW source

# Conclusion

We discussed, observed and proposed:

Observation 1: Phase noise need to be specified only for the closest D2R bandwidth.

Proposal 1: Ignore first 500 Hz of the phase noise.

Proposal 2: For CW without shared reference, the phase noise of CW is 90 dBc for +/- 8.25 kHz.

# Reference

[1]	RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

[2]	R4-2420470 “WF on UE RF requirements for A-IoT”, Huawei, 3GPP TSG-RAN WG4 Meeting # 113, Orlando, United States,18 – 22 Nov., 2024

[3]	TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP

[4]	R4-2500402, “Discussion on Ambient IoT deployment scenario in Rel-19”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[5]	R4-2500401, “Discussion on Ambient IoT Testability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[6]	R4-2500490, “Ambient IoT device requirements”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114, Athens, Greece,  16th – 21st February, 2025

[7]	R4-2502858, “WF on requirements for system parameters and A-IOT device”,  CMCC, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[9]	R4-2502859, “WF on requirements for A-IoT BS and CW”, Huawei, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, 17– 21 February, 2025

[10]	R4-2505230, “WF on A-IOT device requirements”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[11]	R4-2505097, “WF on A-IoT BS and CW requirements”, Huawei, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[12]	R4-2505227, “(new) TS 38.191 Ambient IoT device radio transmission and reception”, CMCC, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[13]	R4-2505228, “3GPP_TS 38.194 skeleton”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #114bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[14]	R4-2503560, “Relevance of phase noise and LLS for SNR for REFSENS”, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[15]	R4-2503559, “Ambient IoT device backscattering and SFO error”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #114Bis, Wuhan, Hubei, China, 07th – 11th April, 2025

[16]	R4-2506163, “Phase noise specification for CW”, 3GPP TSG-RAN WG4 Meeting #115, St Julian’s, Malta, 19th – 25th May 2025

[17] R4-2508101, “WF on requirements for A-IoT BS and CW”, Moderator (Huawei), 3GPP TSG-RAN WG4 Meeting # 115, Malta, MT, 19th – 23rd May, 2025

# Appendix: Specification for CW phase noise

## x.1 Phase noise requirement for the CW source

CW phase noise is measured on both sides of the CW frequency and integrated over 500 Hz to 8.25 kHz on both sides (DSB). The integrated phase noise is less than the values in table x.1-1. In Table x.1-1, the shared reference unit is a CW source with external reference clock that is used by the reader node in the same topology. Which requirement applies is based on manufacturer declaration. The reference power is the power of the CW at the desired frequency.

Table x.1-1 Phase noise requirement for CW source

|  | IPn | Unit |
| --- | --- | --- |
| Standalone CW unit | -90 | dBc |
| Shared reference unit | -68 | dBc |