# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2510989

# Bangalore, India, 25th – 29th August 2025

# Title: 	Discussion on Testing issues for CSI compression two-sided models

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.18.3

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI compression.

# 2 Discussion

Topic 1: Two-sided CSI compression

Further simulations (not agreement): 
In previous meetings, RAN4 evaluated many cases to verify the feasibility of option 3 and option 4. It was not until the last meeting[2] that RAN4 concluded that option 3 track 2 and option 4a, option 4b were feasible according to the simulation results, which based on the testing process for two side model. RAN4 assumed that the test decoder which is based on the previous agreements and the mixed dataset. And other companies load this fixed model to train new encoder and test new encoder. Currently, the simulation results have been proved that the SGCS could be converged at least. Furthermore, companies mentioned that the current training process only focuses on the one-to-one joint training, and ignores the N-to-one joint training, which could be used to demonstrate the generalisability of the test decoder. Therefore, RAN4 achieved the following conclusion in the last meeting[2].

Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

From the above conclusion, RAN4 agreed to assume other models to do joint training, such as transformer, low complexity CNN, and MLP models. We strictly follow the conclusion and our simulation results could be found  in the following table. And we used n1=0.7 and n2=0.3 to calculate the weight loss value.

| Model | (1-to-1) Joint training | New encoder training with (Samsung’s) frozen decoder | (N-to-1) Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- |
| Encoder 1 (CNN) | 0.6842 | 0.6923 | 0.6901 | 0.6901 |
| Encoder 2 (Transformer) | 0.7061 | 0.6962 | 0.7036 | 0.7036 |
| Encoder 3 (low complexity CNN) | 0.6809 | 0.6882 | 0.6849 | 0.6849 |
| Encoder 4 (MLP) | 0.6797 | 0.6338 | 0.6681 | 0.6682 |

Based on our simulation results, it is clearly found that the transformer model has a better performance, both for one-to-one training and N-to-one training. And MLP has a worst performance in four models. On the other side, transformer, low complexity CNN, and MLP models have a low complexity of Flops compared to the CNN model, which has more Resnet blocks.

Observation 1. Based on our simulation results, transformer model has a better performance compared to other models and MLP has a worst performance. 
Observation 2. For N-to-1joint training and separate testing, the simulation results could be very similar with one-to-one joint training and testing.

Observation 3. It is feasible to train a common decoder and test it with different models of encoder.

- What should be captured or specified for each of the options

Agreements from RAN4#114bis - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) New agreement from RAN4#115: The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else

Currently, training dataset is assumed to training the test decoder that the results could be converged in RAN4 assumption. From this point of view, the training dataset should be captured in 3GPP to help TE vendors to train test decoder. Regarding how to capture the training dataset, such as channel, eigenvector or something else. From our understanding, RAN4 currently use eigenvectors as uniform format for CSI compression. And companies uploaded their training dataset into 3GPP as eigenvectors. Therefore, if there is no technical issues about the storage, it is better to assume eigenvectors as default format for training dataset to avoid some issues of data processing.

Propose 1. Propose to assume eigenvectors as default format for training dataset.

Option 4a and Option 4b:

- Dataset is specified for option 4a. - Assume reference decoder is captured. New agreements from RAN4#115: - Reference encoder should be captured - FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated Agreements from RAN4#114bis - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. New agreements from RAN4#115: - Specify dataset to train frozen encoder and decoder - TE vendor shall use the dataset to train the test decoder - This is the same dataset as used to train the frozen encoder - FFS whether the datasets consist of Eigenvector, channel or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

Regarding option 4a and option 4b, dataset based and reference encoder based different approaches in RAN4 assumption. According to the last meeting discussion, RAN4 agreed that the reference encoder should be captured. The remaining issues is about the label dataset format. Based on the current study, the automatic encoder structure is assumed for CSI compression, and eigenvector is assumed as encoder input in the previous meeting. From our point of view, the label dataset could be based on the eigenvectors at least.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI-CSI , The conclusions are:

Observation 1. Based on our simulation results, transformer model has a better performance compared to other models and MLP has a worst performance. 
Observation 2. For N-to-1joint training and separate testing, the simulation results could be very similar with one-to-one joint training and testing.

Observation 3. It is feasible to train a common decoder and test it with different models of encoder.

Propose 1. Propose to assume eigenvectors as default format for training dataset.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-250805, WF on AI/ML part 2, Ericsson.

# 5 Appendix

- System simulation assumption

Table 5-1. System simulation assumptions for training data

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | ideal channel estimation |

- AI model structure

- CNN model - Encoder 1

- Transformer model -Encoder 2

- CNN based lower-complexity encoder -Encoder 3

- MLP - Encoder 4