3GPP TSG-RAN WG4 Meeting #116	R4-2510990

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	7.13.4

Document for:	Discussion

Introduction

Discussions on Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC in RAN4#115 have led to agreements which were documented in the way forward (WF) [1]. They are summarized below [1]:

DMRS configuration - Ratio of PUSCH EPRE to DM-RS EPRE - Online agreement: - Ratio of PUSCH EPRE to DM-RS EPRE - -3dB with Number of DM-RS CDM group(s) without data = 2 for all cells Propagation condition - Online agreement: - Based on the previous agreement, the TDLC300-100 is for all interference cells and ‘TDLA30-10 for HetNet and TDLC300-100 for HomNet’ is for the serving cell. MMSE-IRC receiver assumption - estimation of interference covariance matrix - RAN4 agree that MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec - There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption. SNR requirement value deriving rule - Online agreement: - Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in R4-1904713

In addition to the above-mentioned agreements, [1] documented also the remaining open issues as below:

Sub-topic 2 Manufacturer declarations and applicability rules Whether to consider additional applicability rule on how the test is performed - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes. Sub-topic 3 SNR requirement value Collection of simulation assumptions - Companies are encouraged to bring both ideal and impairment simulation results for all the agreed test cases in the next meeting.

In RAN4#115 there were also 3 endorsed draft (big)CRs for 3 corresponding specifications, namely, TS 38.104 [5], TS 38.141-1 [3] and TS 38.141-2 [4]. While those draft (big)CRs have been endorsed, nonetheless, one may observe that they still need some improvements. A better alignment across (big)CRs to be agreed in RAN4#116 is needed to have consistency across the 3 specifications.

This contribution provides our views on the remaining open issues mentioned above as well as the issue concerning the consistency of the terms used in the draft (big)CRs endorsed in RAN4#115.

Discussion

The remaining issues can be classified into 2 subtopics, namely, Manufacturer declarations and applicability rules and SNR requirement value. In the following, our views on those remaining issues are provided in each of the respective subsections.

Manufacturer declarations and applicability rules

The first issue to be addressed in this subtopic is:

Whether to consider additional applicability rule on how the test is performed - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped - Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

Referring to LTE in [7], the “Note*: Not applicable for Local Area BS and Home BS.” inside the tables 8.2.6.1-1 to 8.2.6.1-6 implies that the HomNet requirements are not applicable for Local Area BS and Home BS. As agreed in RAN4#114bis [2], Rel-19 NR BS MMSE-IRC will follow such an applicability rule as in LTE on Local Area BS.

It is more probable that HomNet will use Wide Area BS or Medium Range BS rather than Local Area BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements. As Local Area BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to only pass HomNet requirements.

Wide Area BS and Medium Range BS are more probable to be deployed in HomNet rather than Local BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements.

As Local BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to only pass HomNet requirements.

As in LTE and already agreed in RAN4#114bis, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “For Wide Area BS and Medium Range BS, it is sufficient to pass the HomNet requirements”.

The second issue in this subtopic is the naming of the newly introduced clause in the endorsed draft bigCRs. In RAN4#114bis the corresponding agreement on Manufacturer declaration is as follows [2]:

Manufacturer declaration on MMSE-IRC receiver - Agreement: - Introduce new manufactory declaration for BS supporting enhanced performance requirements.

As highlighted above, the Manufacturer declaration is for BS supporting “enhanced performance requirements”.

In the two endorsed draft bigCRs for 38.141-1 [3] and 38141-2 [4], the agreement on the Manufacturer declaration is captured properly in the corresponding clause for Manufacturer declaration. The corresponding applicability rule is also using the same term as the Manufacturer declaration. For the sake of clarity, in the following, we use [3] as an example case. The Manufacturer declaration is written as follows [3]:

D.126; Enhanced performance requirements; Declaration of BS supporting enhanced performance requirements...

and the corresponding applicability rule is [3]:

##### 8.1.2.1.11 Applicability of enhanced performance requirements Unless otherwise stated, enhanced performance requirements tests shall apply only for BS declared to be supported (see D.126 in table 4.6-1).

The highlighted Manufacturer declaration and applicability rule are using the term “enhanced performance requirements” which is aligned with the agreement made in RAN4#114bis.

Nonetheless, the inconsistency appears in the naming of the newly introduced clause for the requirements. Let us take the example from [3]:

### 8.2.xx Performance requirements for PUSCH with inter-cell interference

The new clause is using the term “inter-cell interference” instead of “enhanced performance requirements”.

If we refer to the LTE specification [7], the title of the clause and the term used on the corresponding applicability rule are identical, as shown below:

### 8.2.6 Enhanced performance requirement type A in multipath fading propagation conditions with synchronous interference The enhanced performance requirement type A of PUSCH is determined by a minimum required throughput for a given SINR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ retransmissions. The purpose is to verify the demodulation performance when the wanted PUSCH signal in the serving cell is interfered by PUSCH of one or two dominant interferer(s) applying the interference model defined in clause B.6.2. The requirements apply to the BS supporting the enhanced performance requirements type A.

Hence, it is compact and easier to follow by the reader, as there is no additional term such as “inter-cell interference”.

If we refer to the draft CR for TS 38.104 endorsed in RAN4#115 [5], it is using the term “enhanced performance requirements” in the title of the new clause for the requirements and the applicability of the requirements, as shown below:

### 8.2.X Requirements for PUSCH with enhanced performance requirements. #### 11.2.1.X Requirements for PUSCH with enhanced performance requirements Apply the requirements defined in clause 8.2.X for 2Rx..

One may observe that the 3 endorsed draft (big)CRs for the 3 corresponding specifications are not aligned in the naming/title of the clause. Hence, an alignment across the (big)CRs for the 3 corresponding specifications to be agreed in RAN4#116 is needed.

The agreement in RAN4#114bis is to introduce a new manufacturer declaration for BS supporting enhanced performance requirements.

The endorsed draft bigCRs in RAN4#115 for TS 38.141-1 and TS 38.141-2 are using a term “inter-cel interference” in the title of the clause, while the Manufacturer declaration and the applicability rule are using the term “enhanced performance requirements”.

The draft CR endorsed in RAN4#115 for TS 38.104 is using the term “enhanced performance requirements” in the title of the clause and the applicability of the requirements.

LTE specification TS 36.104 consistently uses the term “enhanced performance requirements” in the title of the clause and in the applicability of the requirement.

RAN4 shall consistently use the term “enhanced performance requirements” in the Manufacturer declaration, Applicability rule and the title of the clause across all the corresponding specifications for Rel-19 NR BS MMSE-IRC.

SNR requirement value

To conclude the work of Rel-19 NR Demodulation Phase 5 BS MMSE-IRC, companies should provide the full set of simulation results for the ideal case and the case with impairments as suggested in the following [1]:

Collection of simulation assumptions - Companies are encouraged to bring both ideal and impairment simulation results for all the agreed test cases in the next meeting.

To address the issue above, in this meeting we provide our simulation results in [6].

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: Wide Area BS and Medium Range BS are more probable to be deployed in HomNet rather than Local BS. Hence, Wide Area BS and Medium Range BS need to be tested with HomNet requirements.

Observation 2: As Local BS needs only to pass HetNet requirements, so do Wide Area BS and Medium Range BS need to only pass HomNet requirements.

Proposal 1: As in LTE and already agreed in RAN4#114bis, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “For Wide Area BS and Medium Range BS, it is sufficient to pass the HomNet requirements”.

Observation 3: The agreement in RAN4#114bis is to introduce a new manufacturer declaration for BS supporting enhanced performance requirements.

Observation 4: The endorsed draft bigCRs in RAN4#115 for TS 38.141-1 and TS 38.141-2 are using a term “inter-cel interference” in the title of the clause, while the Manufacturer declaration and the applicability rule are using the term “enhanced performance requirements”.

Observation 5: The draft CR endorsed in RAN4#115 for TS 38.104 is using the term “enhanced performance requirements” in the title of the clause and the applicability of the requirements.

Observation 6: LTE specification TS 36.104 consistently uses the term “enhanced performance requirements” in the title of the clause and in the applicability of the requirement.

Proposal 2: RAN4 shall consistently use the term “enhanced performance requirements” in the Manufacturer declaration, Applicability rule and the title of the clause across all the corresponding specifications for Rel-19 NR BS MMSE-IRC.

- References

- R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#115, China Telecom, Malta, May 19-23, 2025

- R42504768, Way Forward for [114bis][321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#114, China Telcom, Wuhan, China, April 7-11, 2025

- R4-2508616, Draft Big CR on TS 38.141-1 for MMSE-IRC requirements, 3GPP RAN4#115, CMCC, Malta, May 19-23, 2025

- R4-2507622, Draft BigCR Rel-19 BS MMSE-IRC in 38.141-2, 3GPP RAN4#115, Nokia, Malta, May 19-23, 2025

- R4-2505382, Draft CR on 38.104: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#115, China Telecom, Malta, May 19-23, 2025

- R4-2510991, Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC, 3GPP RAN4#116, Nokia, Bengaluru, India, August 25-29, 2025

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024