3GPP TSG-RAN WG4 Meeting #116	R4-2510995

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia

Title:	On the Evolution of NR Duplex Operation - Sub-Band Full Duplex

Agenda item:	7.21.6

Document for:	Discussion

Introduction

The way forward of Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD) from RAN4#115 is documented in [1]. The following agreements have been made during RAN4#115 meeting [1]:

Issue 1-1-1 Workplan - agreement - Agree the work plan of demodulation performance part of R4-2507487 Issue 2-1-1 whether to introduce UE demodulation requirement related with inter-UE CLI for Rel-19 SBFD aware UEs - agreement - Not to introduce UE demodulation test related with inter-UE CLI for Rel-19 SBFD-aware UEs Issue 2-1-2 whether to introduce UE CSI reporting requirement for Rel-19 SBFD aware UEs - agreement - Not to introduce UE CSI reporting requirement for Rel-19 SBFD-aware UEs Issue 2-1-4 whether to introduce UE demodulation with repetition transmission - agreement - Do not introduce UE demodulation requirement with repetition transmission cases for SBFD-aware UE Issue 3-1-4 whether to introduce new PUCCH requirement to support SBFD operation - agreement - No new PUCCH requirement introduced. The legacy PUCCH test cases will be applied for SBFD capable-gNB Issue 3-1-5 whether to introduce new PRACH requirement to support SBFD operation - agreement - No new PRACH requirement introduced. The legacy PRACH test cases will be applied for SBFD capable-gNB

In addition to the above-mentioned agreements, [1] also captured open issues from the discussions in RAN4#115.

This contribution describes our views on those open issues for further discussions during RAN4#116 meeting.

Brief overview of SBFD

SBFD is being specified in Rel-19 as a new mode of operation in unpaired (TDD) spectrum. Different to non-SBFD (legacy) operation, in which the transmission within each symbol/slot can only be in one direction (i.e., either UL or DL), SBFD operation allows both UL and DL transmission to happen simultaneously in different frequency resources. Separate sets of RBs within SBFD symbols/slots are allocated for UL and DL transmissions. Figure 1 illustrates examples of two different arrangements of UL and DL resources for SBFD operation, namely, DU/UD (Fig. 1A) and DUD (Fig. 1B). As shown in Figure 1, SBFD symbols are symbols (or slots) which simultaneously allocate one UL sub-band and one (or two) DL sub-band(s) for SBFD operation.

Figure 1: SBFD resources in time and frequency domain

Different time-domain sequences were discussed by RAN1 in [4], e.g. DXXXU, XXXXX, or XXXXU, where D, U, and X refer to DL, UL and SBFD slots, respectively. Several RBs are used as guard-band between the UL and DL sub-bands to avoid cross-link interference. Furthermore, one or more OFDM symbols are used as guard-time (or -period) for the transition between different symbol/slot types.

Discussion

In this section, we provide our views on open issues concerning BS Demodulation and UE demodulation in two corresponding subsections.

BS Demodulation

General scope

The open issue on the general scope of BS demodulation is as follows [1]:

Sub-topic 3-1 General Scope lssue 3-1-1 whether to introduce BS demodulation requirement with considering interference modeling to account for inter-sub-band inter-gNB CLI for SBFD-capable gNB Issue 3-1-2 interference modeling for inter-sub-band inter-gNB CLI when introducing BS demodulation requirement for SBFD-capable gNB if considered interference modeling Issue 3-1-3 whether to introduce new PUSCH requirement with UL resource muting configured - Way forward - Introduce PUSCH demodulation requirement with UL resource muting configured with interference modelling - FFS on which type(s) of interference will be considered to specify the PUSCH requirement - gNB self-interference - co-site inter-sector interference - inter-site gNB-gNB co-channel inter-subband CLI - FFS on how to model interference - gNB self-interference - Option 1: modelling as additive white gaussian noise with fixed INR=-6dB targeting 1dB desense as starting pointing - Other options are not precluded - inter-sector and inter-site interference - Option 1: modelling as additive white gaussian noise with fixed INR = - X dB based on assumption of co-site isolation - Other options are not precluded - inter-site gNB-gNB co-channel inter-sub-band CLI

Interference modeling

Due to different transmitter and receiver imperfections, SBFD faces challenges from cross-link interference (CLI) between UEs and CLI between gNBs. Figure 2 illustrates the CLI types studied in RAN1 and RAN4, namely:

- gNB self-interference (1),

- gNB-to-gNB inter-sector (or intra-site) interference (2),

- gNB-to-gNB inter-site interference (3),

- UE-to-UE intra-cell interference (4), and

- UE-to-UE inter-cell interference (5).

Figure 2: CLI types present in SBFD

Concerning interference signals at the gNB, the modelling of CLI assumed during the Rel-18 Duplex Evolution SI is briefly described in the following, with primary focus on FR1 (more details (and on FR2-1) can be found in TR 38.858 [4]).

- gNB Self-interference modeling

The feasibility of SBFD was assessed by breaking down the self-interference into different components, such as transmit power, frequency isolation capability, spatial isolation, TX beam nulling and RF interference cancellation. The assessment was based on a framework for calculating the residual self-interference calculation (RSIC) under the assumption that the UL sensitivity degradation at the gNB receiver due to the self-interference is 1 dB. Candidate values of RSI were provided by RAN4 to RAN1, so that the desense target was also considered in the performance evaluation of SBFD.

- gNB Inter-sector interference

The objective of gNB inter-sector interference analysis was to identify the inter-sector spatial isolation values that would result in additional desensitization at the gNB receiver. RAN4 considered different factors in both the transmitter characteristics (e.g., tx power and beam nulling) and the BS deployment (e.g., antenna isolation and absorbent materials between the different sectors). gNB inter-sector spatial isolation values between 75 dB to 103 dB for FR1 were discussed, where the latter would also result in a negligible UL received desensitization (considering the effects of ACLR/ACS on top of the spatial isolation) even for high-power FR1 macro base station deployments.

- Inter-site gNB-to-gNB interference

The co-channel inter-sub-band inter-site interference analysis considered the unwanted emission impact on the receiver selectivity. The same transmitter leakage and receiver impairment model as for investigating the gNB self-interference was used. However, the antenna isolation was replaced with the inter-site isolation which can be derived using 3GPP propagation models (based on TR 38.901 [5]). The Tx leakage baseline studied was the gNB adjacent channel leakage ration (ACLR), and the receiver impairment was the gNB adjacent channel selectivity (ACS).

UL Resource muting

RAN1 introduced the concept of UL resource muting for handling the gNB-to-gNB CLI [3]. A UE transmitting PUSCH with UL resource muting configured shall mute, assuming a comb-2 pattern, the resource elements overlapping with the up to two UL muting symbols configured within a slot. The UE applies the muting to the whole PUSCH bandwidth and with 3 dB power boosting over the active REs within the UL muting symbol(s). UL muting is not performed if an UL muting symbol collides with the UL DMRS of the PUSCH.

PUSCH requirements

Compared to the legacy TDD operation, the PUSCH demodulation in an SBFD-capable gNB is affected by its own self-interference as well as the gNB-to-gNB CLI (cf. CLI types 1, 2, 3 in Figure 2). Concerning the gNB self-interference, on how to accommodate such an interference into the link-level simulation is not yet clear. As the transmit antennas and the receive antennas are close to each other, propagation conditions as in [5] may not be used as the propagation condition of the self-interference signal. Nonetheless, referring to [4] self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

Concerning the inter-site gNB-to-gNB interference, as this is appearing in SBFD multi-cell scenarios, RAN4 needs to properly model it. It can be modelled similarly as RAN4 has modeled the interference in UL inter-cell interference scenario of LTE (cf. TR 36.884 [7]). In TR 36.884 [7], the UL inter-cell interference model of LTE is obtained from system-level simulations. Such extensive system-level simulations are needed to quantify the so-called Dominant Interference Proportion (DIP), which would then be used in the link-level simulation to define the corresponding demodulation requirements. Different to TR 36.884 for LTE, in SBFD case, the system-level simulation needs to consider the other gNBs from other cells as sources of interference instead of the UEs from other cells as in the LTE case. Moreover, different allocations (e.g., DUD vs DU/UD, cf. Figure 1) in SBFD symbols/slots and different SBFD BW sizes among different cells should be considered.

RAN4 first needs to discuss whether such extensive system-level simulations to properly model gNB-to-gNB CLI for link-level simulations can be performed in due time. If system-level simulation is not possible to be conducted during Rel-19 SBFD work, RAN4 may need to exclude the gNB-to-gNB CLI. Hence, only an isolated SBFD gNB would be considered, which is disturbed by only AWGN noise (with enhanced noise, if the self-interference is agreed to be modelled as additive noise). If such a scenario is considered, consequently, the WID needs to be updated and agreed in RANP to mention that the demodulation performance requirements will consider only an isolated gNB without gNB-to-gNB CLI.

With UL muting, the UE shall apply 3 dB power boosting (over the active REs within UL muting symbols) and rate-matching around the muted REs.

In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Referring to TR 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

To properly model the gNB-to-gNB interference signals, system-level simulations are needed to obtain the corresponding Dominant Interference Proportion (DIP) values for SBFD. On how the system-level simulations for SBFD shall be conducted, TR 36.884 can be used as a reference.

If system-level simulations to obtain DIP for SBFD cannot be performed by companies, the scenario RAN4 may consider is an isolated SBFD gNB, where the intended SBFD gNB is only affected by AWGN (with an enhanced noise due to the self-interference, if it is modelled as additive noise).

RAN4 to first discuss the possibility of conducting system-level simulations to properly model the gNB-to-gNB CLI in a similar methodology as for modelling the Dominant Interference Proportion (DIP) in LTE in TR 36.884 with necessary adaptations.

If companies cannot perform system-level simulation to obtain the proper gNB-to-gNB interference signals for SBFD, RAN4 needs to discuss whether considering only AWGN (and possibly adding noise to represent gNB self-interference) without having gNB-to-gNB CLI is sufficient for defining Rel-19 SBFD PUSCH demodulation performance requirements.

If RAN4 agrees to define PUSCH requirements for SBFD with isolated gNB (i.e., with only AWGN), the WID shall be updated and agreed in RANP to state that the demodulation performance requirements for Rel-19 SBFD will only consider an isolated SBFD gNB with only AWGN and without gNB-to-gNB CLI.

Test setup for demodulation requirements

The open issues concerning BS demodulation on the test setup for demodulation requirements (if introduced) are as below [1]:

Sub-topic 3-2 Test setup for demodulation requirement if introduced Issue 3-2-1 Frequency Range - Way forward - Option 1 - FR1 and FR2-1 - Other options are not precluded Issue 3-2-2 Channel Bandwidth - Way forward - Option 1 - prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands - if additional bandwidth is required, 100MHz for FR1 - Option 2 - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS - Other options are not precluded Issue 3-2-3 TDD pattern - Way forward - Option 1 - 30 kHz SCS: XXXXXXXXUU - 120 kHz SCS: XXXXU - Other options are not precluded Issue 3-2-4 Waveform - Way forward - Option 1 - Both DFT-s-OFDM and CP-OFDM - Other options are not precluded

Frequency Range and Channel Bandwidth

Concerning Issues 3-2-1 and 3-2-2, we have already addressed them in RAN4#115 in [6]. RAN4 BS RF have agreed on the operating bands for SBFD as listed in R4-2502310 [2] for NR TDD FR1 and FR2-1 bands. For FR1, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39. For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

SBFD over narrow carrier bandwidths (e.g. 40 MHz for FR1) have the challenge of larger overhead as the number of RBs used for the guard bands are relatively large compared to the number of RBs that are used for UL and DL transmissions. At least for FR1, RAN4 should discuss whether to keep existing carrier bandwidth assumption, e.g. 40 MHz, or whether to increase it to e.g. 100 MHz given that larger bandwidths are more attractive for SBFD operation.

SBFD is specified for NR TDD bands in both FR1 and FR2-1.

For FR1 the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.

For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

For FR1, current channel bandwidth assumption for UE/BS demodulation is 40 MHz, while SBFD is especially intended for large carrier bandwidths, e.g. 100 MHz, due to lower relative overhead of the guard-bands.

If RAN4 agrees to define PUSCH requirements, RAN4 shall prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands for SBFD demodulation performance requirements. If additional bandwidth is required, RAN4 to consider 100 MHz for FR1.

TDD pattern

As mentioned in Section 1, different time-domain sequences were discussed by RAN1 in [4], e.g. DXXXU, XXXXX, or XXXXU, where D, U, and X refer to DL, UL and SBFD slots, respectively. As Rel-19 is the first release for SBFD, adaptation from legacy TDD pattern which contains DL and UL slots should be considered as a starting point.

If RAN4 agrees to define PUSCH requirements, TDD pattern of DXXXU shall be considered.

Waveform

As the intention of SBFD is to boost UL performance, the increase in data rate can be more significant in the case of CP-OFDM than DFT-s-OFDM. Hence, RAN4 shall prioritize CP-OFDM.

If RAN4 agrees to define PUSCH requirements, CP-OFDM shall be prioritized for Rel-19 SBFD.

Inclusion in Specification

The open issue concerning BS demodulation on this subtopic is [1]:

Sub-topic 3-3 Inclusion in Specification Issue 3-3-1 Specification Impact - Way forward - Option 1: New sub-clause for demodulation requirements under same chapter with RF requirements can be considered. If RAN4 agree Not to introduce new BS demodulation requirements for SBFD, only applicability rule can be captured. - Other options are not precluded

As the discussions on the PUSCH requirements is still ongoing, this specification impact can be treated as FFS.

RAN4 to keep the specification impact discussion as FFS.

UE Demodulation

General Scope

The open issue in this subtopic is as follows:

Sub-topic 2-1 General Scope Issue 2-1-3 whether to introduce UE demodulation with SBFD carrier (with DL sub-band and UL sub-suband configured) - Way forward - Option 1: Define limited number of additional SBFD cases - Other options are not precluded

UE-to-UE CLI exists because of the transmitter leakage and receiver selectivity. RAN1 agreed to model the UE-to-UE interference as a combination of UE-to-UE spatial isolation (based on TR 38.901 [4] propagation models), in-band emission (IBE) model (based on 38.101-1 [5]) for transmitter leakage, and a fixed selectivity value of 33 and 23 dB for FR1 and FR2-1, respectively.

UEs operating in SBFD networks will be affected by UE-to-UE CLI from nearby transmitting UEs (from the same or different cell as illustrated in Figure 2 (cf., CLI (4) and (5))). More specifically, a UE transmitting in UL direction would generate interference to another UE receiving e.g. a PDSCH channel or performing measurements for CSI reporting.

Considering the effects of timing advance on the UL transmission, UE-to-UE CLI may be quite different to the traditional inter-cell DL cochannel interference present in legacy TDD networks. In addition to the effects of UE-to-UE CLI on UE DL reception and measurements, RAN1 has specified various enhancements to account for the two disjoint DL sub-bands in case SBFD DUD operation. For UE demodulation, some of the most relevant ones are as follows:

- For PDSCH FDRA Type 0 allocation (bitmap-based), the part of a RBG that partially overlaps with the DL sub-band, only the RBs within the DL sub-band are used by the UE for PDSCH reception.

- For PDSCH FDRA Type 1 allocation (start RB and RB length), only the RBs within the DL sub-band are used by the UE for PDSCH reception.

- For CSI-RS, only CSI-RS frequency resources within DL sub-band are valid for the UE. For CSI reporting, the following has been additionally specified by RAN1:

  - For a CSI reporting sub-band which overlaps with at least one PRB within DL PRBs and one or more PRBs outside DL PRBs, the CSI reporting sub-band includes PRB(s) within DL PRBs only.

  - For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Figure 3: RAN1 specified enhancements for SBFD DL reception and CSI-RS measurement across disjoint DL sub-bands

While UE-to-UE CLI will impact the DL performance of SBFD UEs, RAN4 has agreed in RAN4#115 to not consider UE-to-UE CLI in SBFD demodulation performance and CSI reporting requirements.

RAN1 has specified various enhancements to account for the two disjoint DL sub-bands in case SBFD DUD operation for both PDSCH FDRA Type 0 and PDSCH FDRA Type 1.

If RAN4 specifies PDSCH demodulation requirements for SBFD, DUD operation with both PDSCH FDRA Type 0 and Type 1 shall be considered.

Test setup if demodulation requirement if introduced

The open issues in this subtopic are [1]:

Sub-topic 2-2 Test setup if demodulation requirement if introduced Issue 2-2-1 Number of DL sub-band if introduced UE demodulation - Way forward - Option 1 - Consider one DL sub-band and one UL sub-band case as high priority. FFS whether to consider two DL sub-band and one UL sub-band case based on RAN1 agreements - Other options are not precluded Issue 2-2-2 Number of Case related TDD pattern - Way forward - Option 1 - Define limited number of additional SBFD cases - Other options are not precluded Issue 2-2-3 TDD pattern - Way forward - Option 1 - 30khz: XXXXXXXXUU - 120KHz: XXXXU - Other options are not precluded Issue 2-2-4 Channel bandwidth & SCS - Way forward: - Option 1 - FR1 with 30KHz, 40MHz DL sub-band + 20MHz UL sub-band for 30kHz SCS - FR2-1 with 120KH, 100MHz DL sub-band + 100MHz UL sub-band for 120kHz SCS - Other options are not precluded Issue 2-2-5 MCS & Rank for PDSCH requirements - Way forward: - Option 1 - MCS 13 and rank 2 - Other options are not precluded Issue 2-2-6 Propagation condition for PDSCH requirements - Way forward: - Option 1 - TDLC300-100 - Other options are not precluded Issue 2-2-7 HARQ timeline - Way forward: - Option 1 - use the HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS and XXXXU TDD pattern for 60kHz SCS as above Figure 2.1.2-1 and Figure 2.1.2-2 respectively Figure 2.1.2-1 HARQ timeline for XXXXXXXXUU TDD pattern for 30kHz SCS Figure 2.1.2-2 HARQ timeline for XXXXU TDD pattern for 120kHz SCS - Other options are not precluded Issue 2-2-8 number of HARQ processes - Way forward : - Option 1 - Using 4 HARQ processes for both TDD pattern - Other options are not precluded Issue 2-2-9 Other parameters - Way forward : - Option 1 - For PDSCH demodulation performance requirements, use the parameters as above Table 2.1.2-1 Parameter; Unit; Value. Duplex mode; TDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1. - Other options are not precluded

The details of the test parameters for UE demodulation can be kept as FFS subject to agreements in PDSCH demodulation requirements.

The detail of test parameters for UE demodulation performance can be kept as FFS subject to agreements on having the PDSCH requirements.

Conclusion

In this contribution, we provide our views on demodulation performance requirements for Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD). The following Observations and Proposals were made:

Observation 1: With UL muting, the UE shall apply 3 dB power boosting (over the active REs within UL muting symbols) and rate-matching around the muted REs.

Observation 2: In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Observation 3: Referring to TR 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

Observation 4: To properly model the gNB-to-gNB interference signals, system-level simulations are needed to obtain the corresponding Dominant Interference Proportion (DIP) values for SBFD. On how the system-level simulations for SBFD shall be conducted, TR 36.884 can be used as a reference.

Observation 5: If system-level simulations to obtain DIP for SBFD cannot be performed by companies, the scenario RAN4 may consider is an isolated SBFD gNB, where the intended SBFD gNB is only affected by AWGN (with an enhanced noise due to the self-interference, if it is modelled as additive noise).

Proposal 1: RAN4 to first discuss the possibility of conducting system-level simulations to properly model the gNB-to-gNB CLI in a similar methodology as for modelling the Dominant Interference Proportion (DIP) in LTE in TR 36.884 with necessary adaptations.

Proposal 2: If companies cannot perform system-level simulation to obtain the proper gNB-to-gNB interference signals for SBFD, RAN4 needs to discuss whether considering only AWGN (and possibly adding noise to represent gNB self-interference) without having gNB-to-gNB CLI is sufficient for defining Rel-19 SBFD PUSCH demodulation performance requirements.

Proposal 3: If RAN4 agrees to define PUSCH requirements for SBFD with isolated gNB (i.e., with only AWGN), the WID shall be updated and agreed in RANP to state that the demodulation performance requirements for Rel-19 SBFD will only consider an isolated SBFD gNB with only AWGN and without gNB-to-gNB CLI.

Observation 6: SBFD is specified for NR TDD bands in both FR1 and FR2-1.

Observation 7: For FR1 the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.

Observation 8: For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

Observation 9: For FR1, current channel bandwidth assumption for UE/BS demodulation is 40 MHz, while SBFD is especially intended for large carrier bandwidths, e.g. 100 MHz, due to lower relative overhead of the guard-bands.

Proposal 4: If RAN4 agrees to define PUSCH requirements, RAN4 shall prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands for SBFD demodulation performance requirements. If additional bandwidth is required, RAN4 to consider 100 MHz for FR1.

Proposal 5: If RAN4 agrees to define PUSCH requirements, TDD pattern of DXXXU shall be considered.

Proposal 6: If RAN4 agrees to define PUSCH requirements, CP-OFDM shall be prioritized for Rel-19 SBFD.

Proposal 7: RAN4 to keep the specification impact discussion as FFS.

Observation 10: While UE-to-UE CLI will impact the DL performance of SBFD UEs, RAN4 has agreed in RAN4#115 to not consider UE-to-UE CLI in SBFD demodulation performance and CSI reporting requirements.

Observation 11: RAN1 has specified various enhancements to account for the two disjoint DL sub-bands in case SBFD DUD operation for both PDSCH FDRA Type 0 and PDSCH FDRA Type 1.

Proposal 8: If RAN4 specifies PDSCH demodulation requirements for SBFD, DUD operation with both PDSCH FDRA Type 0 and Type 1 shall be considered.

Proposal 9: The detail of test parameters for UE demodulation performance can be kept as FFS subject to agreements on having the PDSCH requirements.

- References

- R4-2508723, Way Forward for [115][330] NR_duplex_evo_demod, Samsung, 3GPP TSG RAN4#115, Malta, May 19 – 23, 2025

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, 3GPP TSG RAN4#114, Athens, GR, February 17 – 21, 2025

- R1-2504980, Endorsed CR on TS 38.211 for the introduction of sub-band full duplex (SBFD), Ericsson, 3GPP TSG RAN1#121, Malta, May 19 – 23, 2025

- 3GPP TR 38.858, Study on Evolution of NR Duplex Operation, V18.2.0, 12-2024

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0, 03-2024

- R4-2507626, On the Evolution of NR Duplex Operation - Sub-Band Full Duplex, Nokia, 3GPP TSG RAN4#115, Malta, May 19 – 23, 2025

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, V13.1.0, 09-2016