3GPP TSG-RAN WG4 Meeting #116                                           R4-2510999

Bangalore, India, 25th – 29th August, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on UE RF testability issues for Rel-19 LP-WUS

Agenda Item:	7.24.3.4

Document for:	Approval

Introduction

Some progress was made in previous RAN4 meeting on testability issues for LP-WUS [1].

Issue 4-1-3: skipping rule for LP-WUR different mode

Agreements:

  - For idle-mode only or connected-mode only LP-WUR, the test should be done accordingly.

  - FFS For LP-WUR support both..

In this contribution, we will focus on the FFS issue.

Discussion

Test under different RRC states

From the study in RAN1, LP-WUS can operate in both RRC_IDLE/INACTIVE and RRC_CONNECTED modes. It is noted that the process for LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode are different. In addition, supporting IDLE/INACTIVE mode and CONNECTED mode is based on UE capabilities, which means a UE supporting LP-WUS operation in IDLE/INACTIVE mode does not necessarily to support operation in CONNECTED mode, and vice versa.

The UE capabilities can be found in Annex, which are cited from RAN1 feature list for LP-WUS [2].

Since the RF test is not the functionality test, ideally the test under one mode is sufficiently to verify the RF capability, and most likely UE would support IDLE/INACTIVE mode for better utilizing the WUS mechanism in terms of power saving. However, we are not so sure that RAN1 could provide confirmable feedback that all UE supporting LP-WUS should support the feature under IDLE/INACTIVE mode. Therefore, to cover the completed operation scenarios, test cases should be designed for both IDLE/INACTIVE mode and CONNECTED mode.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

Conclusion

In this paper we provide our consideration on the framework of LP-WUS with following observations and proposals.

Observation 1: LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode depends on UE capability, which means UE may not support both modes.

Proposal 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LR, which mode is selected for test should be up to UE declaration.

- References

[1] R4-2507936, WF on NR LP-WUS UE requirements, vivo

[2] R4-2505303, LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis

- Annex

RAN1 feature list for LP-WUS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. Support of LP-WUS frequency resource within active DL BWP FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |