3GPP TSG-RAN WG4 Meeting # 116	R4-2511010

# Bengaluru, India, Aug 25th – 29th, 2025

Title: 	Clarification on the TDD n50 asymmetric channel bandwidth

Source: 	Huawei, HiSilicon

Agenda item:	4.2.1

Document for:	Discussion/Decision

- Background

During RAN4#95-e, CR [2] on asymmetric channel bandwidth operation for FR1 TDD band was agreed with the clarification in discussion paper [1]. After checking the three notes added by CR [2], and with the understanding on the centre frequency and BWP id restriction defined in TS 38.331 and TS 38.213, we think that further clarification is needed in RAN4 to seek for common understanding.

- Discussion

As per CR [2], the following three notes for TDD asymmetric UL/DL channel operation are added in Table 5.3.6-2 of TS 38.101-1:

Table 5.3.6-2: TDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n50 | 60 | 80 | 0 |
| NOTE 1: Both centre frequency and BWP-ID shall match between DL and UL carriers as defined in TS 38.331 [7] cl. 6.3.2 and TS 38.213 [8] clause 12. NOTE 2: In a case a UE is configured with a full width of BWP within both UL/ DL channels, the centre frequency of UL/ DL channels shall be same. NOTE 3: A position of Point A is common between UL and DL carriers as defined in TS 38.331 [7] cl. 6.3.2. |  |  |  |

The max transmission bandwidth configuration for 60MHz and 80MHz as defined in Table 5.3.2-1 of TS 38.101-1 are 162 RBs and 217 RBs which is even and odd number of PRBs respectively.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |

As per TS 38.331 section 6.3.2 and TS 38.211, the offsetToCarrier is the Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier); the locationAndBandwidth is the frequency domain location and bandwidth of this BWP, from the derived RBstart can get the offset between the BWP and the carrier in number of PRBs. Related configurations are extracted from TS 38.331 section 6.3.2 as below:

FrequencyInfoDL-SIB ::= SEQUENCE { frequencyBandList MultiFrequencyBandListNR-SIB, offsetToPointA INTEGER (0..2199), scs-SpecificCarrierList SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier } SCS-SpecificCarrier ::= SEQUENCE { offsetToCarrier INTEGER (0..2199), subcarrierSpacing SubcarrierSpacing, carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks), ..., [[ txDirectCurrentLocation INTEGER (0..4095) OPTIONAL -- Need S ]] } BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED { extended } OPTIONAL -- Need R }

The relative position relationship among Point A, carrier and BWP can be shown in Figure 1 below:

Figure 1: Relative position relationship among Point A, carrier and BWP

As per the clarification in [1] by referring to TS 38.331 and TS 38.213 and the NOTE 1 and NOTE 3 added in Table Table 5.3.6-2 of TS 38.101-1[3]: 1)the center frequency and BWP-ID shall be same between UL and DL BWP for asymmetric channel bandwidth operation; 2) the common Point A position should be configured for UL and DL carrier. But for n50 with asymmetric full bandwidth operation between UL and DL channel with max 162 RBs and 217 RBs configured which are even and odd PRBs respectively for UL and DL, it is impossible to configure the same center frequency for UL and DL with full width of BWP and offset in integer number of PRBs between Point A and BWP configured as shown in Figure 2 below:

Figure 2: Different center frequency for n50 UL and DL channel with full width of BWP configured

Observation 1: It is impossible to configure the same center frequency between UL and DL BWP with full width of BWP and offset in integer number of PRBs between BWP and Point A configured for TDD n50 asymmetric CBWs.

To ensure the same center frequency between UL and DL BWP, either even or odd number PRBs can be configured for both UL and DL BWP, i.e. one less RPB can be configured for either DL or UL BWP as shown in Figure 3 and Figure 4 respectively compared to full width BWP.

Figure 3: Scheme 1: the same center frequency for n50 UL and DL channel with even number PRBs for both UL and DL BWP

Figure 4: Scheme 2: the same center frequency for n50 UL and DL channel with odd number PRBs for both UL and DL BWP

From the Figure 3 and Figure 4, we can get the following observation:

Observation 2: Either even or odd number of PRBs for both UL or DL BWP can be configured to ensure the same centre frequency between UL and DL BWP.

To ensure the higher DL throughput, we prefer Scheme 2 with one less PRB configured for UL BWP. Specifically, the NRB number for n50 UL channel under asymmetric CBW condition could be revised from 162 RB to 161RB as an exception.

Proposal: Update the clarification for TDD asymmetric CBWs:

- Add a general statement in section 5.3.6 for TDD: Either both odd or even number of PRBs should be configured for carrier bandwidth and UL/DL BWP.

- Add NOTE 4 to make it clear that NRB for 60MHz asymmetric UL channel bandwidth is 161 instead the NRB number in Table 5.3.2-1, which has no impact on the NRB for symmetric UL/DL channel bandwidth.

- Conclusion

As per our analysis shown above, our observations and proposals are summarized as following:

Observation 1: It is impossible to configure the same center frequency between UL and DL BWP with full width of BWP and offset in integer number of PRBs between BWP and Point A configured for TDD n50 asymmetric CBWs.

Observation 2: Either even or odd number of PRBs for both UL or DL BWP can be configured to ensure the same centre frequency between UL and DL BWP.

Proposal: Update the clarification for TDD asymmetric CBWs:

- Add a general statement in section 5.3.6 for TDD: Either both odd or even number of PRBs should be configured for carrier bandwidth and UL/DL BWP.

- Add NOTE 4 to make it clear that NRB for 60MHz asymmetric UL channel bandwidth is 161 instead the NRB number in Table 5.3.2-1, which has no impact on the NRB for symmetric UL/DL channel bandwidth.

- Reference

[1] R4-2006143 Clarification on asymmetric channel bandwidth operation in FR1 Meeting #107, Incheon, Korea, March 12-14, 2025, Anritsu, RAN4#95-e

[2] R4-2006144 CR to asymmetric CBW operation in FR1, Anritsu, RAN4#95-e

[3] TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, V15.28.0, 2025-01