# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511023

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance requirements for NR NTN phase3

Agenda item:	7.27.6

Document for:	Approval

Introduction

RRM core requirements for Rel-19 NTN enhancements are discussed in past meetings. Since the scope of the core requirements is stable enough, it is time for RAN4 to start working on the performance part of the WI. In this paper we will provide our initial views on RRM performance requirements on NR NTN phase 3.

Discussion

In this section, we would like to provide our initial views on performance requirements on two different topics, which are redcap UEs in NTN scenario and downlink coverage enhancement. In this meeting, we mainly study general scope of these two topics.

  - 2.1 RedCap UEs in NTN scenario

In previous meetings, we have spent lots of time on discussing core requirements on redcap UEs in NTN scenario. To our understanding, the core requirements are quite stable so far and the performance requirements can be discussed in this meeting.

2.1.1 Test Scope

For RedCap UEs in NTN scenario, according to previous agreements discussed in core part, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 1: For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

In core part discussion, we have the agreement as below:

<Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location

From our perspective, first of all, the test cases for legacy normal UE in NR NTN scenario could be as the baseline. Secondly, we know that the some legacy core requirements are impacted by redcap UEs, thus, we shall only recognize the new requirements compared to legacy requirements and analyze the relevant test cases.

For cell re-selection requirements, we have normal cell-re-selection requirements and enhanced measurement requirements, we find that both two requirements are applied for 1Rx and 2Rx UEs. We find that only the UE capability and additional margin are different for 1Rx and 2Rx UEs, which are the same as legacy defined in RedCap WI. Because of this, the new test cases shall be defined in NTN scenario. We also observe that RAN4 will reuse the enhanced cell re-selection requirements for normal UE except for Tdetect being extended by [2] DRX cycles. Thus, we can only have the test case on this part for cell re-selection requirements.

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |

For handover requirements, we have RACH-based handover and RACH-less handover in NTN scenario. For RACH-based handover requirements, the legacy requirements for 2Rx UEs are reused, thus, the test cases for RACH-based handover could be reused. For 1Rx RACH-based handover requirements, RAN4 shall define the related test cases. For RACH-less handover, only 1Rx UEs shall be defined in core part discussion. Consequently, we would like to define the test cases for 1Rx UE.

| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
| --- | --- |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |

For conditional handover, we have time-/location-based CHO without/with L3 requirements. Based on the discussion on core part, we find that only the test cases for time-/location-based CHO with/without L3 measurement for 1Rx UE shall be defined.

| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
| --- | --- |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |

For satellite switching, RAN4 defined the new requirements for 1Rx RedCap UEs with FR1-NTN bands. We shall only consider the relevant test cases on it.

| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| --- | --- |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |

For RRC re-establishment requirements, RAN4 has defined the new requirements for 1Rx RedCap UEs with FR1 NTN bands. However, the requirements for 2Rx RedCap UEs are still under discussion since the existing NTN requirements may need be revised depends on the supported UE capabilities. Consequently, we deem that the new test cases for 1Rx UEs shall be considered. Whether to define the new test cases for 2Rx UE could be discussed.

| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
| --- | --- |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |

The same story for RRC Connection Release with redirection requirements, the potential test cases can be seen as below:

| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
| --- | --- |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |

For RA requirements, the following test cases shall be considered based on core part discussion:

| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
| --- | --- |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |

The same story as RRC re-establishment for RLM/BFD/CBD and L1/L3 measurement. The following test cases could be considered:

| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
| --- | --- |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

For timing requirements, we observe that the existing timing requirements shall be reused without differentiating 1Rx/2Rx. Thus, we deem that the legacy test cases for timing can be reused without defining any test cases for timing requirements.

In core part discussion, for signaling characteristics for 1Rx/2Rx redcap UEs, the agreement can be seen as below:

< Agreement >: - The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands: - Active BWP switch delay, - active TCI state switch delay - UE specific CBW change requirements - Pathloss reference signal switching delay - The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

For the above feature, no new test cases is needed since the legacy test cases could be reused.

Based on above, we propose the following test cases shall be considered in this meeting:

Proposal 2: RAN4 shall define the following test cases for redcap UEs in NTN scenario:

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |
| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |
| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |
| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |
| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

2.1.2 Test Configurations

General principle

When defining the specific test cases for RedCap UEs in NTN scenario, the existing NTN test cases can be used as a baseline. Since many test procedure and parameters defined for NTN can be reused for RedCap UEs in NTN, so we think RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

Proposal 3: The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

Proposal 4: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

General test configuration

- For general test configuration, since RedCap UEs in NTN scenario will also contain the HD-FDD mode, we shall consider this part in test configuration.

- Proposal 5: RAN4 shall consider the following general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

  - 2.2 Downlink coverage enhancement

In previous meetings, we have spent lots of time on discussing core requirements on downlink coverage enhancement. To our understanding, the core requirements are quite stable so far and the performance requirements can be discussed in this meeting.

For downlink coverage enhancement, according to previous agreements discussed in core part, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 6: For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

In core part discussion, only the following requirements are under discussion:

- RRC Re-establishment - RRC Connection release with redirection to NR - HO - CHO - Satellite switch with re-sync - Cell re-selection

For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, we observe that the existing requirements are not changed. Thus, we deem that the current test cases can be reused for these cases.

Proposal 7: For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

For RRC Re-establishment, the new core requirements are defined in the last meeting. Thus, we think the new test case shall be considered for RRC re-establishment requirements.

Proposal 8: For RRC Re-establishment, the new test case shall be considered.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR NTN phase 3:

Proposal 1: For RedCap UEs in NTN scenario, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 2: RAN4 shall define the following test cases for redcap UEs in NTN scenario:

| Cell Re-selection | TC 1: Cell re-selection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements. |
| --- | --- |
|  | TC 2: Cell re-selection to FR1 intra-frequency NR case for 1Rx UE. |
|  | TC 3: Cell re-selection to FR1 inter-frequency NR case for 1Rx UE. |
| Handover | TC 4: Intra-frequency SAN RACH-less handover from FR1-FR1 |
|  | TC 5: Intra-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
|  | TC 6: Inter-frequency SAN handover from FR1-FR1 for 1Rx UE: unknown cell |
| CHO | TC 7: Intra-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 8: Inter-frequency SAN time-based conditional handover without L3 measurement from FR1 to FR1 for 1Rx UEs |
|  | TC 9: Intra-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
|  | TC 10: Inter-frequency SAN time-based conditional handover from FR1-FR1 for 1Rx UE |
| Satellite switching | TC 11: RACH-based hard satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
|  | TC 12: RACH-less soft satellite switching with re-sync from FR1 to FR1 for 1Rx UEs |
| RRC Re-establishment | TC 13: Intra-frequency RRC Re-establishment in FR1 for 1Rx UEs: unknown cell |
|  | [TC 14: Intra-frequency RRC Re-establishment in FR1 for 2Rx UEs: unknown cell] |
| RRC Connection Release with redirection | TC 15: Redirection from NR in FR1 to NR in FR1 for 1Rx UE |
|  | [TC 16: Redirection from NR in FR1 to NR in FR1 for 2Rx UE] |
| Random Access | TC 17: 4-step RA type CBRA test in FR1 for NR standalone for 1Rx UEs |
|  | TC 18: 4-step RA type CFRA test in FR1 for NR standalone for 1Rx UEs |
| RLM/BFD/CBD | TC 19: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 1Rx UEs |
|  | [TC 20: Intra-frequency RLM/BFD/CBD SSB-based measurement in FR1 for 2Rx UEs] |
| L1/L3 measurement procedure | TC 21: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 1Rx UEs |
|  | [TC 22: SA event triggered reporting tests without gap under non-DRX with SSB index reading in FR1 for 2Rx UEs] |

Proposal 3: The existing NTN test cases can be used as a baseline for RedCap UEs in NTN scenario.

Proposal 4: RAN4 only clarify the procedure and parameters which can be reused from NTN, and then define new parameters applicable to RedCap UEs.

- Proposal 5: RAN4 shall consider the following general test configurations as follows:

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | GSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 4 | NGSO, NR HD-FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Proposal 6: For downlink coverage enhancement, some new test cases will be added in the existing NTN test cases, which will also be defined in Clause A.14.

Proposal 7: For RRC connection release with redirection to NR, handover, satellite switch with re-sync and cell re-selection, no new test cases shall be defined.

Proposal 8: For RRC Re-establishment, the new test case shall be considered.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133 Clause A.14