# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511024

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Ku bands for NR NTN

Agenda item:	7.7.7

Document for:	Approval

Introduction

At RAN #107 meeting the revised WI“Introduction of Ku bands for NR NTN”[1] was approved. In the WID, Ku bands have been introduced for NR NTN with the mobile VSAT UE. In #116 meeting we will specify the remaining issues on the topics. In current WID description, RAN4’s work could be seen as below:

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

Discussion

In the last meeting, RAN4 have discussed the most issues for NR NTN on Ku bands. In this paper, we will provide the other potential RRM requirements for NR NTN on Ku bands.

  - 2.1 Assumption for defining RRM requirements in Ku-band NTN

In previous meetings, we did not spend any time on discussing the assumption for defining RRM requirements in Ku band NTN. In this meeting, we would like to provide our views on this issue. First of all, we could see the following candidate options:

- Option 1: (LGE) - The same assumptions considered for Rel-18 Ka-band RRM requirements should apply to Ku-band RRM requirements for both FR1-NTN and FR2-NTN numerologies; - Single SAN Tx beam per radio cell in DL - For inter satellite HO, only blind HO and no neighbor cell measurement - Intra satellite neighbor cell measurement is needed - Same UE Rx beam is used for both serving and neighboring cells which belong to the same satellite (i.e., no Rx beam sweeping) - Option 2: other options are not precluded.

We understand the motivation of this issue, the company deems that the UE type of Ka band and Ku band are same , the same assumption considered in R18 can be considered for Ku band NTN. However, from our perspective, this issue shall be discussed in other specific issues instead of discussing these dependently.

Proposal 1: Assumption for defining RRM requirements shall be discussed in other specific issues instead of discussing these dependently.

  - 2.2 Satellite switch requirements

In the last meeting, RAN4 has discussed the satellite switch delay requirements including hard satellite switch and soft satellite switch, the agreements could be seen as below:

Agreement: - Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies - Define requirement for hard satellite switch, - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms - Further discuss the details on the definition of each component. - FFS whether to define requirements for soft satellite switch - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement - Option for the delay requirement: - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing ms

We have agreed that at least the hard satellite switch will be considered and the switching requirements has already defined in last meeting. The agreement above suggests us to further discuss the details on the definition of each component. From our perspective, the formulation only adds TSAT_beam, which reused from inter-satellite handover requirements. Consequently, the new component refer to inter-satellite handover. The other components refer to legacy hard satellite switching requirements.

Observation 1: The formulation only adds TSAT_beam, which reused from inter-satellite handover requirements. The other components refer to legacy hard satellite switching requirements.

Proposal 2: The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay.

For soft satellite switch on VSAT UE, some companies mentioned that there is no satellite switching delay requirements defined on Ka band. The same manner shall also be considered, that is, RAN4 shall not define the satellite switching delay requirements on Ku band. If defined, only hard satellite switching delay requirements could be considered. However, some companies deem that we could define requirements only for Ku band without considering Ka band. From our perspective, we need to be cautious when defining soft satellite switch on Ku band.

There are two issues shall be considered:
1.Whether to define soft satellite switching delay requirements.

2. If defining soft satellite switching delay requirements, how to define scheduling restriction requirements.

When UE performs soft satellite switch, note that during the switching procedure, the VSAT UE must:

- Maintain the connection with serving satellite

- Synchronize with the target satellite.

Until switching to the target satellite, UE shall release connection with serving satellite. Due to the physical limitation, VSAT UE will only generate one high-gain narrow beam to receive like PDCCH/PDSCH from serving satellite or measure SSB based on SSB time offset from target satellite to do time and frequency synchronization. Consequently, the scheduling restriction mechanisms become essential to resolve this conflict.

However, the motivation of soft satellite switching is to guarantee uninterrupted service for the UE and the source satellite. As we just mentioned before, VSAT UE will only generate one high-gain narrow beam, even if a scheduling restriction is ensured, the interruption for the mechanical UE would still be significant, especially during the rotation of the antenna. Consequently, for VSAT UE, we could compromise to define the soft satellite switching delay requirements or not consider soft satellite for VSAT UE at all. If companies stick to defining soft satellite switching for VSAT UE, only electronic VSAT UE shall be considered. The detailed formulation refers to the above agreement as below:

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing ms

Observation 2: When UE performs soft satellite switch, note that during the switching procedure, the VSAT UE must:

- Maintain the connection with serving satellite.

- Synchronize with the target satellite.

Observation 3: VSAT UE will only generate one high-gain narrow beam, even if a scheduling restriction is ensured, the interruption for the mechanical UE would still be significant, especially during the rotation of the antenna.

Proposal 3: For soft satellite switching, we have two options below:

- Option 1: Do not consider soft satellite switch for VSAT UE.

- Option 2: If defined, only consider electronic VSAT UE. The detailed delay requirements could be as:

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Proposal 4: If soft satellite switching delay requirements for electronic VSAT UE is defined, scheduling restriction shall also be considered on Ku band.

  - 2.3 Inter-satellite cell re-selection requirements

In the last meeting, we did not spend any time on discussing whether to define inter-satellite cell re-selection requirements. In this meeting, we would like to provide our views on this issue.

Candidate options: - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 (Nokia) - Option 2: Do not define inter-satellite cell reselection requirements in this WI. (HW, ZTE, E///) - Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20). (LGE)

It cannot be denied that inter-satellite cell reselection is not a corner case, which is benefit to the VSAT UE located at the cell edge in a different satellite. Nevertheless, we prefer to not define inter-satellite cell reselection for VSAT UEs in R19 since RAN4 does not define this requirement in Ka band in R18. The company also said that during R18 WI, when the VSAT requirements were first created for spec, there was limited time for discussing this potential scenario, and RAN plenary took the decision to limit the scope of NTN applicability. Thus, we would not like to define such complex scenario unless RAN plenary agree to discuss this.

Proposal 5: Do not define inter-satellite cell reselection requirements in this WI.

  - 2.4 RLM

The following agreements can be seen:

Candidate options: - Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. - Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

In the last meeting, we just clarified that LMAX is only impacted by different frequency range, the legacy values can be seen as below:

Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM

| Carrier frequency range of PCell | Lmax | Maximum number of RLM-RS resources, NRLM |
| --- | --- | --- |
| FR1, ≤ 3 GHzNote | 4 | 2 |
| FR1, > 3 GHzNote | 8 | 4 |
| FR2-NTNNote 2 | 64 | 8 |
| NOTE: For unpaired spectrum operation with Case C - 30 kHz SCS, 3 GHz is replaced by 1.88 GHz, as specified in clause 4.1 in TS 38.213 [3]. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |  |

For this reason, we hold the view that LMAX defined for FR1-NTN applies for VSAT working in FR1-NTN. Meanwhile, LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. RAN4 shall not change this table.

Proposal 6: LMAX defined for FR1-NTN applies for VSAT working in FR1-NTN. Meanwhile, LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. RAN4 shall not change table 8.1C.1-2.

  - 2.5 Terminology update

In previous meeting, RF session has already defined the new frequency range for Ku band NTN. We confirm the range of FR2 is extended and starting from 10GHz. The range of FR1 is also extended.

Proposal 7: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Proposal 1: Assumption for defining RRM requirements shall be discussed in other specific issues instead of discussing these dependently.

Observation 1: The formulation only adds TSAT_beam, which reused from inter-satellite handover requirements. The other components refer to legacy hard satellite switching requirements.

Proposal 2: The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay.

Observation 2: When UE performs soft satellite switch, note that during the switching procedure, the VSAT UE must:

- Maintain the connection with serving satellite

- Synchronize with the target satellite.

Observation 3: VSAT UE will only generate one high-gain narrow beam, even if a scheduling restriction is ensured, the interruption for the mechanical UE would still be significant, especially during the rotation of the antenna.

Proposal 3: For soft satellite switching, we have two options below:

- Option 1: Do not consider soft satellite switch for VSAT UE.

- Option 2: If defined, only consider electronic VSAT UE. The detailed delay requirements could be as:

Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

Proposal 4: If soft satellite switching delay requirements for electronic VSAT UE is defined, scheduling restriction shall also be considered on Ku band.

Proposal 5: Do not define inter-satellite cell reselection requirements in this WI.

Proposal 6: LMAX defined for FR1-NTN applies for VSAT working in FR1-NTN. Meanwhile, LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. RAN4 shall not change table 8.1C.1-2.

Proposal 7: For Ku-band in FR2-NTN numerology, not to change the existing terminologies: ‘VSAT UE in FR2-NTN’ and ‘FR2-NTN’.

- References

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN.

- R4-2508262, WF on RRM requirements for NR_NTN_Ku_bands, Apple.