# 3GPP TSG-RAN WG4 Meeting #116                                  R4-2511026

# Bangalore, India, 25th – 29th August, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LP-WUS

Agenda item:	7.24.5.2

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

LP-WUR status at legacy case

In previous meetings, we do not discuss the UE behaviour at legacy case (Case#5), the following way forward can be seen as below:

- Proposals - P1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. - P1-1: No LP-WUR measurement and evaluation requirement is applicable when LR is “on” at the legacy state; - Note: in detail, no requirement is applicable on LR for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). - P2: After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined. - P3: LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures

There is a common understanding that the partially offloading and fully offloading measurement will be performed when LR is ON. LR only performs the serving cell measurement. Regarding this issue, we would like to split two parts:

- Whether the LR is ON in legacy case.

- If LR is ON, whether the RRM requirements will be defined.

- For the first, we have reached the agreement as below:

Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

- For the second point, RAN2 has the agreement as below:

- NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

- From our perspective, LR only performs serving cell measurement in Case#3 and Case#1. If LR is ON in legacy case, there is no measurement behaviour, only MR performs L3 measurement. Thus, no requirement is applicable when LR is at legacy case. However, RAN2’s agreement shows that NW configures thresholds for both MR and LR measurement, then the LP-WUS monitoring entry condition is met when all the measured results are above the configured thresholds. It means if LR is ON, then the LR would perform serving cell measurement. We know that the channel condition shall be guaranteed if LR is ON and performs serving cell measurement. However, if LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern, that is, we should not consider this case and do not define any RRM requirements.

- Observation 1: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 2: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 1: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

Periodicity for LP-WUR RRM requirements

In previous meetings, RAN4 has already discussed the periodicity for LP-WUR RRM requirements. There are still some detailed issues shall be discussed and the previous agreement as follows:

RAN4 113 agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. RAN4 114 way forward: - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity. RAN4 114bis meeting: Agreement: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity.

We think the upper bound is need on SSB-based LP-WUR measurement periodicity. Actually, defining the upper bound is beneficial to the service quality for LR measurement. It is a trade-off between power saving and how fast UE can exit LR when the channel condition is worse. To our understanding, in cases where channel condition drastically deteriorates, if there is a rapid exit from the current LP-WUR mode coupled with corresponding adjustments, UEs will be able to respond more effectively to unexpected situations. For instance, during mobility, when a UE transitions from a region with high-quality links to an area with weaker links, it needs to promptly detect this change and implement appropriate measures to maintain communication quality. If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Observation 3: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 2: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

RRM requirements for FR2

The current way forward could be seen as below:

- Proposals P1: Prioritize defining RRM requirements for FR1 in this WI (Apple Nokia) P2: There is not any concrete proposal on FR2 requirements so far. Suggest RAN4 to confirm whether FR2 RRM requirement needs be defined in this release, or confirm the status of FR2 requirement will not impact the completion of WI (vivo, Nokia)

First of all, the latest WID does not preclude FR2, RAN1/RF session have some discussion and agreements on FR2. In theory, RRM session shall consider defining related RRM requirements on FR2. However, considering RAN4 workload and the complexity of defining FR2 requirements with beam sweeping, RAN4 could prioritize FR1 issues in this WI. To sum up, we are open to consider LP-WUS on FR2.

Proposal 3: We are open to consider RRM requirements on FR2.

Detail on LR accuracy and side conditions requirements

In the last meeting, we spent some time on discussing RF impairment margin, there are two options as below:

Proposals - P1: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. - P2: Consider 3.5 dB RF impairment margin for LR

Most companies support P1, which use 2.5dB as RF impairment margin like legacy. The legacy margin could be seen as below:

R4-1904820: Normal condition with Io=-70dBm/BW is derived by adding 1.0dB (fading) + 2.5dB (RF margin) to the average of alignment results (1.5dB). => 1.5+2.5+1.0 = [+/-5.0]dB

Furthermore, RF session has already defined the IM margin in previous meeting which is similar to RF impairment margin as below:

- Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB) - Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS. - LS should also cc RAN2

From MAPL perspective, we know that the more MAPL, the larger coverage. We know that

MAPL= Tx_power + Tx_ant_gain + Rx_ant_gain - Rx_sensitivity - Margin_total

RF margin is included in Margin total, if the RF margin is large, the MAPL will be decreased and the communication coverage will be decreased. If the same coverage is ensured under different RF margin, the power consumption will be increased, which is not good to power saving neither for network nor UE. For network, if larger RF margin is used, the transmission power shall be increased, or the REFSENSE of UE will be improved.

From the perspective of L1-RSRP accuracy, low RF margin reduces the system's tolerance for measurement errors. This requires the UE to provide higher reliability in L1-RSRP measurement accuracy to maintain system performance.

Hence, we strongly deem that the RF impairment margin shall reuse legacy value or follow RF session decision as 2.5dB to define L1-RSRP accuracy in FR1.

Observation 4: RF session has already defined the IM margin as 2.5dB.

Observation 5: If the RF margin is large, the MAPL will be decreased and the communication coverage will be decreased. If the same coverage is ensured under different RF margin, the power consumption will be increased, which is not good to power saving neither for network nor UE.

Proposal 4: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements.

Maximum interruption in paging reception

For maximum interruption in paging reception, all companies deem that this requirements shall be reused. However, there is no discussion on it, so we just would like to clarify the requirements shall be reused in LP-WUS.

Proposal 5: The maximum interruption in paging reception requirements shall be reused in LP-WUS.

RedCap related

In previous meetings, several companies mentioned that RAN4 shall also consider the RedCap UEs with LP-WUR measurement. We agree with this feature since RAN1 has already studied the normal UE and RedCap UE with LP-WUR, and we do not see any issues to combine with them.

The next step is how to define the related requirements for RedCap UEs with LP-WUR. To our understanding, we shall divide into five parts:

- 2Rx RedCap UE with MR;

- 1Rx RedCap UE with MR;

- 1Rx RedCap UE with LR;

- HD-FDD RedCap UE with MR;

- HD-FDD RedCap UE with LR.

Subsequently, we would like to provide our initial views on above six parts.

1. 2Rx RedCap UE with MR:

Actually, the requirements for normal UE and 2Rx RedCap UE with MR are same in legacy definition. To follow the same manner, the requirements defined for like MR RRM relaxation, evaluation requirements etc will be reused for 2Rx RedCap UEs. The MR wake up delay requirements (the number of SSB) could also be reused.

- 2.1Rx RedCap UE with MR:

  - For 1Rx RedCap UE, the requirements are the same as normal UE with specific side conditions, such as

4.2B.2.3 Measurements of intra-frequency NR cells for RedCap UE when rangeToBestCell is not configured: - the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 for 2 Rx RedCap. - the cell is at least 4 dB better ranked in FR1 for 1 Rx RedCap. when rangeToBestCell is configured: - the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell. - if there are multiple such cells, the cell has the highest rank among them. - the cell is at least 3 dB better ranked in FR1 or 4.5 dB better ranked in FR2 if the current serving cell is among them for 2 Rx RedCap. - the cell is at least 4 dB better ranked in FR1 if the current serving cell is among them for 1 Rx RedCap. - The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

  - Consequently, the requirements for 1Rx RedCap UE with MR is the same as 1Rx RedCap UE in legacy with specific side conditions. However, there is no MR wake up delay requirements in legacy. In the last meeting, we spent lots of time on discussion of MR wake up delay, the agreements could be seen as below:

Agreement: For the 400ms and 800ms ramping up time: {5, 5} samples - For the 10ms ramping up time: 3 samples

  - The same manner shall be followed, that is, the same number of SSB can be reused for 1RedCap UE with LR. RAN4 shall consider how to define the side conditions.

- 1Rx RedCap UE with LR:

  - One agreement agreed in RAN4 #112bis meeting could be seen as follows:

Issue 1-2-5: Nubmer of Rx for LP-WUR requirements Agreement - Single Rx is the baseline for LR-WUR RRM measurement.

  - During the discussion, we think LP-WUR is a quite low power-consumption and low-cost component on the UE, and it shall be assumed as single Rx as a more typical case for requirement design. The final agreement above says the 1Rx is the baseline for LP-WUR RRM measurement. Consequently, we believe that the requirements for normal UE with LR could be reused for 1Rx RedCap UE with LR.

- HD-FDD RedCap UE with MR

  - For LP-WUS topic, RAN4 only study the RRM requirements for RRC IDLE/INACTIVE state based on the latest WID. Thus, we shall only consider requirements for the HD-FDD RedCap UE with MR, which is maximum interruption in paging reception for RedCap. In current spec, one description is following:

4.2B.2.6 Maximum interruption in paging reception for RedCap The FDD, HD-FDD and TDD RedCap UE shall meet all applicable requirements specified in clause 4.2.2.6. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2.2.6 under the following conditions - - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

  - In my opinion, the requirements defined in clause 4.2.2.6 shall be reused for HD-FDD RedCap UE with MR based on above description.

- HD-FDD RedCap UE with LR

  - For HD-FDD RedCap UE with LR, the above table also gives the requirements availability. Hence, the same availability shall be reused, then the legacy requirements on maximum interruption in paging reception can be the same.

Based on above analysis, the summary table as follows:

| Cases | Requirements to define |
| --- | --- |
| 2Rx RedCap UE with MR | - Reused legacy 2Rx RedCap UE RRM requirements; - Reused wake up delay requirements (the number of SSB) |
| 1Rx RedCap UE with MR | - the requirements for 1Rx RedCap UE with MR is the same as 1Rx RedCap UE in legacy with specific side conditions. - Reused wake up delay requirements (the number of SSB) - RAN4 shall define the specific side conditions |
| 1Rx RedCap UE with LR | The requirements for normal UE with LR could be reused for 1Rx RedCap UE with LR. |
| HD-FDD RedCap UE with MR | The legacy maximum interruption in paging reception requirements shall be reused for HD-FDD RedCap UE with MR |
| HD-FDD RedCap UE with LR | The same legacy availability rule shall be reused, then the legacy requirements on maximum interruption in paging reception can be the same. |

Proposal 6: RAN4 shall cosnsider RedCap UEs with MR/LR, there are six cases shall be considered, the following table shall be considered:

| Cases | Requirements to define |
| --- | --- |
| 2Rx RedCap UE with MR | - Reused legacy 2Rx RedCap UE RRM requirements; - Reused wake up delay requirements (the number of SSB) |
| 1Rx RedCap UE with MR | - the requirements for 1Rx RedCap UE with MR is the same as 1Rx RedCap UE in legacy with specific side conditions. - Reused wake up delay requirements (the number of SSB) - RAN4 shall define the specific side conditions |
| 1Rx RedCap UE with LR | The requirements for normal UE with LR could be reused for 1Rx RedCap UE with LR. |
| HD-FDD RedCap UE with MR | The legacy maximum interruption in paging reception requirements shall be reused for HD-FDD RedCap UE with MR |
| HD-FDD RedCap UE with LR | The same legacy availability rule shall be reused, then the legacy requirements on maximum interruption in paging reception can be the same. |

UE capability

we shall clarify the related UE capability before defining the related requirements.

In my opinion, for the normal UE with LP-WUS, RAN4 shall not consider the related UE capability firstly since RAN1 is under discussion. RAN4 could wait for RAN1’s final decision and then to decide whether to define UE capability. For RedCap UE with LP-WUS, to our understanding, if the UE supports feature group 26 (R17 RedCap)/feature group 28 (R18 RedCap) and feature group 62 (R19 LP-WUS), then the requirements for RedCap with LP-WUS can be applied instead of introducing new capability. However, the discussion on RedCap UE with LP-WUS capability is not observed in RAN1, maybe RAN4 could define this or send LS to RAN1 to define this. Of course, this feature only apply for R19 UEs not for R17 or R18 UEs.

Observation 6: For normal UE with LP-WUS, RAN1 is discussing the UE capability.

Observation 7: For RedCap UE capability with LP-WUS, the UE capability among R17 RedCap, R18 RedCap and R19 LP-WUS are independently. The discussion on RedCap UE with LP-WUS capability is not observed in RAN1.

Proposal 7: For UE with LP-WUS, RAN4 shall wait for RAN1’s discussion on UE capability. The new UE capability shall not be introduced.

Proposal 8: The feature for RedCap UE with LP-WUS only applies for R19 UEs not for R17/R18 UEs.

Conclusion

In this paper we provided our views on LPWUS.

- Observation 1: The channel condition shall be guaranteed if LR is ON and performs serving cell measurement.

- Observation 2: If LR performs serving cell measurement at legacy case, how to guarantee the channel condition in legacy case and how to ensure the measurement accuracy by LR constitute the main concern.

Proposal 1: RAN4 shall not define the RRM requirements if LR is ON at legacy case.

Observation 3: If the communication quality can not be guaranteed, the power saving gain would not be considered, and we do not think if the upper bound is settled, the power consumption will be largely increased.

Proposal 2: The upper bound is needed on SSB-based LP-WUR measurement periodicity if LP-SS is not configured.

Proposal 3: We are open to consider RRM requirements on FR2.

Observation 4: RF session has already defined the IM margin as 2.5dB.

Observation 5: If the RF margin is large, the MAPL will be decreased and the communication coverage will be decreased. If the same coverage is ensured under different RF margin, the power consumption will be increased, which is not good to power saving neither for network nor UE.

Proposal 4: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements.

Proposal 5: The maximum interruption in paging reception requirements shall be reused in LP-WUS.

Proposal 6: RAN4 shall cosnsider RedCap UEs with MR/LR, there are six cases shall be considered, the following table shall be considered:

| Cases | Requirements to define |
| --- | --- |
| 2Rx RedCap UE with MR | - Reused legacy 2Rx RedCap UE RRM requirements; - Reused wake up delay requirements (the number of SSB) |
| 1Rx RedCap UE with MR | - the requirements for 1Rx RedCap UE with MR is the same as 1Rx RedCap UE in legacy with specific side conditions. - Reused wake up delay requirements (the number of SSB) - RAN4 shall define the specific side conditions |
| 1Rx RedCap UE with LR | The requirements for normal UE with LR could be reused for 1Rx RedCap UE with LR. |
| HD-FDD RedCap UE with MR | The legacy maximum interruption in paging reception requirements shall be reused for HD-FDD RedCap UE with MR |
| HD-FDD RedCap UE with LR | The same legacy availability rule shall be reused, then the legacy requirements on maximum interruption in paging reception can be the same. |

Observation 6: For normal UE with LP-WUS, RAN1 is discussing the UE capability.

Observation 7: For RedCap UE capability with LP-WUS, the UE capability among R17 RedCap, R18 RedCap and R19 LP-WUS are independently. The discussion on RedCap UE with LP-WUS capability is not observed in RAN1.

Proposal 7: For UE with LP-WUS, RAN4 shall wait for RAN1’s discussion on UE capability. The new UE capability shall not be introduced.

Proposal 8: The feature for RedCap UE with LP-WUS only applies for R19 UEs not for R17/R18 UEs.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869

[3] R4-2503116