3GPP TSG-RAN WG4#116	R4-2511057

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	4.2.1

Source: 	Qualcomm

Title: 	About near-miss RX mixing

Document for:	Discussion

# Introduction

Three band combinations, CA_n7A-n26A, CA_n14-n30A, CA_n28A-n78A have been observed as “near miss” combinations in [1] and further captured in WF [2]. This contribution discusses some further aspects related to these combinations.

- Discussion

In short, the difference to RX mixing with direct hit and Near-miss RX mixing is that in the case of direct hit the aggressor TX power is down-converted on top of victim DL, while in near miss RX missing some of the ACLR power of the aggressor TX is down-converted on top of victim DL. MSD analysis was conducted according to usual RAN4 practices for all combinations mentioned above, using some of the ACLR power as the aggressor.

A bunch of CR’s starting Rel-15 [3],[4],[5], [6], aligned with evaluation in [1] were submitted in Malta meeting.

After the submission it was identified that while the impact of some of the ACLR power down-converted on top of victim DL was fully evaluated in [1], the impact of the down-converted TX aggressor, which is very close to the victim channel was not evaluated. Consequently, the CR’s were postponed. We note that this TX aggressor could act as a blocker in the RX chain after down-conversion. We evaluated this aspect for PC3, PC2, and PC1.5 and found the impact to be negligible to the MSD’s evaluated in [1]. Hence CR’s can be agreed in RAN4-116.

Observation 1: The impact of down-converted TX aggressor is negligible to the MSD’s in [1], so respective CR’s can be agreed in RAN4-116

- Conclusion

Further considerations on near miss RX mixing for CA_n7A-n26A, CA_n14-n30A, CA_n28A-n78A were provided with the following observation.

Observation 1: The impact of down-converted TX aggressor is negligible to the MSD’s in [1], so respective CR’s can be agreed in RAN4-116

- Reference

[1] R4-2507910, “On near-miss for Rx harmonic mixing”, Skyworks Solutions, Inc., Anritsu, Qualcomm France, Oppo

[2] R4-2502956, “WF on MSD rules for near-miss Rx mixing”, Skyworks

[3] R4-25-7892, “[NR_newRAT-Core] CR to TS38.101-1-gn0: Correction to harmonic mixing MSD – CatF”, Skyworks Solutions Inc., Anritsu, Qulacomm France

[4] R4-2507686, “[NR_CADC_R17_2BDL_xBUL-Core] CR to TS38.101-1-hh0: Correction to harmonic mixing MSD – CatF, Skyworks Solutions Inc., Anritsu, Qulacomm France

[5] R4-2507688,  [NR_CADC_R18_2BDL_xBUL-Core] CR to TS38.101-1-i90: Correction to harmonic mixing MSD – CatF”, Skyworks Solutions Inc., Anritsu, Qulacomm France

[6] R4-2507689, “[NR_CADC_R18_2BDL_xBUL-Core] CR to TS38.101-1-j10: Correction and missing harmonic mixing MSD – CatA, Skyworks Solutions Inc., Anritsu, Qulacomm France