3GPP TSG-RAN WG4 Meeting #116															R4-2511073

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.3.3

Source:	Samsung

Title:	TP for 38.768 on switching period

Document for:	Discussion

1	Introduction

RAN4 has discussed a Rel-19 Work Item (WI) on low band (LB) carrier aggregation (CA) via switching, which was initiated to address a market demand for LB utilizations [1]. At the latest meeting, RAN4 achieved concrete progress such as time mask with transient period, switching period and switching gap as captured in [2]. This contribution is a text proposal to reflect the agreement in [2] on switching period.

2	Discussion

As captured in [2], it is agreed to support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us. The agreement should be reflected in TR 38.768 [3].

Reference

[1] 	RP-243317,	New WID on low band carrier aggregation via switching, RAN#106

[2]	R4-2508118,	WF on NR_LBCA_Sw, RAN4#115

[3]	3GPP TR 38.768 V1.0.0

TEXT PROPOSAL:

< start of text proposal >

## 5.1	Switching periods

To accommodate different UE implementations, RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs, 70μs and 140μs for switching between Case 1 and Case 2 as an optional UE capability.

## 5.2	Switching period location

### 5.2.1	General

For what follows we consider switching between an FDD and SDL band for the CA-n5A-29A configuration in Table 4-1 but without loss of generality: one UL is configured in a paired band with the DL Scell configured in either a DL band of another paired band or in an SDL band.

The switching period, defined in units of 𝜇s according to UE capability, is located within a switching gap located within the switched-from carrier. Switching gaps, expressed in units of symbols, are accommodated by the network according to the physical layer design of the feature.

The timing reference for the switching pattern and switching gap at the UE is the DL timing of the Pcell (in the FDD band). The uplink radio frame transmission on the Pcell takes place TTA = (NTA + TAoffset)Tc before the reception of the first detected path (as defined in TS 38.133) in time on the corresponding DL radio frame. This means that the timing advance must be considered at switching events.

The duration of the switching gap is RRC configured. The network ensures that the switching gap is large enough to cover the switching period (35, 70 or 140 𝜇s according to the UE capability), transient periods and the TA, if needed, in both switch directions.

### 5.2.2	FDD to SDL switch

For the FDD to SDL switch the gNB configures a gap long enough to accommodate timing advance such that no collisions should occur. Figure 5.2.2-1 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods, switching period (35, 70 or 140 𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier.

The UL slot will always end before the DL slot, neglecting any timing errors in case TA = 0 µs. The worst case in terms of switching is that in which TTA = 0 µs, the DL and UL timing are the same (again neglecting the errors that are of the order of 1 µs) and the SDL starts RTD µs before the Pcell DL slot timing. The switching gap with respect to the end of the last FDD DL symbol must therefore be at least

actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period.

where the MRTD = [CP length/TAE] assuming that the FDD and SDL carriers are colocated.

### 5.2.3	SDL to FDD switch

For the SDL to FDD switch, the switching gap ends at end of slot on “switch from” carrier (SDL). The timing advance must also be accounted for; Figure 5.2.3-1 shows the case where the switch is assumed to occur before the start of the time advanced UL. The figure illustrates the applicable transient periods, switching period (35, 70 or 140 𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Figure 5.2.3-1: SDL to FDD switch with the switch occurring before the start of the UL slot.

The time mask could be devised such that the switching period occurs before the first symbol of the UL slot of the switched-to FDD carrier, this to avoid dropping symbols on the UL that starts TTA before the first symbol of the FDD DL slot. Notwithstanding, the switching gap on the SDL carrier must be at least

Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period

with account of the SDL received at maximum RTD of a [CPlength/TAE]. The worst case is when the SDL is traling the Pcell DL as shown in the figure.

< end of text proposal >