3GPP TSG-RAN WG4 Meeting #116	R4-2511080

Bengaluru, India, August 25th – 29th 2025

Agenda Item:	11

Source:	Ericsson

Title:	Considerations on RAN4 5G-A package

Document for:	Discussion

# 1	Introduction

During RAN#108, further discussion took place on the Rel-20 RAN4 5G-A RAN4 led package, in parallel with conclusion on the RAN1-3 led package. This contribution presents views on the WF from RAN#108 on RAN4 led topics.

It is important to bear in mind that the total RAN4 workload will consist of:

- Maintenance

- Release 19 performance

- Spectrum

- RAN4 work arising from the RAN1-3 5G-A package

- The RAN4 led 5G-A package

- Last but not least, the 6G Study Item

With this in mind, it is important to tightly manage the workload arising from the RAN4 package. Although the WF arising from RAN#108 is good in general, further work to reduce the scope is needed.

# 2	Discussion

## 2.1	UE RF evolution

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109):

- FR1 HPUE enhancement

  - Further consider/discuss until RAN#109 on the detailed scope

  - This work is to focus on the TN application

- 6MHz channel bandwidth

  - It can be prioritized to FDD band <1GHz

- Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM

  - BS demod scope is included.

  - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact

    - The details for study is to be discussed until RAN#109

  - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G.

- No RAN1/2 scope is expected

- Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

The scope of the UE enhancements discussed at RAN#108 consists of HPUE, 6MHz and UE EVM enhancements based on post-distortion receiver.

Regarding the HPUE, filling in the remaining gaps HPUE for 5G-A is an important and commercially relevant step, and it is natural that HPUE should be prioritized as part of the UE RF enhancements work. A variety of different single carrier and CA HPUE architectures have been proposed, including 4*26dBm, 2*29dBm, PC1.5 single TX, PC1.5 2TX FDD and CA variations. Due to the workload of RAN4, it is important to identify which of the UE architectures is likely within Rel-20 timescales and should be prioritized.

An important aspect of the prioritization is to determine which PA architectures are of interest to target. The commercial availability of 29dBm Pas should be considered and if confirmed, it should be discussed whether to focus on 29dBm PA related options. For the other combinations, in particular for PC1 it is important to understand whether the MPR requirements may end up so large that the desired power capability is compromised.

Establish whether to focus on 29dBm PA technology and/or existing PA technology and keep the total scope for HPUE manageable.

Regarding 6MHz, this is a final piece of the irregular channel bandwidths discussion and is not likely to be a significant workload.

Adding 6MHz bandwidth is reasonable

Regarding post-distortion compensation for UE EVM, it is important that the topic is properly studied. The aim of BS post-distortion compensation is to enable a reduced MPR for UEs that are transmitting higher order modulation and also are at near to maximum TX power. In principle, if the BS can compensate PA distortions at the UE then the UE PA can be driven harder, implying reduced MPR and greater UE transmit power and coverage. This is only the case if the factor that limits MPR at the UE is transmit signal quality. In many circumstances, the limiting factor is adjacent channel emissions. Since adjacent channel basestations do not demodulate the UE RX signal, and there is no in-band UE signal within their channels, adjacent channel basestations will not compensate UE distortion. Hence, adjacent channel emissions cannot be relaxed and consequently, if adjacent channel emissions limit the UE MPR then the MPR cannot be relaxed.¨

When the MPR is limited by in-channel self-interference (EVM) then post-distortion compensation by the intended basestation does have some chance of enabling MPR reduction. Of course, MPR reduction is only relevant if the UE is at maximum TX power, and so a system evaluation of the overall relevance of MPR reduction for high order modulations would be useful.

Depending on the SNR and UE RF front end, the distortions within the UE may not be linear. If the UE RF front end employs any form of clipping and/or pre-distortion then the EVM will contain non-linear components. Also, depending on the RB allocation and operating point as well as the frequency, other forms of distortion may be present in the self-interference such as phase noise, IQ imbalance, carrier leakage etc.

When evaluating the potential for increasing EVM, it is important to base the evaluation on a realistic model for the UE RF front end. Furthermore, it is important to determine if there is a dependency of the potential gains on RB allocation, or if the basestation compensation algorithm may need adjustment or may exhibit different performance for different UEs. In particular, if the post-distortion compensation performance would differ between UEs then it would become difficult for the network to manage EVM relaxation.

BS post-distortion compensation may be non-AI or AI based. It has been proposed to consider non-AI based in 5G, and AI based is likely to be proposed for 6G study. Consideration should be given as to whether it might make sense to align the 5G and 6G studies, possibly by studying both within the context of 6G.

UE EVM reduction based on BS post-distortion compensation requires proper study and will need expertise from both UE RF and demodulation. Consider including in 6G to ensure proper study.

## 2.2	BS RF evolution

A more detailed analysis of BS RF evolution priorities for 5G is provided in [R4-2510229]

For BS RF, consider co-location requirement improvement and the new hybrid BS type for 5G-A

For BS RF, consider re-evaluation of the receiver blocking requirement

## 2.3	RRM

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

- (e)RedCap UE enhancement

  - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI

  - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI

    - This also include example bands, UE RF scope including potential complexity reduction, UE demod

- Fast SCell activation

  - Further discussion on the possible down scoping in RAN#109 based on the following two objectives

    - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting

      - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026.

    - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA

  - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation

- No RAN1/2 scope is expected

- Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

Relaxed RLM and BFD measurements for Redcap can enable improved energy efficiency for Redcap devices with low mobility. For 2RX, as long as it is based on the existing non-Redcap functionality, incorporation of relaxed RLM and BFD requirements should be straightforward. For 1RX, some work is needed for performance requirements, but the workload is not large, and experience already exists from existing 2RX requirements and work on relaxations for 1RX for Redcap devices.

Relaxed RLM and BFD for Redcap should be based on non-Redcap and existing principles for 1RX relaxation.

Less than 5MHz for eRedcap is straightforward for similar reasons, as long as the requirements and functionality is based on non-Redcap less than 5MHz implementation. 2RX requirements should be straightforward to carry from non-Redcap, whilst a small amount of work will be needed to calculate 1RX relaxations. As noted in the WF, RF and demodulation work will be needed.

Less than 5MHz for Redcap should be based on non-Redcap and existing principles for 1RX relaxation.

Faster SCell activation of CA is highly important for CA performance in bursty traffic. It is important that the network and UE can make use of CA quickly when the UE has traffic bursts without the signaling and energy overheads of keeping carriers active when the UE does not have data. It is important to take into account that fast SCell activation has already been the subject of much attention.

RAN4 has defined RRM requirements for various types of SCell activation procedures. However, due to the limited timeframe for the Rel-20 RRM package, efforts should be focused on areas relevant to existing deployments and/or those with commercial interest. While multiple SCell activation procedures are supported in the specifications, only a subset is currently implemented and used in real-world networks. Therefore, enhancements or optimizations for SCell activation procedures that are not commercially deployed should be deprioritized.That said, if companies are still interested in further improving SCell activation requirements, we recommend that RAN4 consider extending the Rel-19 FR2 measurement delay reduction work for for intra-band contiguous CA to FR1. RAN4 is already working on FR2 measurement delay reduction for intra-band contiguous CA, and extending this solution to FR1 would be both feasible and manageable from a workload perspective.

Keep the scope of fast SCell activation small or zero.

If there is any introduction of fast SCell activation, consider extending the Rel-19 FR2 measurement delay reduction solution for intra-band noncontiguous CA to FR1.

Any proposed improvements must be tangible and with a low standardization effort, and it is important that there is a commitment to implement the CA features.

One potential improvement is for CA activation for intra-band contiguous co-located scenarios, and specifically is to skip the CSI reporting as part of the activation requirement. The CSI reporting is used to inform the network about when the procedure is completed and thereby to start scheduling. The impact of removing this step is unclear.

The A-TRS based unknown S-Cell activation is likely to be more complex, in particular when other bands in which there is no active carrier are considered, since the criteria to schedule A-TRS for the network is not known. Furthermore, it is not obvious that A-TRS will be implemented within the Release-20 lifetime. In order to keep workload reasonable, we propose not to further consider the A-TRS/TRS proposals.

Do not consider A-TRS based faster SCell activation.

## 2.4	Demod

The high-level scope for WI on Demod enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

- Normative work on SCM based requirements for SU-MIMO in FR1

  - The exact scope also depends on the conclusion of R19 study

- Study on SCM for MU-MIMO

- Performance requirements for 6Rx UE with interference

  - Up to 4 MIMO layers for the target cell is considered

- No RAN1/2 impact is expected

- Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

Development of a spatial channel model is important in order that advanced MIMO features, that are based on AAS basestation systems that aim to take advantage of spatial characteristics of the radio channel function properly and bring a return on the investment on the network and UE sides. During Rel-15, there was an attempt to introduce CDL channel models for demodulation requirements, but this was not successful due to lack of convergence in the available time. Meanwhile, all MIMO feature development and evaluation in RAN1 is based on CDL.

During Release 19, a Study has taken place in RAN4 to develop a stable, reproducible spatial channel model and to verify that the CDL model is indeed the most relevant model to consider. The study is now converging, at least for the case of FR1 and SU-MIMO.

During Release 20, it is useful to create an additional UE requirement based on the spatial channel model, in order to enhance 5G and to be ready for 6G. The requirement should be straightforward and should be in addition to, not a replacement of, existing requirements. We propose that the following is considered for a release 20 requirement:

- PDSCH requirement for SU-MIMO (8 layers)

- Type II PMI reporting requirement for SU-MIMO

Create optional demodulation requirements based on SCM:

  - PDSCH requirement for SU-MIMO (8 layers)

  - Type II PMI reporting requirement for SU-MIMO

In addition, it is proposed to continue to study the spatial channel model. Areas that have not been studied in Rel-19 include MU-MIMO, FR2 and SCM for AI. For the MU-MIMO case, the study is essentially about an interference model for a multi-user system. Regarding AI, a key issue to study is how to extend the model to incorporate more diversity into the model parameters and in particular the spatial directions of arrival, in order to adequately test that the AI model generalizes and is not overtrained for specific spatial modes. The SCM should also be studied for 6G. For 5G, we propose to continue with a study of MU-MIMO.

Study further MU-MIMO SCM. Within the context of 6G, further study SCM for AI/ML.

## 2.5	NTN

High level scope for WI on NTN enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

  - NTN HPUE

    - Finish remaining PC1.5 (~29 dBm) work for hand-held devices for both NR NTN and IoT NTN (several studies and discussions already done in R19)

      - NR NTN with 2TX and/or FFS 1TX

  - NTN HD-FDD VSAT for Ka/Ku bands

    - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm).

    - Once the mobile VSAT with small antennas work is done,

      - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable

    - RAN1/2 impact is expected and will be further discussed in RAN#109

      - Leverage RedCap over NTN HD-FDD solution to NTN as applicable.

  - NTN DL CA

    - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC

      - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands

      - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

    - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3

      - Take one example band with minimum work with PC3

      - The rest cases can be considered as part of spectrum work once the requirements on the example band is completed

- Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26

The proposed NTN topics are in general commercially relevant, although in different use cases. HPUE is an enabler for improved performance for L- and S- bands. Similar to the TN case, it is important to prioritize to architectures that are likely to be commercially relevant and realistic in the Release-20 lifetime.

The HD-FDD reduced VSAT proposal has relevance to optimizing Ku band terminals.

Regarding CA, intra-band contiguous CA is most relevant for Ku band and non-contiguous for L band.

The additional bandwidths should aim to re-use TN bandwidths as much as possible. Data on the real relevance of the bandwidths has not as yet been presented.

In general, considering the available RAN4 time and the significant diversity of NTN proposals, it would be desirable to aim to further rationalize the proposals, taking into account what is most relevant for 5G and the Rel-20 timeline, as far as is possible. Some considerations with regard to downsizing the NTN work scope include:

- Whether 29dBm PA is seen as feasible

- Whether PC1.5 2TX FDD can be operated for NTN without significant power back-off

- Synergies with the TN HPUE

- Identifying the relevance of the CA scenarios

- For the bandwidths, identifying the relevance of the bandwidths, and maximizing synergies with the TN specifications

Further consider how to limit the scope of HPUE, CA and bandwidths for NTN as far as possible.

## 2.4	Cross topic

The following two WI are agreed to be introduced in R20.

- Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109):

  - Further discussion on the possible scoping in RAN#109 based on the following two objectives

    - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration

    - Dual DL solution

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

- UAV (the exact scope and WID draft will be discussed and decided in RAN#109):

  - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum

  - RAN1/2 scope is not expected

  - Target 0.5 TU allocation (0.25 RF and 0.25 RRM)

The following two proposals will be discussed at RAN#109, where it will be decided whether or not to include either of the proposals

  - Fragmented DL:

    - Introducing RF/RRM requirements related to new UE behavior for reducing the number of UE Rx RF chains

    - RAN1/2 scope is TBC

  - Intra-band non-collocated EN-DC/NR-CA deployment

    - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld

    - RAN1/2 scope is TBC

Two cross-topic proposals have been agreed for inclusion into the release-20 packaging; LB-LB switching and high-power UAV.

The LB-LB switching follows on from the Rel-19 WI, which considered switching between FDD (UL/DL) and SDL carriers. The Release 20 extension would incorporate the “dual DL” architecture, which would enable switching between FDD (UL/DL) and (SDL + FDD DL). This would mean that the FDD DL carrier would be available all of the time.

The high power UAV proposal is related to the envisaged vertical economy. To enable high power UAVs, co-existence study is needed. However, co-existence between ground networks and UAVs is different to co-existence between different ground systems. An aggressor UE flying directly above a victim BS would be at an angle with respect to the BS antenna with low-gain. However, a UAV further away from a victim BS would be closer to the horizon, for which there would be a greater amount of BS antenna gain. UAVs in the air have a line of sight propagation to basestations, which means that interference power from UAVs that are several km or 10s of km away from the BS may be received. A victim ground basestation could be impacted by accumulated power from a potentially large number of UEs that are in the order of up to 10s of km away and at the horizon.

For UAV, co-existence study is required. The co-existence study needs to take into account the special circulstances of UAV with accumulated interference over long distances.

It is also important to consider the impact of adjacent channel networks on UAVs, even though this is not directly related to HPUE operation. A UAV may have line of sight to a large number of TN basestations. For basestations directly below, the antenna gain of the basestations may be low, but more distant basestations for which the UAV is on the horizon may contribute more interference power. It may be important to establish that the accumulated power from the ground basestations does not cause degradation to the UAVs. We note that, during the ATG WI, it was found that for below 3000m, there may be losses due to selectivity towards adjacent channel basestations.

Study of co-existence degradations to UAV receivers is also necessary.

Two further cross-topic proposals remain for further consideration; fragmented carriers and intra-band non-colocated type 3 UE.

Regarding fragmented carriers, the Study Item established that there may be significant sensitivity degradation with single RX chain for FDD band combinations. It is essential that PCell is not degraded, and so any single RX operation should be restricted to SCell. The degration will not exist for TDD. As discussed during the Study Item, from a network point of view, degrading ACS is highly undesirable, and any WI should not consider ACS degradation.

Due to the extremely high workload of RAN4, any WI on fragmented carriers should be extremely simple, and there should be a very clear need to consider the WI for 5G.

Any fragmented carriers WI needs to be very simple in scope and have a very commercial motivation why it is needed in the 5G-A Rel-20 timescale,

Regarding the inter-band non-co-located, the general issue of enabling CA for non-co-located deployments has been solved with a variety of UE architectures over two Releases. During these discussions, it has also become clear that the type 3 receiver is unlikely to be implementable on either the UE or the network side. Since good solutions already exist to the issue and the optimization of type 3 would be difficult to agree, and considering the overall RAN4 workload situation it is preferable not to address this scenario further.

Do not pursue type 3 receiver for no-co-located CA, as it is not likely to be feasible and there are already other solutions.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Establish whether to focus on 29dBm PA technology and/or existing PA technology and keep the total scope for HPUE manageable.

Proposal 2	Adding 6MHz bandwidth is reasonable

Proposal 3	UE EVM reduction based on BS post-distortion compensation requires proper study and will need expertise from both UE RF and demodulation. Consider including in 6G to ensure proper study.

Proposal 4	For BS RF, consider co-location requirement improvement and the new hybrid BS type for 5G-A

Proposal 5	For BS RF, consider re-evaluation of the receiver blocking requirement

Proposal 6	Relaxed RLM and BFD for Redcap should be based on non-Redcap and existing principles for 1RX relaxation.

Proposal 7	Less than 5MHz for Redcap should be based on non-Redcap and existing principles for 1RX relaxation.

Proposal 8	Keep the scope of fast SCell activation small or zero.

Proposal 9	If there is any introduction of fast SCell activation, consider extending the Rel-19 FR2 measurement delay reduction solution for intra-band noncontiguous CA to FR1.

Proposal 10	Do not consider A-TRS based faster SCell activation.

Proposal 11	Create optional demodulation requirements based on SCM:

o	PDSCH requirement for SU-MIMO (8 layers)

o	Type II PMI reporting requirement for SU-MIMO

Proposal 12	Study further MU-MIMO SCM. Within the context of 6G, further study SCM for AI/ML.

Proposal 13	Further consider how to limit the scope of HPUE, CA and bandwidths for NTN as far as possible.

Proposal 14	For UAV, co-existence study is required. The co-existence study needs to take into account the special circulstances of UAV with accumulated interference over long distances.

Proposal 15	Study of co-existence degradations to UAV receivers is also necessary.

Proposal 16	Any fragmented carriers WI needs to be very simple in scope and have a very commercial motivation why it is needed in the 5G-A Rel-20 timescale,

Proposal 17	Do not pursue type 3 receiver for no-co-located CA, as it is not likely to be feasible and there are already other solutions.