3GPP TSG-RAN WG4 Meeting #116                                                                            R4-2511082

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:                           Discussion on the requirements for NES SSB adaptation

Agenda item:			7.23.3.2

Document for:	Discussion

1 Introduction

In last meeting, RAN4 had some discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1]. RAN1 and RAN2 agreements please find in Appendix

2 Discussion

SSB adaptation – L3 Requirements impacts

2.1 Colliding with MG

Issue 1-3-4: Colliding with MG - Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

From our understanding, after SSB adaptation, the time relationship of the SMTC and MG may be re-organized, and if the SMTC occasion of the selected additional SMTC configuration (for SSB adaptation, indicated via dynamic signaling) collides with MG occasion, the following principle inherited from the legacy can be expected:

- Principle 1 : If the additional SMTC occasions are fully overlapped with MG

- Measurement Gap sharing. The required serving SCell L3 measurement should share the measurement occasion with other measurements which need gaps. The scope of the existing CSSFwithin_gap,i definition can cover NES measurement based on SSB adaptation

- Principle 2 : If the additional SMTC occasions are partially overlapped with MG

- The required serving SCell L3 measurement are only performed on SMTC occasions outside MG.

In this sense, we have the following proposals:

Proposal 1:

- For the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the legacy principle to solve the collision can be reused:

  - If SMTC are fully overlapped with MG, gap sharing measurement, clarification on the existing definition of CSSFwithin_gap,i applies to SSB adaptation L3 measurement can be needed

  - If SMTC are partially overlapped with MG, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell, the measurements are only performed on SMTC occasions outside MG

2.2 L3 requirements for SSB adaptation – after transition for serving cell

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell - Proposals - Option 1: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC mapped with SSB periodicity. (Samsung (for activated SCell), ZTE, Oppo, CTC, CMCC, Ericsson, Xiaomi) - Option 1: UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation (Oppo) - Option 2: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication. (Samsung) - The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC - FFS: Additional UE processing/preparation time for reception of SSB after adaptation - Option 3: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements. (LGE) - Option 3a: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation. (Oppo) - Option 3b: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. (Nokia) - Recommended WF: - For activated SCell, Agree on option 1. - For deactivated SCell, discuss following options: - Option 1: legacy framework based on SMTC and meascycleScell - Option 2: reuse same framework as OD-SSB case#2.

#114-bis Issue 1-2-1: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreement - The UE follows legacy requirements based on measCycleSCell if UE has sent a valid measurement report before OD-SSB activation indication within the time window as follow. - max(5*measCycleSCell, 5*DRX cycles) for FR1, or - 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 in FR2. - Otherwise, the same approach in Case 1 can be reused. - Note: The agreement is applied when OD-SSB and AO-SSB are in the same frequency and with same spatial relation. - Note 2: T2 discussion is a separate issue.

Whether there should be any restrictions on applicable scenarios for DCI 2_9 based SSB burst periodicity adaptation for SCell is extensive discussed in RAN1, but with no conclusions yet. Majority support: No additional restriction on scenarios, while some companies insist: Applicable only for activated SCell, wherein, the key arguments are from the limitation defined in TS 38.321 5.9	Activation/Deactivation of SCells (not monitor the PDCCH on the SCell; not monitor the PDCCH for the SCell).

At the same time, if no restriction on the applicable scenario for DCI 2_9, and based on the mechanism that upon receiving the DCI 2_9, switching the SSB periodicity to one of X additional SSB burst periodicities (Xmax=2) for the Scell, we think it is feasible to perform fast deactivated SCell measurement based on adapted SSB with smaller periodicity indicated via DCI 2_9 to some extent. And considering the commonality between OD-SSB and SSB adaptation, the same framework (i.e., legacy measCycleSCell + condition & FMW, otherwise ) from OD-SSB case#2 (same frequency but with different periodicities) could be applied but with DCI indication rather than MAC-CE.

Observation 1: If  no any restrictions on applicable scenarios for DCI 2_9 based SSB burst periodicity adaptation for SCell, same framework as OD-SSB case#2 (same frequency but with different periodicities) can be reused by using adapted SSB with smaller periodicity indicated via DCI 2_9

However, considering the difference of DCI and MAC-CE triggering SSB adaptation/OD-SSB activation, the deactivated Scell measurement requirement based on OD-SSB will be modified/updated in SSB adaptation based measurement. On the other hand, SSB adaptation implies SSB transmission always exists and just SSB periodicity changes dynamically for the aim of energy saving, RRM measurement and low-latency SCell operations. In this sense, focus on DCI indication of adapted SSB periodicity occurred on activated SCell and/or SCell activation and to define the corresponding requirements are more necessary and urgent. Considering the limited TU and from requirement importance/urgency perspective, we have the following proposal

Proposal 2: Considering the limited TU and from requirement importance/ urgency perspective, for SSB adaptation based deactivated SCell measurement,

- UE follows legacy framework based on measCycleSCell.

SSB adaptation – Other requirements impacts

2.3 SCell activation requirement for SSB adaptation

Agreement we achieved in last meeting regarding Processing time for SSB adaption, below

Issue 1-1-1: Processing time for SSB adaption Agreement RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. - FR1: 2ms FR2: 3ms

Agreement we achieved in last meeting regarding SSB adaptation command, below

Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related Agreement: - For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command.

Based on RAN1 agreement, the SFN offset and HF index are configured per additional SSB periodicity value, and UE can apply the associated SFN offset and HF index to determine the time location of the first possible SSB burst after adaptation after receiving the DCI based SSB adaptation indication without blind detection, especially for the periodicity adaptation case: ShortLarge with the assumption: SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity. In this sense, SCell timing information is known to the UE prior to activation.

Observation 2: Based on RAN1 agreement, the SFN offset and HF index are configured per additional SSB periodicity value, UE can apply the associated SFN offset and HF index to determine the time location of the first possible SSB burst after adaptation after receiving the DCI based SSB adaptation indication

Proposal 3: According to the DCI 2_9 based mechanism for adaptation of SSB in time domain for SCell, the SCell timing information is known to the UE, blind detection is not needed.

The application time/first possible SSB burst determination is discussed in RAN1, value d is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. Obviously, the timeline of SSB adaptation (DCI based m+d) can not be incorporated to the one of SCell activation (MAC-CE based THARQ+3 ms) if SSB adaptation command comes at the same time as SCell activation command and, even if SSB adaptation command comes earlier than SCell activation command, there is a risk that the worst case, no immediate available 1st complete SSB burst after adaptation can to measure after SCell activation command reception, which also can cause delay for the SCell activation or UE measurement behaviour ambiguity. From this, we have the following proposal

Proposal 4: If to define SCell activation requirement for SSB adaptation when SSB adaptation command comes at the same time as SCell activation command, d and additional processing time need to be contained in TfirstSSB

Proposal 5: When to define SCell activation requirement for SSB adaptation when SSB adaptation command comes earlier than SCell activation command, it would be better to ensure the first possible SSB burst after SSB adaptation was acquired before SCell activation command reception

- FFS: Condition related to slot m+d+∆ and slot n

L1 Requirements impacts

2.4 L1 requirements – Measurement restriction impact

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei) - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CTC, Vivo) - Recommended WF: - Discuss above option.

On CSI-RS validation, the current measurement restriction defined for L1 measurement in the current spec. says, taking L1-RSRP measurement as example:

9.5.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement For FR2, when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

After SSB adaptation, it is unavoidable that the adapted SSB may be overlapped with some important CSI-RS resource, based on which, the inaccurate L1-RSRP/channel quality report issue may arise. However, the current measurement restriction give UE sufficient freedom to measure the SSB or CSI-RS (if it is valid). From this, we think the existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS.

Proposal 6: The existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS

3 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES common signal adaptation. The following observations and proposals are obtained:

Observation 1: If  no any restrictions on applicable scenarios for DCI 2_9 based SSB burst periodicity adaptation for SCell, same framework as OD-SSB case#2 (same frequency but with different periodicities) can be reused by using adapted SSB with smaller periodicity indicated via DCI 2_9

Observation 2: Based on RAN1 agreement, the SFN offset and HF index are configured per additional SSB periodicity value, UE can apply the associated SFN offset and HF index to determine the time location of the first possible SSB burst after adaptation after receiving the DCI based SSB adaptation indication

Proposal 1:

- For the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the legacy principle to solve the collision can be reused:

  - If SMTC are fully overlapped with MG, gap sharing measurement, clarification on the existing definition of CSSFwithin_gap,i applies to SSB adaptation L3 measurement can be needed

  - If SMTC are partially overlapped with MG, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell, the measurements are only performed on SMTC occasions outside MG

Proposal 2: Considering the limited TU and from requirement importance/ urgency perspective, for SSB adaptation based deactivated SCell measurement,

- UE follows legacy framework based on measCycleSCell.

Proposal 3: According to the DCI 2_9 based mechanism for adaptation of SSB in time domain for SCell, the SCell timing information is known to the UE, blind detection is not needed.

Proposal 4: If to define SCell activation requirement for SSB adaptation when SSB adaptation command comes at the same time as SCell activation command, d and additional processing time need to be contained in TfirstSSB

Proposal 5: When to define SCell activation requirement for SSB adaptation when SSB adaptation command comes earlier than SCell activation command, it would be better to ensure the first possible SSB burst after SSB adaptation was acquired before SCell activation command reception

- FFS: Condition related to slot m+d+∆ and slot n

Proposal 6: The existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS

- Appendix

4.1 RAN1 agreement update (SSB related)

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured)

Agreement

Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

Agreement

When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication,

- Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value.

  - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d.

SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

  - RAN2 agreement update

For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

5 References

[1] R4-2508441, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei