3GPP TSG-RAN WG4 Meeting #116                                                                            R4-2511089

Bengaluru, India, Aug 25th – 29th, 2025

Source:	Samsung

Title:                           Discussion on test case on SSB based inter-frequency measurement delay reduction

Agenda item:			7.14.5

Document for:	Discussion

1 Introduction

Based on WF agreed in last meeting [1]. In this meeting, we will give some high-level views on the test case design for measurement delay reduction for SSB based Inter-frequency measurement by optimizing BSF.

2 Discussion

2.1 Issue 2-1-2: baseline setup for BSF test case

Issue 2-1-2: baseline setup for BSF test case - Agreement: - For BSF optimization, for scenario 1/2/3/4, the existing test configurations in A.7.6.1 and A.7.6.2 can be used as baseline. - For BSF optimization, for scenario 5/7, the existing test configurations in A.7.3.1 and A.7.3.2 can be reused except that the test requirements need to be updated based on the reduced beam sweeping factor reported by UE.

In last meeting, we agreed to define the test case with the consideration of DRX.1 and DRX.7

Recall test setup conclusions in Rel-15 NR_newRAT-Perf WI, the following information can be found:

- For SA NR inter-frequency measurement test case:  Setup # 1 in non-DRX and short DRX. Setup # 3 for long DRX [from R4-1904783],

from our understanding, since multi-rx simultaneous reception is supported, the legacy Setup #1 and Setup #3 are not applicable anymore. Considering in Rel-18 L1 multi-Rx Setup #5, 2 AoAs for simultaneous reception with different QCL Type-D was introduced (actually in the L1 multi-Rx RLM TC, AoA setup #5 defined in A.3.15 was already considered), it is possible to consider the same test setup in our fast BSF based L3 measurement case, then we have

Proposal 1: For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

2.2 Issue 2-1-4: other detailed design for BSF test cases

Since a new mechanism (i.e., FR2 serving cell’s signal quality based triggering condition) to trigger UE using FBS for L3 measurement are introduced, rather than the existing one (event-triggered reporting with Event A3) . In this sense, the test requirements should be defined by using serving cell’s signal quality based triggering condition, i.e., serving cell’s signal quality is worse than a threshold, including RSRP, RSRQ threshold.

While, in last meeting, timer-based evaluation/re-evaluation mechanism was introduced and extensively discussed but without conclusion yet, and whether FBS can be taken as an event even taking hysteresis into consideration is not stipulated. In this sense,

Proposal 2: To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

- How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions.

3 Conclusion

In this contribution, we provided our viewpoints on delay reduction requirements for SSB based Inter-frequency measurement by optimizing BSF test case. The following observations and proposals are obtained:

Proposal 1: For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

Proposal 2: To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

- How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions.

4 References

[1] R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1