3GPP TSG-RAN WG4 Meeting #116                                          R4-2511103 Bangaluru, India, August 25 - 29, 2025

Source: 	Huawei, HiSilicon

Title: 	On power domain enhancements for single carrier

Agenda Item:	7.1.2.2.1

Document for:	Approval

Introduction

During RAN4#115, power domain enhancement for single carrier has been extensively discussed. The progress can be found in [1] and we reproduce those agreements here.

- Issue 1-1-1: Extension values/ratios - Agreement in main session: - At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases - It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides. - It is FFS on details of the single side case. - Issue 1-1-2: Guard band for extended CBW - WF - GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB - Extended CBW is not limited to the existing UE CBW - RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests - Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below: - RBStart,Low_new = max(0, RBStart,Low – RBshift_low); - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB); - RBStart,Low_new ≤ RBStart ≤ RBStart,High_new, and LCRB ≤ ceil((NRB+ RBshift_low + RBshift_high)/2); - RBshift_low, RBshift_high ≤ NRB/2 - Other approach is not precluded - The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios - Issue 1-2-1: Derivation of the extended IBE - Agreement in main session: - Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed) - The details and the related test cases can be discussed and decided later. - If significant issue is identified, other options can be further considered. - WF Further discuss whether to apply the extended IBE in the GB of extended CBW. - Option1: Yes - Option2: No - Issue 1-2-2: Relaxation of the extended IBE - Options - Option 1: using BPSK EVM in the IBE formula for all extended RBs - Option 2: no relaxation is needed. - WF - If there is no obvious system performance impact observed, option 1 is adopted - Issue 1-3-1: NW signaling - Agreement in main session: - The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities, - The details of the signaling including the contents are FFS - WF - It is up to the NW to judge the gap width on both sides of the UE - NW sends info of extension ratio (at least 1/2) per side - Issue 1-3-2: UE capability - Agreement in main session: - No separate UE capabilities for scenario 1 and scenario 2 - Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only - Both dual-sided and single sided cases are supported. - Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension - Both dual-sided symmetric and single sided cases are supported. - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced. - Note: the other ratio number depends on discussion of issue 1-1-1

In this contribution, we would like to share our views on this topic concentrating on the most controversial issues.

Discussion

- 2.1 Extended ratio

In last meeting, there was an extended discussion on the extended ratio. Although the benefit for allowing more extended ratios could be justified by operator spectrum holdings as it provides configuration flexibility accordingly, the UE implementation efforts on a number of extended ratios cannot be ignored. Other side arguments that could support the allowance of more extended ratios are 1) according to the endorsed UE capability design, one UE can indicate that it only supports 1/2 extended ratio by default capability; 2) there is no additional spec implementation effort if more extended ratios can be allowed. Thus we have the following proposal.

Proposal 1: The definition of default UE capability, which is to indicate the support of three extension ratio patterns (1/2, 0), (0, 1/2) and (1/2, 1/2), should stay remained. Other extended ratios along with dual-sided asymmetric case can be included into the separate UE capability.

- 2.2 Guard band for extended CBW

By hearing all the views from companies, the followings are essential points to this issue:

- For UE, does the OOB boundary extension have to be based on UE CBW rather than NRB?

- If the above extension will be based on UE CBW, do the extended IBE requirements have to cover all extended frequencies in order to ensure efficiently low level interference from UE?

For the first point, of course the extended OOB can be more relaxed if it is based on UE CBW. But we understand there could be some margin which can also be observed in simulation. As for the second point, we provided some analysis before to show that no requirements for the gap between extended RBs and extended CBW won’t be a problem because the characteristic of UE Tx PSD is normally roll-off rather than curl-up.

Observation 1: If the OOB boundary extension is based on UE CBW, it is not necessary to apply extended IBE within the frequencies between the edge of extended UE CBW and extended RB.

However, we can compromise to define the OOB boundary extension as NRB basis, then RAN4 doesn’t need to discuss about whether and how to define the extended IBE for the guard band for extended CBW anymore, which must be an easy way to move forward.

Proposal 2: Define the extension as NRB basis, then RAN4 can bypass the discussion on whether and how to define the extended IBE for the guard band for extended CBW.

- 2.3 UE feature list and network signalling

Regarding the feature list, we would like to propose following revisions in order to better reflect our thinking.

Proposal 3: Adopt the following revisions on UE feature 46-4 and 46-5 for Rel-19 WI NR_ENDC_RF_Ph4 objective Power boosting and/or MPR reduction.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*NRB extension for dual-sided symmetric and single-sided case. NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with X*NRB extension for dual-sided symmetric/asymmetric and single-sided case, where X = {1/2, 1/4}. NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |

It is self-evident that RRC signalling is necessary for network to indicate UE on which extension ratio pattern based on UE capability reporting. We propose to send LS to RAN2.

Observation 2: RRC signalling for extension ratio configuration needs to be designed as indicating one extension ratio out of all permutations. E.g. considering a UE that indicates the support of default UE capability, one out of (1/2, 0), (0, 1/2) and (1/2, 1/2) can be selected.

Proposal 4: Send LS to RAN2 about the RRC signalling for extension ratio configuration.

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following observations and proposals:

Observation 1: If the OOB boundary extension is based on UE CBW, it is not necessary to apply extended IBE within the frequencies between the edge of extended UE CBW and extended RB.

Observation 2: RRC signalling for extension ratio configuration needs to be designed as indicating one extension ratio out of all permutations. E.g. considering a UE that indicates the support of default UE capability, one out of (1/2, 0), (0, 1/2) and (1/2, 1/2) can be selected.

Proposal 1: The definition of default UE capability, which is to indicate the support of three extension ratio patterns (1/2, 0), (0, 1/2) and (1/2, 1/2), should stay remained. Other extended ratios along with dual-sided asymmetric case can be included into the separate UE capability.

Proposal 2: Define the extension as NRB basis, then RAN4 can bypass the discussion on whether and how to define the extended IBE for the guard band for extended CBW.

Proposal 3: Adopt the following revisions on UE feature 46-4 and 46-5 for Rel-19 WI NR_ENDC_RF_Ph4 objective Power boosting and/or MPR reduction.

Proposal 4: Send LS to RAN2 about the RRC signalling for extension ratio configuration.

- References

- R4-2507930, “WF on NR_ENDC_RF_Ph4_part1”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #115, Malta, MT, May 19-23, 2025.