3GPP TSG-RAN WG4 Meeting #116	R4-2511104

Bengaluru, India, August 25th – 29th, 2025

Title:	LS on RRC signalling for power domain enhancement

Response to:	-

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Core

Source:	RAN4

To:	RAN2

Cc:	-

Contact person:	Xiang GAO

gaoxiang74@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

For Rel-19 MPR reduction for single carrier, RAN4 has agreed on following UE feature design to indicate the supported extension ratio for both lower and higher side abutted UE assigned channel.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*NRB extension for dual-sided symmetric and single-sided case. NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
| 46-5 | MPR reduction for single carrier with multiple values | Indicates whether UE supports MPR reduction for single UL carrier with X*NRB extension for dual-sided symmetric/asymmetric and single-sided case, where X is 1/2 and 1/4. NOTE 1: NRB is the number of RBs defined per channel bandwidth by RAN4 in TS 38.101-1, Table 5.3.2-1 for FR1. |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per band | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |

Specifically, if 46-4 is indicated by UE, it means that the UE supports 1/2 extension ration including 3 permutations in total, which are (1/2, 0), (0, 1/2) and (1/2, 1/2). If 46-5 is indicated, the UE supports both 1/4 and 1/2 extension ratio then 8 permutations can be configured, which are (1/4, 0), (0, 1/4), (1/4, 1/4), (1/2, 0), (0, 1/2), (1/2, 1/2), (1/4, 1/2) and (1/2, 1/4).

Thus, it is up to RAN2 to design the signaling which should allow the network configuration for a certain permutation consisting of the extension ratio for lower and higher side abutted UE assigned channel.

2	Actions

To RAN2

ACTION:  RAN4 respectfully asks RAN2 to implement the RRC signaling for the above mentioned extension ratio configuration.

3	Dates of next TSG RAN WG 4 meetings

TSG-RAN WG4 Meeting #116bis		13 – 17 October 2025				Prague, CZ

TSG-RAN WG4 Meeting #117		17 – 21 November 2025				Dallas, US