3GPP TSG-RAN WG4 Meeting #116                                           R4-2511123

# Bengaluru, India, 25th – 29th of August 2025

Title: 	Proposals for LS Reply to ETSI on Nominated Bandwidth concept under Harmonised Standard for NTN capable UE

Agenda Item:	7.28.2 – General Aspects

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# Information

ETSI TC SES has started the development of new harmonized standards for 3GPP-based IoT NTN and NR NTN capable UEs operating in respectively 256/255 and n256/n255 bands. The harmonised standards will be referenced respectively “ETSI EN 304 121” and “ETSI EN 304 122”. ETSI TC SES provided information to 3GPP RAN4 in SES(23)000035r1/R4-2317932 and SES(24)000037/R4-2411017.

Further, in the LS R4-2509023 (“LS on Harmonised Standard for NTN capable UE”), ETSI describes:

- The “nominated bandwidth concept” used for the purpose of defining the Spectral Emission Mask requirement and defined in the relevant existing ETSI harmonised standards (e.g. EN 301 442 and EN 301 444), will be further discussed in ETSI TC SES. ETSI TC SES would welcome feedback from 3GPP on the usefulness of the concept before ETSI TS SESSCN#53e meeting where conclusion on this topic is expected.

- It is expected that same approach will also apply to bands n253/253, n251 and n250. This means that in addition to 3GPP requirements, only out-of-band emissions into 1610-1626.5 MHz will be considered for these bands when defined.

- ETSI TC SES would remain at the disposal of 3GPP RAN4 for any additional information.

This document will further provide a few proposals for the LS Reply to ETSI and challenges that were identified by 3GPP RAN4 Work Group.

# General observations on Nominated Bandwidth concept

General NOTE: the following discussions shall be considered as general to all NTN NB-IoT bands. 249 has been taken as example, but similar issues currently exists also for other NTN (at least NB-IoT related) bands definitions.

For instance, during RAN4#114-bis it was decided to use at least the following references: ETSI EN 301 441, ITU-R M.1343-1, other references not precluded.

3GPP TS 36.102 indicates the following NB1 and NB2 spectral emission mask (where ΔfOOB is defined with respect to the edge of the NB-IoT Channel BandWidth):

Figure 2: NB1/NB2 spectrum Emission Mask (36.102)

Following RAN4#114-bis, we have the following observations and proposals for RAN4#115:

Observation 1: ETSI EN 301 441 unwanted emission requirements are using a “nominated bandwidth” concept, which is different from 3GPP channel bandwidth concept:

Figure 3: Maximum unwanted emissions within the band (ETSI EN 301 441)

Observation 2: As a matter of fact, following 3GPP RAN4 understanding, ETSI EN 301 441 permits UE device manufacturers to declare a nominated bandwidth with some restrictions on the maximum nominated bandwidth that can be declared as below from Clause 3.1 ETSI EN 301 441. Basically, this ETSI EN 301 441 nominated bandwidth its one to five channel bandwidths as long as the nominated bandwidth is less than the bandwidth of the licensed spectrum, as per information below:

With respect to 3GPP RAN4 understanding, the rationale behind ETSI’s nominated bandwidth is related to a few reasons:

- Declaring a nominated bandwidth permits ETSI to know and control the maximum emissions that a UE will spill into adjacent licensed spectrum.

- A UE should not operate closer than its declared nominated bandwidth to its licensed band edges.

If one would represent all the previous information in one single figure, it would obtain the following limits:

Figure 1: SEM limitation with respect to different Nominated Bandwidth configurations

Observation 3: However, 3GPP identified the following challenges in using nominated BW concept from ETSI:

1/ It seems that the definition/usage is slightly different across different ETSI EN specifications;

2/ According to the ETSI specifications and nominated bandwidth concept, the manufacturer can declare/specify the a and b values that will meet the requirement, which will translate in a very flexible approach of defining the requirements. If different manufactures declare different a & b values, it would be difficult to have a common conformance testing for all users;

3/ a and b parameters should be therefore considered as fixed values per each band definition, and therefore the nominated bandwidth value should be considered a fixed value per band.

Observation 4: On the other hand, if the nominated bandwidth ETSI concept is considered by 3GPP RAN4 equal to the Channel Bandwidth (CBW of 200 kHz for NB-IoT technology), this will result in Spectrum Emission Mask (SEM) requirements very difficult to meet by manufacturers, which translates into an important UE power reduction. And for instance, for the NTN satellite technology, is essential that UEs are allowed to transmit at maximum power defined by the 3GPP standards. This can be further seen in the figure below:

Figure 2: Example of NB-IoT transmission with 1 tone allocation at the edge of the channel (worst case)

NOTE: In the figure above, NB-IoT CBW = 200 kHz and Nominated Bandwidth = 200 kHz CBW (even if according to our understanding for NB-IoT signals it can be up to 5 times CBW).

With a zoom on Figure 2:

Figure 3: Zoom on example of NB-IoT transmission with 1 tone allocation at the edge of the channel (worst case)

NOTE: In the figure above, NB-IoT CBW = 200 kHz and Nominated Bandwidth = 200 kHz CBW (even if according to our understanding for NB-IoT signals it can be up to 5 times CBW)

Where the NB-IoT CBW is defined as follows in 3GPP NTN NB-IoT UE TS 36.102:

And the 3GPP NTN NB-IoT UE TS 36.102 Spectrum Emission Mask (SEM) is defined as follows:

Proposals for the LS reply (a combination of different proposals is not precluded):

Proposal 1: For the time being ETSI nominated bandwidth concept was not introduced by 3GPP RAN4 in TS 36.102 for NB-IoT NTN.

Proposal 2: RAN4 shall consider the utility to integrate the nominated bandwidth concept as part of all NTN bands (e.g. as part of Rel-20 or TEI under Rel-19) – not only 256 and 255 but also to e.g. 254, 253, 251, 250, 249.

Proposal 3: RAN4 kindly asks ETSI to provide the exact definition of the nominated bandwidth since this is an ETSI concept (and not part of ITU, 3GPP or other standardization organisms).

Proposal 4: RAN4 kindly asks ETSI to indicate if nominated bandwidth concept is different in several ETSI EN specifications (at least EN 301 442 and EN 301 444, ETSI EN 301 441) and how it can be used.

Proposal 5: RAN4 would like to use a fixed nominated value per band. This may include using one or two nominated values. However, the current ETSI definition for nominated bandwidth concept seems too broad and cannot be used directly as it is, since RAN4 has to define UE power reduction mechanisms applicable for a given channel bandwidth/nominated bandwidth value.

# Conclusion

Observation 1: ETSI EN 301 441 unwanted emission requirements are using a “nominated bandwidth” concept, which is different from 3GPP channel bandwidth concept:

Figure 3: Maximum unwanted emissions within the band (ETSI EN 301 441)

Observation 2: As a matter of fact, following 3GPP RAN4 understanding, ETSI EN 301 441 permits UE device manufacturers to declare a nominated bandwidth with some restrictions on the maximum nominated bandwidth that can be declared as below from Clause 3.1 ETSI EN 301 441. Basically, this ETSI EN 301 441 nominated bandwidth its one to five channel bandwidths as long as the nominated bandwidth is less than the bandwidth of the licensed spectrum, as per information below:

Observation 3: However, 3GPP identified the following challenges in using nominated BW concept from ETSI:

1/ It seems that the definition/usage is slightly different across different ETSI EN specifications;

2/ According to the ETSI specifications and nominated bandwidth concept, the manufacturer can declare/specify the a and b values that will meet the requirement, which will translate in a very flexible approach of defining the requirements. If different manufactures declare different a & b values, it would be difficult to have a common conformance testing for all users;

3/ a and b parameters should be therefore considered as fixed values per each band definition, and therefore the nominated bandwidth value should be considered a fixed value per band.

Observation 4: On the other hand, if the nominated bandwidth ETSI concept is considered by 3GPP RAN4 equal to the Channel Bandwidth (CBW of 200 kHz for NB-IoT technology), this will result in Spectrum Emission Mask (SEM) requirements very difficult to meet by manufacturers, which translates into an important UE power reduction. And for instance, for the NTN satellite technology, is essential that UEs are allowed to transmit at maximum power defined by the 3GPP standards. This can be further seen in the figure below:

Figure: Example of NB-IoT transmission with 1 tone allocation at the edge of the channel (worst case)

NOTE: In the figure above, NB-IoT CBW = 200 kHz and Nominated Bandwidth = 200 kHz CBW (even if according to our understanding for NB-IoT signals it can be up to 5 times CBW).

Proposals for the LS reply (a combination of different proposals is not precluded):

Proposal 1: For the time being ETSI nominated bandwidth concept was not introduced by 3GPP RAN4 in TS 36.102 for NB-IoT NTN.

Proposal 2: RAN4 shall consider the utility to integrate the nominated bandwidth concept as part of all NTN bands (e.g. as part of Rel-20 or TEI under Rel-19) – not only 256 and 255 but also to e.g. 254, 253, 251, 250, 249.

Proposal 3: RAN4 kindly asks ETSI to provide the exact definition of the nominated bandwidth since this is an ETSI concept (and not part of ITU, 3GPP or other standardization organisms).

Proposal 4: RAN4 kindly asks ETSI to indicate if nominated bandwidth concept is different in several ETSI EN specifications (at least EN 301 442 and EN 301 444, ETSI EN 301 441) and how it can be used.

Proposal 5: RAN4 would like to use a fixed nominated value per band. This may include using one or two nominated values. However, the current ETSI definition for nominated bandwidth concept seems too broad and cannot be used directly as it is, since RAN4 has to define UE power reduction mechanisms applicable for a given channel bandwidth/nominated bandwidth value.