3GPP TSG-RAN WG4 Meeting#116                                R4-2511125

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussions on General aspect for A-IoT

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Topic 2-1: R2D bandwidth Issue 2-1-1: R2D transmission bandwidth Agreement in RAN4#115: - Define symmetric guard band as 10kHz, 20kHz, 30kHz and 40kHz for 1PRB, 2PRB, 3PRB and 4PRB channel bandwidth configuration - Topic 2-2: D2R bandwidth Issue 2-2-1: D2R bandwidth Agreement in RAN4#115: - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1. - Assumed 10% SFO for device - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣) - The following are for information: - Figure 1-1 Small frequency shift diagram of FDMA without SFO - Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value . - The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows: - Note : BW and Small frequency shift are in kHz;Tc and Tb are in μs 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).. - Topic 2-3: Channel raster Issue 2-3-1: R2D channel raster Agreement in RAN4#115: - Define 10KHz channel raster - FFS on channel raster offset

Discussion

- 2.1 General aspects for A-IoT BS in D1T1

Issue 2-3: R2D channel raster

- Regarding the channel raster for A-IoT service, if the standalone spectrum is allocated for A-IoT R2D transmission, then channel raster for R2D transmission should be at least 100KHz. If the in-band spectrum is allocated for A-IoT R2D transmission, then channel raster could be defined as following:

- N*100kHz channel raster+delta_offset

- where offset could be -47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5KHz

Figure 2. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- During the last RAN4 meeting, it was agreed to define channel raster as 10KHz, then the corresponding freq offset compared with 10KHz channel raster should be updated as following:

- Proposal 1: for R2D transmission in Standalone operation mode, to define the channel raster as 10KHz; for R2D transmission in in-band operation mode, to define the channel raster as 10KHz*N+delta_offset where delta_offset could be as [ -7.5, 2.5]kHz.

Issue 2-1-6: D2R bandwidth

- In the last RAN4 meeting, it was agreed to use the following equation for BS D2R bandwidth as starting point:

- Using following equation for BS as starting point: D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz], [3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS

Figure 3 Relationship with D2R transmission bandwidth, D2R chip duration, SFS factor R

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Freq. shift to CW =1/2Tchip | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 45 kHz | 60 kHz | 90 kHz | 120 kHz | 180 kHz | 240 kHz | 360 kHz | 480 kHz | 720 kHz | 960 kHz |
| Btx,D2R =4/Tb | Tb (μs) | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 15 kHz | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  |  |
| 30 kHz | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 60 kHz | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 120 kHz | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 180 kHz | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 240 kHz | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 360 kHz | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 480 kHz | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 720 kHz | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 960 kHz | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 1440 kHz | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 1920 kHz | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 2880 kHz | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 3840 kHz | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

- Based on the latest RAN1 agreement for DSB and R_block as shown in the following, the D2R bandwidth for BS with SFO and GB considered is summarized in the following table. If all the potential D2R bandwidth are to be specified in the spec, then it will cause big implementation burden for Reader and lose the compatibility for different device. From our understanding, the limited implementation D2R bandwidth should be considered in the RAN4. Considering this kind of design principle also consider the maximum repetition and DSR bandwidth as defined in RAN1, the D2R carrier bandwidth could be categorized into different 5 classes as indicated in the different color in the following table. If we only consider the D2R bandwidth as 200kHz and 3.52MHz, then only the following case as highlighted in yellow would be specified in the spec which might cause the misalignment between RAN1 and RAN4 and also leave no freedom for other combination of DSR and Repetition.

| - DSB [KHz] | - Tb [us] | - D2R bandwidth SFO, GB [KHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 | - R=64 | - R=128 |  |
| - 15.0 | - 266.67 | - 18.3 | - 27.5 | - 45.8 | - 82.5 | - 155.8 | - 302.5 | - 595.8 | - 1182.5 |  |
|  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 | - R=64 |  |
| - 30.0 | - 133.33 |  | - 36.7 | - 55.0 | - 91.7 | - 165.0 | - 311.7 | - 605.0 | - 1191.7 |  |
|  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 |  |
| - 60.0 | - 66.67 |  |  | - 73.3 | - 110.0 | - 183.3 | - 330.0 | - 623.3 | - 1210.0 |  |
|  |  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 |  |
| - 120.0 | - 33.33 |  |  |  | - 146.7 | - 220.0 | - 366.7 | - 660.0 | - 1246.7 |  |
|  |  |  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 |  |
| - 240.0 | - 16.67 |  |  |  |  | - 293.3 | - 440.0 | - 733.3 | - 1320.0 |  |
|  |  |  |  |  |  |  | - R=1 | - R=2 | - R=4 |  |
| - 480.0 | - 8.33 |  |  |  |  |  | - 586.7 | - 880.0 | - 1466.7 |  |
|  |  |  |  |  |  |  |  | - R=1 | - R=2 |  |
| - 960.0 | - 4.17 |  |  |  |  |  |  | - 1173.3 | - 1760.0 |  |
|  |  |  |  |  |  |  |  |  |  | - R=1 |
| - 2880.0 | - 1.39 |  |  |  |  |  |  |  |  | - 3520 |

- Proposal 2: for D2R bandwidth for BS, propose to define the full set of D2R bandwidth in Rel-19.

- For the D2R bandwidth from device perspective, considering the small freq shifting capability from device perspective, we propose to follow the agreement reached in the last RAN4 meeting to calculate the D2R bandwidth.

- Proposal 3: for D2R bandwidth for device, propose to define the full set of bandwidth as following:

| - DSB - [KHz] | - Tb [us] | - D2R bandwidth without SFO, GB [KHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| - 15 | - 266.67 | - 15 | - 22.5 | - 37.5 | - 67.5 | - 127.5 | - 247.5 | - 487.5 | - 967.5 |  |
|  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| - 30 | - 133.33 |  | - 30 | - 45 | - 75 | - 135 | - 255 | - 495 | - 975 |  |
|  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| - 60 | - 66.67 |  |  | - 60 | - 90 | - 150 | - 270 | - 510 | - 990 |  |
|  |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| - 120 | - 33.33 |  |  |  | - 120 | - 180 | - 300 | - 540 | - 1020 |  |
|  |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| - 240 | - 16.67 |  |  |  |  | - 240 | - 360 | - 600 | - 1080 |  |
|  |  |  |  |  |  |  | - R=1 | - R=2 | - R=4 |  |
| - 480 | - 8.33 |  |  |  |  |  | - 480 | - 720 | - 1200 |  |
|  |  |  |  |  |  |  |  | - R=1 | - R=2 |  |
| - 960 | - 4.17 |  |  |  |  |  |  | - 960 | - 1440 |  |
|  |  |  |  |  |  |  |  |  |  | - R=1 |
| - 2880 | - 1.39 |  |  |  |  |  |  |  |  | - 2880 |

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

- Proposal 1: for R2D transmission in Standalone operation mode, to define the channel raster as 10KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [ -7.5, 0, 2.5]kHz.

- Proposal 2: for D2R bandwidth for BS, propose to define the full set of D2R bandwidth in Rel-19.

| - DSB [KHz] | - Tb [us] | - D2R bandwidth SFO, GB [KHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 | - R=64 | - R=128 |  |
| - 15.0 | - 266.67 | - 18.3 | - 27.5 | - 45.8 | - 82.5 | - 155.8 | - 302.5 | - 595.8 | - 1182.5 |  |
|  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 | - R=64 |  |
| - 30.0 | - 133.33 |  | - 36.7 | - 55.0 | - 91.7 | - 165.0 | - 311.7 | - 605.0 | - 1191.7 |  |
|  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 | - R=32 |  |
| - 60.0 | - 66.67 |  |  | - 73.3 | - 110.0 | - 183.3 | - 330.0 | - 623.3 | - 1210.0 |  |
|  |  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 | - R=16 |  |
| - 120.0 | - 33.33 |  |  |  | - 146.7 | - 220.0 | - 366.7 | - 660.0 | - 1246.7 |  |
|  |  |  |  |  |  | - R=1 | - R=2 | - R=4 | - R=8 |  |
| - 240.0 | - 16.67 |  |  |  |  | - 293.3 | - 440.0 | - 733.3 | - 1320.0 |  |
|  |  |  |  |  |  |  | - R=1 | - R=2 | - R=4 |  |
| - 480.0 | - 8.33 |  |  |  |  |  | - 586.7 | - 880.0 | - 1466.7 |  |
|  |  |  |  |  |  |  |  | - R=1 | - R=2 |  |
| - 960.0 | - 4.17 |  |  |  |  |  |  | - 1173.3 | - 1760.0 |  |
|  |  |  |  |  |  |  |  |  |  | - R=1 |
| - 2880.0 | - 1.39 |  |  |  |  |  |  |  |  | - 3520 |

- Proposal 3: for D2R bandwidth for device, propose to define the full set of bandwidth to accommodate all the device capability as following:

| - DSB - [KHz] | - Tb [us] | - D2R bandwidth without SFO, GB [KHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 | R=128 |  |
| - 15 | - 266.67 | - 15 | - 22.5 | - 37.5 | - 67.5 | - 127.5 | - 247.5 | - 487.5 | - 967.5 |  |
|  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 | R=64 |  |
| - 30 | - 133.33 |  | - 30 | - 45 | - 75 | - 135 | - 255 | - 495 | - 975 |  |
|  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 | R=32 |  |
| - 60 | - 66.67 |  |  | - 60 | - 90 | - 150 | - 270 | - 510 | - 990 |  |
|  |  |  |  |  | R=1 | R=2 | R=4 | R=8 | R=16 |  |
| - 120 | - 33.33 |  |  |  | - 120 | - 180 | - 300 | - 540 | - 1020 |  |
|  |  |  |  |  |  | R=1 | R=2 | R=4 | R=8 |  |
| - 240 | - 16.67 |  |  |  |  | - 240 | - 360 | - 600 | - 1080 |  |
|  |  |  |  |  |  |  | - R=1 | - R=2 | - R=4 |  |
| - 480 | - 8.33 |  |  |  |  |  | - 480 | - 720 | - 1200 |  |
|  |  |  |  |  |  |  |  | - R=1 | - R=2 |  |
| - 960 | - 4.17 |  |  |  |  |  |  | - 960 | - 1440 |  |
|  |  |  |  |  |  |  |  |  |  | - R=1 |
| - 2880 | - 1.39 |  |  |  |  |  |  |  |  | - 2880 |

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2505230, WF on A-IoT device requirements, CMCC, approved.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.