3GPP TSG-RAN WG4 Meeting#116                                R4-2511127

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	TP to TS38.194: REFSENS requirement for A-IoT BS and FRC

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to provide TP for REFSENS requirement for A-IoT BS and also Annex for FRC for REFSENS requirement definition.

References

- RP-250796, Core part: Solutions for Ambient IoT (Internet of Things) in NR, Approved.

- TS 38.194 NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception

TP

7	Conducted receiver characteristics

7.1	General

Conducted receiver characteristics are specified at the antenna connector for BS type 1-C , with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements apply during the BS receive period.

-	Reference requirements defined for the conducted receiver characteristics do not assume HARQ retransmissions.

NOTE 1:	In normal operating condition, A-IoT BS is configured as HD-FDD operation.

7.2	Reference sensitivity level

7.2.1	General

The reference sensitivity power level PREFSENS is the minimum mean power received at the antenna connector for BS type 1-C at which a throughput requirement shall be met for a specified reference measurement channel.

7.2.2	Minimum requirements for BS type 1-C

The BLER shall be less than or equal to 10% of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.2-1 for A-IoT Medium range BS.

Table 7.2.2-1: A-IoT Medium range BS reference sensitivity levels

| BS channel bandwidth (KHz) | DSB (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | A-FR1-A1-1 [BPSK] | TBD |
|  |  | A-FR1-A1-2 [OOK] | TBD |
| 200 | TBD | A-FR1-A1-1 [BPSK] | TBD |
|  |  | A-FR1-A1-2 [OOK] | TBD |
| NOTE 1: Reference sensitivity power level is defined based on the CW power at the antenna connector as TBD power. |  |  |  |

[7.3	Dynamic range

7.3.1	General

[The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal at the antenna connector for BS type 1-C inside the received BS channel bandwidth. In this condition, a BLER requirement shall be met for a specified reference measurement channel. The interfering signal for the dynamic range requirement is an AWGN signal.]

7.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The BLER shall be less than or equal to 10% of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.3.2-1 for A-IoT Medium Range BS.

Table 7.3.2-1: A-IoT Medium range BS dynamic range

| BS channel bandwidth (KHz) | DSB (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 200 | 15 | G-FR1-A2-1 | TBD | TBD | AWGN |
|  | 15 | G-FR1-A2-2 | TBD |  |  |
| NOTE 1: Reference sensitivity power level is defined based on the CW power at the antenna connector as TBD power. |  |  |  |  |  |

- ]

Annex A (normative):
Reference measurement channels

A.1	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking, (BPSK, OOK)

| Reference channel | A-FR1-A1-1 | A-FR1-A1-2 |  |  |
| --- | --- | --- | --- | --- |
| DSB (kHz) | 15 | 15 |  |  |
| Payload size (bits) | 96 | 96 |  |  |
| CRC (bits) | 16 | 16 |  |  |
| Preamble length | 31 | 31 |  |  |
| Midamble length | 31 | 31 |  |  |
| Midamble configuration I | 48 | 48 |  |  |
| FEC | 1/3 | 1/3 |  |  |
| Line code | Manchester | Manchester |  |  |
| Modulation | BPSK | OOK |  |  |
| Waveform (CW) | unmodulated single tone | unmodulated single tone |  |  |
| Sampling frequency（SFO） | between 0.01 and 0.1 | between0.01 and 0.1 |  |  |
| Total symbols | 398 | 398 |  |  |