3GPP TSG-RAN WG4 Meeting#116                                R4-2511128

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement of Ambient IoT device

Document for:	Approval

Introduction

- In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

Device RF requirements - Topic 3-1: Transmit output power Issue 3-1-1: Transmit output power Agreement in RAN4#115: - Performance metric: EIRP - FFS on how to determine device’s peak antenna gain direction - FFS on whether to measure multiple directions - Take [10] dB backscatter loss for OOK as starting point, based on the assumption that the incoming CW signal and the backscatter signal are aligned with the peak gain direction of the device antenna. - FFS for BPSK - The power of the backscattered signal shall include only the 1st lower sideband and the 1st upper sideband, excluding the carrier itself. - Topic 3-2: Modulation quality Issue 3-2-1: SFO requirement Agreement in RAN4#115: - No need to directly test SFO if SFO can be indirectly verified via other test. - FFS whether to explicitly define SFO requirements or not. Issue 3-2-2: Modulation quality requirements Agreement in RAN4#115: - The modulation quality won’t be introduced, and it can be verified by the backscattering loss for OOK or BPSK. - Topic 3-3: Emission requirements Issue 3-3-1: SEM requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - The applicable frequency range for SEM: from the CBW edge to [OOB boundary] - Define flat requirements for SEM as starting point, [9dBc~14dBc] based on achievable harmonic performance - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. Issue 3-3-1: spurious emission requirements Agreement in RAN4#115: - Use EIRP as performance metric - Device’s peak antenna gain direction is based on declaration, i.e. same peak gain direction for backscattering loss testing - OOB boundary - Option 1: D2R channel bandwidth plus minimum D2R channel bandwidth - Option 2: max of 500kHz and 10 times NB where NB equals to D2R CBW - Other options are not precluded - Define requirements applicable to all CBWs, FFS on how to reduce the test burden. - Use the [maximum] CW input power level, FFS on the value. - Topic 3-4: Reference sensitivity Issue 3-4-1: Reference sensitivity Agreement in RAN4#115: - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Testing metric: 10% miss detection rate, FFS on whether to consider false alarm - FFS for the FRC, consider following as starting point: SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. TBS; 20; 20; 20; 20. CRC; 6; 6; 6; 6. M; 6; 12; 24; 24. - Topic 3-5: Others Issue 3-5-1: Maximum input power Agreement in RAN4#115: - Define maximum input level at peak antenna direction, using [-4dBm] as starting point. Issue 3-5-2: Occupied bandwidth Agreement in RAN4#115: - Do not define occupied bandwidth requirements.

Discussion

- 2.1. RF requirement for Ambient IoT device

- 2.1 .1Backscattering loss

- In device 1, the backscattering involves changing the impedance of chipset port by digital bit stream, thereby altering the reflection coefficient at the antenna-chip port. From the modulation perspective, it can be observed that the reflection loss is mainly dependent on the following two factors:

- Loss caused by the antenna itself, such as radiation efficiency, antenna directivity, etc.

- the matching condition between the antenna and the chipset port, i.e., the reflection coefficient.

- In the previous RAN4 meeting, it was agreed that backscattering loss is defined at the peak antenna direction. In the following section, we mainly analyze the matching condition between antenna port and chipset port.

- The reflection coefficient is defined as the ratio of the reflected voltage to the incident voltage. It could be calculated as following:

Γ=Zchip−Zant∗Zchip+Zant

Expressed in dB unit:

S11=10logΓ

- The reflection loss is defined as the ratio of the incident power to the reflected power. It could be calculated as following:

Rl=10logPinPout=10log1Γ2

If the device 1 uses Manchester encoding with OOK, where the data bits "0" and "1" each occupy 50% of the time, the reflection loss could be calculated as following under the ideal impedance matching condition:

Rl−OOK=10log11∗Γ12+0∗Γ022=3.01 dB

If the device 1 uses Manchester encoding with BPSK, where the data bits "-1" and "1" each occupy 50% of the time, the reflection loss could be calculated as following under the ideal impedance matching condition:

Rl−BPSK=10log11∗Γ12+1∗Γ022=0 dB

- Observation 1: for the ideal OOK signal, the backscattering loss is expected as 6dB by removing DC power;

- Observation 2: : for the ideal BPSK signal, the backscattering loss is expected as 0dB;

- Considering the practical implementation for the device, when the modulation data is bit 1, due to the modulation circuit pulling the antenna port down to a short-circuit state, the bandwidth of the dipole antenna is only a few tens of megahertz, resulting in a reflection coefficient above 0.9. When modulating the data is bit 0, the antenna impedance changes in both its real and imaginary parts due t2020o non-ideal factors. Under normal conditions, the resonant frequency deviation is around 50 MHz. Additionally, simulations of the antenna as shown in the following table show that the real part of the impedance varies approximately between 6 and 10. For the modulation data '0' state, if the system achieves perfect matching, while in the data '1' state, a slight mismatch results in the maximum reflection loss, approximately 4 dB. However, when non-ideal effects cause a mismatch in the data '0' state, significant power reflection occurs. Combined with the full reflection in the data '1' state, the overall reflection loss may drop below 2 dB.

| - Antenna Impedance | - Resonant Frequency | - S11_0 | - Power Reflection Coefficient | - Total Reflection Loss |
| --- | --- | --- | --- | --- |
| 8+j*200 | 920 | 0 | 0 | 4 |
| 8+j*220 | 877 | 0.78 | 0.61 | 1.5 |
| 6+j*220 | 877 | 0.82 | 0.67 | 1.31 |
| 10+j*220 | 877 | 0.75 | 0.56 | 1.64 |
| 8+j*180 | 969 | 0.78 | 0.61 | 1.5 |
| 6+j*180 | 969 | 0.82 | 0.67 | 1.31 |
| 10+j*180 | 969 | 0.75 | 0.56 | 1.64 |

- Proposal 1: for the practical OOK backscattering loss, propose to define the minimum requirement 8dB (ideal 6+2dB margin) without considering antenna efficiency.

- For the BPSK modulation, it is expected to be 6dB better than OOK backscattering loss in theory, therefore we propose to consider 2dB backscattering loss for BPSK.

- Proposal 2: for the practical BPSK backscattering loss, propose to define the minimum requirement 2dB without considering antenna efficiency.

- 2.1 .2 REFSENS

- In the previous RAN4 meetings, there were lots of discussions on REFSENS requirement for A-IoT device, however there were no consensus reached, therefore in this contribution, we would like to share some further considerations.

- First of all, according to the power consumption as captured in the WID, ~1 µW peak power consumption of passive device 1 is considered in the WID which needs at least around -30dBm input signal at the antenna connector for the RFID device 1 if instant demod reception considered which means device 1 need to utilize the ON power of OOK for charging/decoding and follow up the basic electricity for OFF duration of OOK reception.

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- In the following, some illustration of figures are provided for the better understanding the effective circuit between antenna and chipset of device 1.

- Figure 1. the illustration of passive device 1

- Figure 2. the illustration of the effective circuit between antenna and device 1 chipset (rectifying and other digital processing)

- Since the CMOS manufacturing implementation is considered in the device 1 chipset, then the minimum voltage for the device 1 chipset is around 0.15v to 0.2v considering the impact of the bandgap of silicon material and the substrate doping concentration on the threshold voltage of the device. In addition, considering the optimal antenna design for device 1 chipset with antenna impedance as 3 (resistance)+j*300 (reactance) for charging with Quality value as 300/3, the load impedance as 3 (resistance)-j*300(reactance), the sensitivity requirement could be calculated as following.

- Sens=Vant22(1+Q2)Rs

- Where:

- Vant is 0.15v as minimum voltage to support the CMOS operation;

- Q is 100 which is usually designed for narrow bandwidth operation; the supporting bandwidth as 9MHz.      Resistance Rs is 3 ohm

- Therefore the ideal sensitivity requirement for passive device 1 is around -34dBm without considering antenna gain of dipole antenna.

- However for band n8, the device 1 need to support the whole operating frequency range from 880-960MHz, the minimum value should be 920/80. In addition, considering the similar performance as RFID tag 6C, then Q value should be around 15-20 with Rs as 10 ohm which is much worse than the optimal performance as illustrated above, then the conducted sensitivity requirement could be calculated as -25.5dBm.

- In addition, considering the initial simulation results for Linearly Polarized Folded Dipole for device 1, 1.62dB antenna gain could be achieved in the peak direction, then the OTA REFSENS requirement for device 1 should be -27.1dBm which is equal to -25.5dBm-1.62dB.

- In addition, regarding the 10% miss detection metric, this is usually assumed for 3GPP system with some LLS or baseband digital processing, however for device 1, the implementation is much different compared with other advanced UE or BS receiver. Considering the simple digital processing in the chipset and also the experienced inventory success ration is higher than 99% in the most case, therefore we would like to make the following proposal.

- Observation 3: propose to define the charging REFSENS requirement at the peak direction as -27dBm.

- Figure 3. the illustration of the timeline for charging and decoding

- Based on the previous discussions, RAN4 need to define the decoding requirement based on the charging capability to ensure the chipset working. As shown in the above figure, around 2ms CW signal for charging before R2D device decoding is necessary, in the following time, device 1 could still utilize the ON power for charging to maintain the chipset working during the OFF period under the manchester coding.

- Proposal 3: propose to define the -34dBm with 99% inventory success rate as the decoding REFSENS requirement under the condition of device charged to enable the work.

- In addition, in the last RAN4 meeting, there were some discussions on the necessity of the spatial requirement for device 1, from our understanding, considering the use case of device 1 e.g. attaching on different equipment for the inventory purpose, the spatial coverage requirement is still needed considering its potential rotation and movement of the equipment.

- Considering the following antenna pattern for device 1 and also considering the potential 3dB loss of compared with peak direction, angular 3dB beamwidth is 87.7 degree for one side, therefore the spatial EIS requirement could be defined as -27dBm+3dB for 48.7% (87.7*2/360) spherical coverage of the device.

- Figure 3. the illustration of the 3D spatial antenna pattern of device 1

- Figure 4. the illustration of the polar antenna pattern of device 1

- Proposal 4: propose to define the spherical coverage requirement at the 48% coverage as -31dBm with 99% inventory success rate.

- 2.1 .3 FRC

- In addition, the FRC for device 1 is not agreed yet. According to RAN1 TS 38.291, we propose to define the FRC for REFSENS requirement as following.

|  | SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- | --- |
|  | PRB | 1 | 2 | 3 | 4 |
| SIP | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | OOK |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
| chip number except for SIP,padding |  | 228 | 228 | 228 | 228 |
| Padding |  | 6 | 12 | 2 | 2 |

- In addition, one important issue is to define the FRC for R2D detection, based on some RAN1 inputs, we proposed to have the following FRC for R2D for further discussion.

- Proposal 5: propose to consider the following R2D FRC as starting point for further discussion.

Conclusions

In this contribution, we shared some further views on the RF requirement for Ambient IoT device and proposals are made as following:

- Observation 1: for the ideal OOK signal, the backscattering loss is expected as 6dB by removing DC power;

- Observation 2: : for the ideal BPSK signal, the backscattering loss is expected as 0dB;

- Proposal 1: for the practical OOK backscattering loss, propose to define the minimum requirement 8dB without considering antenna efficiency;

- Proposal 2: for the practical BPSK backscattering loss, propose to define the minimum requirement 2dB without considering antenna efficiency.

- Observation 3: propose to define the charging REFSENS requirement at the peak direction as -27dBm.

- Proposal 3: propose to define the -34dBm with 99% inventory success rate as the decoding REFSENS requirement under the condition of device charged to enable the work.

- Proposal 4: propose to define the spherical coverage requirement at the 48% coverage as -31dBm with 99% inventory success rate.

- Proposal 5: propose to consider the following R2D FRC as starting point for further discussion.

|  | SCS | 15 | 15 | 15 | 15 |
| --- | --- | --- | --- | --- | --- |
|  | PRB | 1 | 2 | 3 | 4 |
| SIP | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OOK |  |  |  |  |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | OOK |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
| chip number except for SIP,padding |  | 228 | 228 | 228 | 228 |
| Padding |  | 6 | 12 | 2 | 2 |

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Approved.

- R4-2414304, WF on Co-existence study for ambient IoT, Approved

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance

- R4-2502235,On the RF requirements for Ambient IoT Device, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2506842

Annex A (normative):
Reference measurement channels

A.1	Fixed Reference Channels for reference sensitivity level (OOK)

| Reference channel | Configuration | A-FR1-B1-1 | A-FR1-B1-2 | A-FR1-B1-3 | A-FR1-B1-4 |
| --- | --- | --- | --- | --- | --- |
| SIP | SCS | 15 | 15 | 15 | 15 |
|  | PRB | 1 | 2 | 3 | 4 |
|  | Bit length | 8 | 8 | 8 | 8 |
|  | M_SIP | 4 | 4 | 4 | 4 |
|  | OFDM | 2 | 2 | 2 | 2 |
| CAP | Bit length | 4 | 4 | 4 | 4 |
|  | M | 2 | 2 | 2 | 2 |
|  | OFDM | 2 | 2 | 2 | 2 |
| PRDCH | TBS | 96 | 96 | 96 | 96 |
|  | CRC | 16 | 16 | 16 | 16 |
|  | Line encoding |  |  |  |  |
|  | M | 2 | 2 | 2 | 2 |
| postamble | Bit length | 4 | 4 | 4 | 4 |
|  | M | 6 | 12 | 2 | 2 |
|  | chip number except for SIP,padding | 228 | 228 | 228 | 228 |
|  | Padding | 6 | 12 | 2 | 2 |