3GPP TSG-RAN WG4 Meeting#116                                R4-2511130

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for CW node

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Issue 3-2-1: OFF power - Agreement: - OFF power is -50dBm, RBW is FFS - Issue 3-4: Phase noise - Agreement: - For BS with phase noise cancellation capability, no phase noise requirement is specified. - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS. - Issue 3-6: Spurious emissions - Agreement: - The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz - Spurious emissions limits are defined as in Table 2.2-5. - Table 2.2-5: Spurious emissions limits Frequency Range; Maximum Level; Measurement bandwidth. 9 kHz f < 150 kHz; -36 dBm; 1 kHz. 150 kHz f < 30 MHz; -36 dBm; 10 kHz. 30 MHz f < 1000 MHz; -36 dBm; 100 kHz. 1 GHz f < 12.75 GHz; -30 dBm; 1 MHz. 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz; -30 dBm; 1 MHz. - Issue 3-7: Intermodulation - Agreement: - No need to define intermodulation.

Discussion

- Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

- In the last RAN4 meeting, there were some discussion how to define ON/OFF power for CW node since the CW transmission is not always on or contiguous transmission, therefore some ON/OFF power might be needed. During the last RAN4 meeting, there were some discussions on measurement bandwidth for CW signal OFF power. From our understanding, to ensure the minimum impacts of CW OFF power on the UL performance of NR uplink, therefore single PRB should be baseline since this is the finest granularity for the scheduling in the NR uplink.

|  | Channel bandwidth / Transmit OFF power / Measurement bandwidth |
| --- | --- |
|  | 0.18MHz |
| Transmit OFF power | -50 dBm |
| MBW | 180KHz |

- Proposal 1: propose to define the OFF power for CW node transmission as -50dBm/180KHz.

- Issue 3-4: Phase noise

- In the previous RAN4 meetings, there were some discussions on the phase noise requirement and also its impacts on D2R reception. In the following figure, some initial simulation results under the AWGN channel are provided for further discussion. It could be observed that phase noise -70dBc/15KHz still have limited impacts e.g. less than 1dB@BLER 10%. If phase noise is improved as -80dBc/15KHz or -90dBc/15KHz, then the impacts on D2R reception will further limited and almost negligible.

Figure 1. phase noise impacts on D2R reception under the AWGN channel

- Observation 1: if the phase noise is larger than 80/90dBc at 15KHz freq offset, the impacts on D2D decoding would be marginal.

- Considering the phase noise is quite implementation dependent, also the BS capability for CW phase cancellation is also under the discussion in RAN4. From the practical deployment perspective, we don’t see too much necessity to have different vendors providing different nodes in the A-IoT deployment. In short, we propose not to define the phase noise for CW signal in RAN4.

Proposal 2: propose not to define the phase noise requirement for CW node.

- Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

- FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

During the last RAN4 meeting, there was one proposal to specify the 3us timing error requirement between A-IoT BS and CW node. From our understanding, this is quite similar as network sync error requirement as defined in RRM spec, we propose not to discuss this requirement in RF session unless there are any request from RRM requirement definition perspective.

Proposal 3: propose not to specify the timing requirement for CW node.

Conclusions

In this contribution, we shared some initial views on the RF requirement for CW node in D1T1-B and proposals are made as following:

- Proposal 1: propose to define the OFF power for CW node transmission as -50dBm/180KHz.

- Observation 1: if the phase noise is larger than 80/90dBc at 15KHz freq offset, the impacts on D2D decoding would be marginal.

Proposal 2: propose not to define the phase noise requirement for CW node.

Proposal 3: propose not to specify the timing requirement for CW node.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- R4-2500782, RF requirements for A-IoT BS, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2501863, RF requirements for CW, Huawei, approved.

- R4-2505097, WF on A-IoT BS and CW requirements, Huawei, approved.

- R4-2508101, WF on A-IoT BS and CW requirements, Huawei, approved.

- R4-2507685 Further discussion on RF requirements for CW, Huawei, approved.