3GPP TSG-RAN WG4 Meeting#116                                R4-2511138

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on UE2UE CLI analysis

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. In the last RAN4 meeting, there were some discussion on UE2UE CLI problems in the adjacent channel for Urban Hotpot scenario. In this contribution, we would like to share some initial views on this topic.

Way Forward - UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception - WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model - WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any. - WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required: - The following table can be used to summarize the above aspects in the future RAN4 meeting:

Discussion

First of all, to reduce the p-max for the aggressor UE if identified by the victim UE would have direct impacts on the SBFD coverage enhancement. We can take the typical dual 2.5ms periodic DDDSU configuration with 30kHz subcarrier spacing, to reserve the some spectrum from the DL slot for uplink transmission, in theory, there would be around 7dB coverage gain (10*log10(3)=7dB, 3 is the number of downlink slot used for uplink transmission), however to reduce 3-4dB power for the aggressor UE, then it would almost reduce the half of coverage gain of SBFD BS.

Observation 1: to reduce the transmission power by 3-4dB for the aggressor UE would almost halve the coverage gain achieved through the SBFD BS configuration, rendering the overall improvement in uplink performance minimal.

In addition, as suggested in the last RAN4 meeting, considering the following deployment if there are coexistence deployment between SBFD BS and TDD BS in the adjacent channel, the uplink sub-band configuration should be placed far away from TDD DL to minimize the ACI impacts. As shown in the following Figure 1, if uplink sub-band is placed at the leftmost frequency (e.g. UD configuration) to minimize the leakage and blocking impacts, then ACLR5 should be dominant factor impacting the adjacent DL channel. According to our understanding, ACLR5 is around 80dBc which is mainly contributed by noise floor/spurious emission etc, however this configuration might be not good for uplink coverage since uplink sub-band configuration is close to carrier edge which will result in relative high MRP from the SBFD aware UE to comply with the emission mask. If the uplink sub-band is placed in the middle of the 100MHz carrier, then ACLR3 should be dominant factor impacting the adjacent DL channel which should be around 60dBc according to the experienced MSD estimation.

Figure 1. the illustration of DU configuration and DUD configuration coexisting with TDD DL

Observation 2: for 20MHz uplink subband configuration with 100MHz carrier, the ACLR3 60dBc should considered for DUD case, ACLR5 80dBc should be considered for UD case.

In addition, from the receiver side of TDD DL, ACS is expected as flat model as shown in the following illustrative filter response with the hanning window considered. The filter response will roll off with the increasing frequency offset. In the following figure, the rejection difference between the near freq offset and far freq offset is around 30dBc.

Figure 2. the illustration of frequency response of digital filter in the receiver side

Proposal 1: from the receiver side of victim UE in the TDD bands, the roll off of ACS should be considered with the realistic digital filter assumption.

Lastly, antenna efficiency should be also considered when analyzing the UE2UE CLI (e.g.6~7dB antenna efficiency).

Proposal 2: from UE2UE CLI analysis, the UE antenna efficiency should be considered in the simulation evaluation.

Conclusions

In this contribution, we want to share some further views on general aspects for SBFD BS and proposals/observations are made as following:

Observation 1: to reduce the transmission power by 3-4dB for the aggressor UE would almost halve the coverage gain achieved through the SBFD BS configuration, rendering the overall improvement in uplink performance minimal.

Observation 2: for 20MHz uplink subband configuration with 100MHz carrier, the ACLR3 60dBc should considered for DUD case, ACLR5 80dBc should be considered for UD case.

Proposal 1: from the receiver side of victim UE in the TDD bands, the roll off of ACS should be considered with the realistic digital filter assumption.

Proposal 2: from UE2UE CLI analysis, the UE antenna efficiency should be considered in the simulation evaluation.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2508708 Way Forward on UE-UE CLI problem statement and potential solutions