3GPP TSG-RAN WG4 Meeting#116                                R4-2511139

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on potentially new requirements for SBFD operation

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

- Issue 4-2-2: Absolute limit for in-channel adjacent subband leakage Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement - Sub-topic 4-3: In-channel adjacent subband blocking - Issue 4-3-1: In-channel adjacent subband blocking requirement Agreement - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

Discussion

## 2.1. Potential adjacent sub-band leakage

During the last RAN4 meeting, there were the agreement to define the in-band leakage requirement with some specific absolute limit e.g. CACLR limit as baseline. In the last RAN4#115 meeting, we reached the consensus for FR2 BS and also FR1 LA SBFD BS, however for the FR1 WA and MR SBFD BS, it still needs further discussions in the coming RAN4 meeting.

Compared with simulation results submitted in last RAN4 meeting, considering the typical AAS architecture for SBFD BS in FR1 instead of the legacy RRU BS, transmission power per TAB connector is updated as 33dBm for WA BS and MR BS. The number of transceivers of FR1 WA BS and FR1 MR BS might be different given different coverage and MIMO layer request. According to the simulation results in the following tab, the existing ACLR or CALCR absolute limit could be reused for it.

| SBFD configuration |  | FR1 WA Cat B | FR1 MR | FR1 LA |
| --- | --- | --- | --- | --- |
|  | Baseline | -15dBm/MHz | -25dBm/MHz | -32dBm/MHz |
| DUD | 40+20+40MHz | -23.5dBm/MHz | -23.5dBm/MHz | -32.5dBm/MHz |
| DU | 80+20MHz | -27.2dBm/MHz | -27.2dBm/MHz | -36.2dBm/MHz |
| DU | 50+50MHz | -28.8dBm/MHz | -28.8dBm/MHz | -37.8dBm/MHz |
| DU | 60+40MHz | -28.8dBm/MHz | -28.8dBm/MHz | -37.82dBm/MHz |
| DU | 70+30MHz | -27.9dBm/MHz | -27.9dBm/MHz | -36.90dBm/MHz |
| DU | 90+10MHz | -24.1dBm/MHz | -24.1dBm/MHz | -33.16dBm/MHz |
| NOTE: the transmission power per TAB connector is assumed as 33dBm for WA and MR BS; MR BS is assumed with different number of transceivers as WA BS. |  |  |  |  |

- Proposal 1: for FR1 WA and MR SBFD BS, reuse the existing absolute ACLR/CALCR limit.

## 2.2. Potential adjacent receiver requirements

During the last RAN4 meeting, there were some discussions on the necessity of in-channel adjacent sub-band blocking, it was agreed to define this requirement, however the exact value for adjacent sub-band leakage requirement is not confirmed yet. From our understanding, the DL sub-band signal from the aggressor BSs within the single carrier is orthogonal to the UL sub-band signal reception. In other words, there are no such kind of selectivity needed at all. However based on some comments received online, this DL sub-band interference might cause some nonlinearity to LNA, other RF component similar as in-band blocking signal. The simulation results are provided in the following figures and summarized in the following table.

FR1:

Without any CLI coordination

With some CLI coordination

Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level.

Observation 2:

|  | Without CLI | With CLI coordination NOTE 1 | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |
| NOTE 1: to find the worst interference, then considering the beam sweeping to alleviate the in-channel blocking by beam coordination solution. |  |  |  |

In the previous RAN4 meeting, the in-channel blocking requirement is kept in the square bracket for further discussion, however this tentative value is somehow close to the BS2BS CLI in the adjacent channel BS2BS CLI which seems not reasonable from our understanding. As shown in the following figure, the closet aggressor for in-channel blocking (e.g. Victim BS 2 in green) is far away with ISD distance isolation compared with the closet aggressor for in-band blocking aggressor (e.g. aggressor BS in red).

Proposal 2: propose to further check the assumptions if in-channel blocking requirements is close to in-band blocking requirements;

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

- Proposal 1: for FR1 WA and MR SBFD BS, reuse the existing absolute ACLR/CALCR limit.

Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level.

Observation 2:

|  | Without CLI | With CLI coordination NOTE 1 | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |
| NOTE 1: to find the worst interference, then considering the beam sweeping to alleviate the in-channel blocking by beam coordination solution. |  |  |  |

Proposal 2: propose to further check the assumptions if in-channel blocking requirements is close to in-band blocking requirements;

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung

- R4-2416584,Way forward for [112bis][306] NR_duplex_evo_General, Samsung

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2508707 Way Forward for [115][306] NR_duplex_evo_General, approved

Annex

Figure 1. FR1 WA SFBD BS with 40+20+40MHz configuration [-21.5dBm/MHz]

Figure 2. FR1 WA SFBD BS with 50+50MHz configuration [-21.5dBm/MHz]

Figure 3. FR1 WA SFBD BS with 80+20MHz configuration [-27.2dBm/MHz]

Figure 4. FR1 WA SFBD BS with 60+40MHz configuration [-28.8dBm/MHz]

Figure 5. FR1 WA SFBD BS with 70+30MHz configuration [-27.9dBm/MHz]

Figure 6. FR1 WA SFBD BS with 90+10MHz configuration [-24.1dBm/MHz]

TP

12.2.9	In-channel adjacent subband leakage for SBFD

12.2.9.1 General

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier.

The requirement shall apply during the transmitter ON period.

12.2.9.2 Limits and Basic limits

For SBFD operation in the unpaired spectrum, the ICASL shall be less than or equal to the value specified in table 12.2.9.2-1.

Table 12.2.9.2-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Category A Wide Area BS | TBD dBm/MHz |
| Category B Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.2.9.2-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |

12.2.9.3 Minimum requirement for SBFD BS type 1-H

The ICASL requirements for SBFD BS type 1-H are that for each TAB connector TX min cell group and each applicable basic limit in clause 12.2.9.2, the power summation emissions at the TAB connectors of the TAB connector TX min cell group shall not exceed a BS limit specified as the basic limit + X, where X = 10log10(NTXU,countedpercell).

NOTE:	Conformance to the SBFD BS type 1-H spurious emission requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the in-channel adjacent sub-band leakage power measured on each TAB connector in the TAB connector TX min cell group shall be less than or equal to the limit as defined in this clause for the respective frequency span.

Or

2)	The  in-channel adjacent sub-band leakage power at each TAB connector shall be less than or equal to the SBFD BS type 1-H limit as defined in this clause for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector TX min cell group.

12.5.10	OTA In-channel adjacent subband leakage for SBFD

12.5.10.1 General

OTA In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of UL sub-band configuration within one NR carrier. The measured power is TRP.

The requirement shall be applied per RIB during the transmitter ON period.

12.5.10.2 Minimum requirement for SBFD BS type 1-O

For SBFD BS type 1-O, the ICASL shall less than or equal to a basic limit defined in table 12.2.9.2-1 + X (where X = 9 dB).

12.5.10.3 Minimum requirement for SBFD BS type 2-O

For SBFD BS type 2-O, the ICASL shall be less than or equal to the value specified in table 12.5.10.3-1.

Table 12.5.10.3-1: SBFD Base station ICASL absolute basic limit

| SBFD BS category / BS class | ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | TBD dBm/MHz |
| Medium Range BS | TBD dBm/MHz |
| Local Area BS | TBD dBm/MHz |

Table 12.5.10.3-2: Filter parameters for the UL sub-band

| RAT | Filter on the assigned UL sub-band and corresponding filter bandwidth |
| --- | --- |
| NR | NR of the closet BW with SCS that provides largest transmission bandwidth configuration compared with UL sub-band |