3GPP TSG-RAN WG4 Meeting#116                                R4-2511143

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.21.3.3

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Issue 2-1: ACS requirement - Way forward - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement. Issue 2-2: In-band blocking for FR1 WA BS - Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered - Issue 2-3: In-band blocking for FR2-1 WA BS - Way forward - Encourage companies to have offline discussion for the simulation assumptions before next meeting. - Issue 2-4: In-band blocking for FR1 Micro BS - Way forward - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point - Companies to check the blocking level in the range of [-20, -32] dBm - Issue 2-5: Necessity of New RX intermodulation requirement with 1 or 2 interfering signal - Way forward - Down-select to one of the below two options in the next meeting - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case - Interfering signal 1: CW - Interfering signal 2: TBD MHz NR - Power level: the final agreed in-band blocking level -9 dB - Issue 2-6: In-channel selectivity - Way forward - Check whether existing ICS requirements can be reused

Discussion

## 2.1. Rx requirements

### 2.1.1. IBB requirements

- During the last RAN4 meeting, there were some discussions on IBB evaluation to figure out the appropriate requirement definition. In the following section, we provide some initial simulation results according to the simulation assumption captured in TR 38.858.

- FR1:

Figure 2.1.1-1. the evaluation results for Scenario 1

Figure 2.1.1-2. the evaluation results for Scenario 5

| Scenario | Grid shift | Per antenna elements [dBm] | Per antenna array [dBm] | Per sub-array [dBm] |
| --- | --- | --- | --- | --- |
| 1 | 100% | -25.3301 | -19.7623 | TBD |
| 5 | 100% | -38.9889 | -35.7181 | TBD |

- It should be noted that -25dBm per antenna elements is based on the assumption of 100MHz DL signal as interfering signal, however the in-band blocking requirement is defined based on 20MHz DFT-s-OFDM, therefore some scaling factors should be considered. Meanwhile considering the sub-array implementation for AAS BS architecture e.g.

- Option 1: -25dBm-10*log10(100/20)+10*log10(3)=-27.2dBm assuming 1x3 sub-array assumption;

- Option 2: -25dBm-10*log10(100/20)+10*log10(4)=-25.96dBm assuming 1x4 sub-array assumption;

- Based on the above considerations, we made the following proposal for this requirement.

- Proposal 1: for FR1 WA SBFD BS, propose the in-band blocking level as -26dBm with the following interfering signal configuration.

Figure 2.1.1-3a. the interfering signal configuration for UD case

Figure 2.1.1-3b. the interfering signal configuration for DU case

Figure 2.1.1-3c. the interfering signal configuration for DUD case

- Observation 1: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement.

- Proposal 2: for FR1 MR SBFD, propose to define the in-band blocking requirement as -32dBm with the same interfering signal configuration.

- FR2-1:

- In the previous RAN4 meeting, there were some discussions on how to conduct the simulation for this FR2 IBB blocking requirement. Here is the approach we used to provide the simulation results as shown in Figure2.1.1-4. The reference point is after the BS analog panel which means the beamforming gain of panel is considered since FR2 BS supported beamforming gain within panel via the beamforming chipset for the panel (which is called as hybrid beamforming). For both wanted signal and IBB interference signal, the beamforming gain is considered as the worst assumption.

Figure 2.1.1-4. the interfering signal configuration for DUD case

Figure 2.1.1-5a. the evaluation results for Scenario 6

Figure 2.1.1-5b. the evaluation results for Scenario 6

Figure 2.1.1-6a. the evaluation results for Scenario 9

Figure 2.1.1-6b. the evaluation results for Scenario 9

Similar as FR1, considering the 99% CDF to define the IBB interfering power level, the blocking level over antenna array and relative power difference between wanted signal and IBB signal is collected as following. It could be observed that the relative power difference between wanted signal and IBB signal in Scenario 6 and Scenario 9 is still close to the existing IBB requirement if without considering 6dB performance degradation of wanted signal. In other words, for Scenario 6 and 9, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

| Scenario | Grid shift | Per antenna array [dBm] | Relative power difference [dBm] |
| --- | --- | --- | --- |
| 6 | 100% | -48.3600 | 25.6302 |
| 9 | N/A | -56.5661 | 16.8709 |

- Observation 2: for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

- Proposal 3: for FR2 WA SBFD, propose to reuse the existing in-band blocking requirements.

### 2.1.2. ACS requirements

- In the previous RAN4 meetings, there were some discussions on ACS requirement for SBFD BS for the BS2BS CLI in the adjacent channel. If we follow the legacy approach with 5% throughput loss as the criteria, then we need to increase the offset for baseline offset. Currently, if ACIR is increased by 8dB, then throughput degradation @5% is still around 43.5% which is much beyond the acceptable degradation according to our simulation results as captured in TR38.858. In addition, considering the DL subband (e.g. 80MHz with 100MHz carrier) serve as guard band between UL sub-band and adjacent channel, the achievable ACIR is expected much better than baseline. In addition, from the receiver of uplink sub-band perspective, the 20MHz DFT-s-OFDM signal in 1st adjacent channel is also far away from the uplink signal reception according to the proposed interfering signal configuration which is somehow similar as the legacy IBB blocking requirement.

- Table 2.1.2-1 Simulation results for FR1 WA case 3

| TP degradation @5% in % |  |  |  |  | TP degradation @Mean in % |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB | Relative ACIR | +2 dB | +4 dB | +6 dB | +8 dB |
| 76.99217307 | 69.82808219 | 61.25902009 | 52.52795818 | 43.50328929 | 13.00074882 | 10.13748818 | 7.913888795 | 6.021514571 | 4.672411119 |

- Based on the above consideration and also fact that TDD BS interfering signal is not expected to be changed due to the different freq offset in the field. We propose to define the power level of ACS interfering signal the same as that of IBB interfering signal.

Figure 2.1.2-1. the interfering signal configuration for UD case

- Proposal 4: for ACS requirement of SBFD BS, propose to reuse the power level of IBB requirement for that of ACS requirement.

### 2.1.3. Receiver intermodulation

Agreement: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

For the SBFD slot/symbols, there might be some self interference and additional Rx intermodulation caused by downlink part especially when considering with CW signal or NBB or general Rx intermodulation signals configured during the Rx IMD test as shown in the following.

Based on the simulation results for IBB and proposed ACS requirement and potential Rx IMD requirement as shown in Figure 2.1.3-1/2.

If we check the 3rd intermodulation between 20MHz general blocking and NBB blocking and CW signal, it would not fall into the uplink sub-band frequency range the 5rd intermodulation signal between 20MHz general blocking and NBB blocking requirement should be marginal compared with 3rd intermodulation with 6dB performance degradation. In other words, this requirement might be not necessary without consideration of DL in-channel in-band blocking signal.

If we check the 3rd intermodulation between 20MHz general blocking and DL in-channel blocking signal as shown in Figure 2.1.3-1, its impacts would be expected to be less than ACS/IBB interfering signal as shown in Figure 2.1.3-2 since its frequency is a bit far away compared with ACS and IBB and with the same power level.

However for Rx intermodulation signal for NBB and DL in-channel blocking signal, it might be difficult to judge which is worse case compared with Rx intermodulation signal for ACS/IBB and DL in-channel blocking due to its high PSD of NBB signal. According to the practical deployment for standalone NB-IoT in the TDD band in the terrestrial network, this is not commercialized yet as far as we know. In other words, the coexistence requirement between standalone NB-IoT and SBFD in the TDD bands is also questionable.

Figure 2.1.3-1: Example of Rx intermodulation requirement for SBFD BS

Figure 2.1.3-2: Example of Rx ACS/IBB requirement for SBFD BS

f1

f2

2f1-f2

BW=b

interfering signal

2f2-f1

BW=a

wanted signal

(a+b)/2

2a+b

2b+a

a + b

Figure 2.1.3-3:. the generic model using for the analysis of 3rd intermodulation mechanism

Based on the above considerations and conditional proposals for ACS and IBB requirement for SBFD symbols, we propose not to define the additional Rx IMD requirement for SBFD symbols.

- Proposal 5: for receiver intermodulation requirements, propose not to define the additional Rx IMD requirement for SBFD symbols.

### 2.1.4. Receiver ICS requirements

In the previous RAN4 meetings, there were some discussions how to define ICS requirement and whether the current requirement can be reused. In the following section, we would like to clarify the background for this requirement.

First of all, this requirement is defined with wanted signal and interfering signal around Fc as shown in Figure 2.1.4-1. For the SBFD BS, the wanted signal and interfering signal should be placed in the middle of uplink sub-band instead of the legacy carrier center Fc due to the DL and UL sub-band configuration with the same carrier as shown in Figure 2.1.4-2.

- Proposal 6: for receiver ICS requirement, the NOTE in ICS requirement should be updated to place the wanted signal and interference signal around the middle of uplink sub-band instead of Fc of the carrier bandwidth.

In addition, self interference which is agreed as 0.7dB would fall within the whole uplink sub-band, in other words, the both REFSENS and interference signal requirement should be increased with 0.7dB. Two interference signal should be linearly added instead of dB added due to the different interference source.

- Proposal 7: for receiver ICS requirement, power level of wanted signal and interference signal should be updated with self interference and IQ image linearly.

Figure 2.1.4-1: the illustration of the legacy ICS requirement in TS38.104

Figure 2.1.4-2: the illustration of the SBFD ICS requirement

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -92.3 | -71.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -98.8 | -78.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -92.6 | -71.4 | DFT-s-OFDM NR signal, 30 kHz SCS, 50 RBs |
| 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -98.2 | -78.4 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -92.7 | -71.6 | DFT-s-OFDM NR signal, 60 kHz SCS, 24 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around in the middle of uplink sub-band PRB configuration. |  |  |  |  |  |

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

- Proposal 1: for FR1 WA SBFD BS, propose the in-band blocking level as -26dBm with the following interfering signal configuration.

- Observation 1: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement.

- Proposal 2: for FR1 MR SBFD, propose to define the in-band blocking requirement as -32dBm with the same interfering signal configuration.

- Proposal 3: for FR2 WA SBFD, propose to reuse the existing in-band blocking requirements.

- Proposal 4: for ACS requirement of SBFD BS, propose to reuse the power level of IBB requirement for that of ACS requirement.

- Proposal 5: for receiver intermodulation requirements, propose not to define the additional Rx IMD requirement for SBFD symbols.

- Proposal 6: for receiver ICS requirement, the NOTE in ICS requirement should be updated to place the wanted signal and interference signal around the middle of uplink sub-band instead of Fc of the carrier bandwidth.

- Proposal 7: for receiver ICS requirement, power level of wanted signal and interference signal should be updated with self interference and IQ image linearly.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2504752	Way Forward for [114bis][307] NR_duplex_evo_BSRF

- R4-2508713 Way Forward for [115][307] NR_duplex_evo_BSRF