3GPP TSG-RAN WG4 Meeting #116	R4-2511150

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.2

Source:	Ericsson

Title:	On general issues for AI/ML

Document for:	Discussion

# 1	Introduction

In this contribution, we further discuss the remaining issues related to:

- post-deployment pre/post activation functionality testing, and

- LCM requirements.

The related RAN4#115 and RAN4#114bis agreements on the above topics are captured in [2] and [1], respectively.

# 2	Post-deployment pre/post activation functionality testing

Post-deployment testing refers to the ability of the AI/ML framework to ensure that deployed models that are updated can continue to be expected to perform similarly to models passing RAN5 conformance testing. It is clearly not possible to re-apply conformance testing to UEs that are already in use, and hence the framework should aim to achieve confidence that performance has not degraded due to model update.

During RAN4#114bis, there was not much discussion on the topic, apart from agreeing a naming update [1].

Agreement:

- Rename “option 1” as “Post-deployment pre-activation functionality/configuration update testing”

- Rename “option 2” as “Post-deployment post-activation functionality testing based on performance monitoring”

The only related agreement from RAN4#115 [2] is the following.

Agreement:

- Option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized.

- it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases

In the following, views are provided on some of the open issues listed in the table in [R4-2502953] relating to option 1, however we note that the discussion on option 2 (monitoring) is prioritized.

Pre-activation functionality/configuration update testing

Whether the testing of a new model is on a device or in a lab.

Assuming that a model update is created within the computing facilities of a UE vendor (or other AI/ML model vendor), then it would make sense for validation/testing of the model to take place within the model development facility. It is unlikely to be preferable to provide additional hardware and software support to emulate model operation within a device. Within the development facility, there would be a need to emulate/test the model considering the realistic performance of the device hardware. Assuming that the AI/ML facility is aiming to produce a useful model for the device in question, it is reasonable to expect that there is a means of emulating performance on the actual device.

Whether models that have been updated post conformance testing should be kept in a device.

Assuming that post-deployment “testing” has already been performed then this question does not appear to be related to post-deployment testing, more on how the device manages it’s already deployed models and can be considered to be outside of the scope of discussing post-deployment validation.

Whether parallel operating of one model and (in-device) testing of another can be assumed.

As discussed above, it would be natural to validate a model update in the same location that the update is created, and it would be reasonable to assume that a facility producing a model update for a specific device would have a means to emulate the device sufficiently well.

It is not obvious why it would be necessary and whether it would even be feasible to run “testing” within a device itself.

Whether signaling from a UE of changes/updates are needed.

In case the download of functionality would be managed from the network, the signaling would be natural. For UE side management of model updates, signaling is not necessary. What would be necessary would be a framework for the AI/ML model vendor to perform validation and declare validation completed. The role of standardization would be to create a set of conditions and potentially test data that could be used for validation and would create confidence that the model could provide reasonable performance in conditions similar to thos envisaged for the actual conformance testing.

Whether there is any relation to GCF procedures.

As discussed at RAN4#114, GCF incorporates procedures for handling UE software updates. These procedures may or may not be suited for AI model updates; it is important to take account of the procedures if they are useful, but ensure a reasonable impact if any to UEs, adapt if needed and not use if inappropriate. As a first step, we recommend to ask RAN5 to consider and clarify the procedures today.

Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes.

It is not fully clear how the scale of a model change could be considered. Clearly updating on a very frequent and short time basis would not be suitable for “Pre-activation functionality/model update testing”, whereas occasional and large changes could be.

Post activation management based on LCM

Conceptually, “monitoring” of functionality appears to be a practical way to manage post-deployment performance. The main questions are whether monitoring that enables specific functionality issues to be remedied is really feasible, and whether monitoring is interoperable.

With regard to feasibility, it is important to consider that in real life, a very large number of factors influence the behavior of a link, for example:

- Changing channel conditions

- Changing user conditions, e.g. Doppler etc.

- Interference variations

- Traffic and scheduling

- Behaviour of analogue components in some circumstances; e.g. BS CFR/DPD, UE AGC, power control etc.

- If there would be multiple AI functionalities present, behaviour of other AI functionalities (e.g. if there would be AI based channel estimation, CSI prediction, CSI compression, decoding etc.)

- ...

Monitoring needs to be able to specifically identify that a performance issue arises within an AI model and could be remedied by some kind of fallback in real world conditions.

Regarding interoperability, in particular if a UE makes reports relating to functionality monitoring, it is important that the reports are comparable between different UE implementations and models. If the interpretation of a report would need an understanding of which particular type of UE has made the report, then interoperability of monitoring would be lost, which would vastly increase monitoring complexity for the network. Examples of variation between UEs could be:

- Confidence metrics that actually lead to different levels of performance, or performance variations in different timescales between different UEs.

- Metrics that may vary in different ways between different UEs because of other factors than the model (e.g. differences in analogue performance etc.)

- Different timescales, averaging etc. when making the reports.

- Observation 1: It is not obvious as yet whether monitoring can useful identify individual model behavior and whether it can be interoperable.

- Proposal 1: Continue to work on monitoring to understand how useful and interoperable it can be.

# 3	LCM requirements

## 3.1	On delay requirements for AI/ML functionality activation

The following was agreed in RAN4#114bis [1] on activation delay requirements:

Agreement:

- For beam management use case

  - FFS on defining the functionality activation delay requirements for the first periodic and semi-persistant reporting based on AI/ML

  - Define functionality activation delay requirement for the first aperiodic reporting based on AI/ML

  - FFS on the definition of first reporting

Further agreement on activation delay requirements was made in RAN4#115 [2]:

Agreement:

- Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction

  - It is FFS for CSI prediction

- For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism.

- For semi-persistent CSI reporting

  - Activation delay starts at the reception of the MAC-CE/DCI

- For aperiodic CSI reporting

  - Activation delay starts at the reception of the DCI

- Proposal 2: Further details on functionality activation delay requirement for AI/ML beam management can be discussed under the beam management use case.

Defining activation delay requirements was agreed for beam management with all periodic/semi-persistent/aperiodic CSI reporting. Similarly to beam management, the functionality activation procedure for CSI prediction can also suffer from longer delays if not controlled by requirements. It is therefore proposed that functionality activation delay is also defined for CSI prediction.

- Proposal 3: Functionality activation delay requirement is defined for the CSI prediction use case.

- Proposal 4: To define functionality activation delay requirement for the CSI prediction use case, the AI/ML beam management framework can be reused.

# Conclusion

In the previous sections we made the following observations:

- Observation 1 (post-deployment management based on LCM): It is not obvious as yet whether monitoring can useful identify individual model behavior and whether it can be interoperable.

- Proposal 1 (post-deployment management based on LCM): Continue to work on monitoring to understand how useful and interoperable it can be.

- Proposal 2 (LCM): Further details on functionality activation delay requirement for AI/ML beam management can be discussed under the beam management use case.

- Proposal 3 (LCM): Functionality activation delay requirement is defined for the CSI prediction use case.

- Proposal 4 (LCM): To define functionality activation delay requirement for the CSI prediction use case, the AI/ML beam management framework can be reused.

# References

[1] R4-2505107, WF on AI ML for air interface study, Ericsson, Apr. 2025.

[2] R4-2508051, WF on AI/ML part 2, Ericsson, May 2025.