3GPP TSG-RAN WG4 Meeting #116 	R4-2511152

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.26.3

Source: 	Ericsson

Title:	On RRM core requirements for XR

Document for:	Approval

# Introduction

The RAN4#115 WF is in [10]. The RAN4#114bis WF is in [9]. The RAN4#114 WF is in [7], RAN4#113 WF is in [6]. The RAN4#112-bis WF is in [5]. The RAN4#112 agreements and WF are in [4]. The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1].

The following agreements were made in RAN4#115 [10]:

- Semi-static control (the issue is now closed with the below)

- From RAN4 perspective, no consensus to support semi-static solutions in Rel-19.

- UAI was further clarified and an LS was sent to RAN2:

- The UE may provide the UAI related information, for the UAI in the agreement captured above, per MG configuration.

- per-FR gap if gap is configured with gapConfig with per-FR gaps

- per gap configuration if gap is configured with gapConfig-r17

- Type 2 gaps:

- MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

- The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules.

- DCI skipping cannot be applied if any gap is configured with gapAssociationPRS

- FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

- NCSG gaps:

- MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17.

- Pre-configured gaps:

- MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17.

- Rel-18 concurrent gaps:

- MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

- Skipping indication on frequency domain:

- It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs.

- Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

- Naming convention in CRs:

- Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI.

- Use Lcancel for number of skipping occasions

- Indication of which formula/table to use

- For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new)

- Use RAN2 pseudo code approach for new clause 9.1.y

- UE capabilities

  - Define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation

- UL XR issues

  - Issues closed, should have been discussed in RAN1.

- Impact on different frequency layers (the original proposals of this issue can be found in R4-2505575 and Chairman notes)

  - Not pursue Proposal 1 and 2 in Rel-19.

Below we further discuss the issues related to RAN4 requirements impact for UE operating XR.

# Discussion

## RRM requirements enhancements for XR

NR-U like approach was agreed for defining measurement period for measurements impacted by MG cancellation under XR operation.

### Inter-frequency cell detection

- Proposal 1: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 2: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 3: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

### SSB time index detection

- Proposal 4: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, (3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, (MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 5: The maximum numbers of extensions of inter-frequency SSB time index detection period are for

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 6: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

### Inter-frequency measurements

- Proposal 7: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, (Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 8: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 9: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

upon exceeding Xmeas,max the UE shall restart the measurement.

### Applicability conditions

Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 10: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 11: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

### UE power class

Considering the WI scope, practical XR scenarios, and scenarios when the same UE can be configured with measurement gaps or inter-frequency measurements, at least some of the UE power classes are not relevant in the requirements related to measurement gap cancellation for XR. The most typical UE power class which would be relevant for such requirements is UE power class 3.

- Proposal 12: The requirements for measurement gap cancellation apply for UE power class 3.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

| Condition | FR1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8+XPSS/SSS,gaps) DRX cycle CSSFinter |

| Condition NOTE1 | FR2-1: TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |

- Proposal 2: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=0.5  Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 3: UE behaviour when the maximum number of extensions for inter-frequency cell detection is exceeded:

Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

- Proposal 4: Inter-frequency SSB time index detection requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, (3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (3+Xind,gaps) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, (MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |

- Proposal 5: The maximum numbers of extensions of inter-frequency SSB time index detection period are for

FR1:

Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xind,gaps,max=1.5  MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and

Xind,gaps,max=0.5  MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 6: UE behaviour when the maximum number of extensions for inter-frequency SSB time index detection is exceeded:

Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

- Proposal 7: Inter-frequency measurement period requirements with MG cancellation due to XR can be as in the tables below for FR1 and FR2-1, respectively:

| Condition NOTE1 | FR1: TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8 + Xmeas) DRX cycle CSSFinter |

| Condition | FR2-1: TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, (Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |

- Proposal 8: The maximum numbers of extensions of inter-frequency cell detection period are for

FR1:

XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

XPSS/SSS,gaps,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms.

FR2:

Xmeas=1.5  Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms,

Xmeas= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms,

Xmeas=0.5  Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms.

- Proposal 9: UE behaviour when the maximum number of extensions for inter-frequency measurements is exceeded:

Upon exceeding Xmeas,max the UE shall restart the measurement.

- Proposal 10: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Proposal 11: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

- Proposal 12: The requirements for measurement gap cancellation apply for UE power class 3.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

[6] R4-2420103, WF on RRM requirements for XR_Ph3, Qualcomm, Nov. 2024.

[7] R4-2502700, WF on RRM requirements for XR_Ph3, Nokia, Feb. 2025.

[8] R4-2503921, draftCR to 38.133, Inter-frequency measurement requirements with XR, Ericsson, Apr. 2025.

[9] R4-2504911, WF on RRM requirements for XR_Ph3, Nokia, Apr. 2025.

[10] R4-2508285, WF on RRM requirements for NR_XR_Ph3, Nokia, May 2025.