3GPP TSG-RAN WG4 Meeting #116	R4-2511155

# Bengaluru, India,  August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5969 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) CR clarifying the UE UL transmit timing requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides a clarifition to the UE UL transmit timing requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update to 7.1.2.1 Gradual timing adjustment and clarifying UE behavior if such gradual adjustment leads to exceeding the UE minimum MTTD requirement. - Update to 7.5.2 for inter-band EN-DC regarding TA adjustment, clarifying UE behavior if such TA adjustment leads to exceeding the UE minimum MTTD requirement. - Update to 7.5.3 for intra-band EN-DC regarding TA adjustment, clarifying UE behavior if such TA adjustment leads to exceeding the UE minimum MTTD requirement. - Update to 7.5.4 for NR CA regarding TA adjustment, clarifying UE behavior if such TA adjustment leads to exceeding the UE minimum MTTD requirement. - Update to 7.5.5 for inter-band NE-DC regarding TA adjustment, clarifying UE behavior if such TA adjustment leads to exceeding the UE minimum MTTD requirement. Update to 7.5.6 for inter-band NR DC regarding TA adjustment, clarifying UE behavior if such TA adjustment leads to exceeding the UE minimum MTTD requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.1, 7.5.2, 7.5.3, 7.5.4, 7.5.5, 7.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This issue has been discussed also in RAN4#114 (Athens) and RAN4#115 (Malta). This CR is an updated resubmission of R4-2501887, R4-2507274 (with a change in applicable release from Rel-15 to Rel-18 and requirement formulation). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Change #1>

#### 7.1.2.1	Gradual timing adjustment

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE shall adjust the timing such that timing error is within Te.

If the UE is configured with more than one TAG, when the transmission timing error between the UE and the reference timing used within the TAG exceeds Te then the UE shall adjust the timing such that timing error is within Te in the TAG.

When the UE is configured with an sTAG, if after adjusting its timing error is within Te, the UE is not required to transmit in the sTAG provided:

-	the UE is configured with a pTAG and one or two sTAG, and the transmission timing difference between pTAG and sTAG exceed the maximum transmission timing difference (i.e., 32.47us) after such adjustment, or

-	the UE is configured with synchronous dual connectivity, and the transmission timing difference between pTAG and psTAG exceed the maximum transmission timing difference (i.e., 35.21us) after such adjustment.

If the transmission timing difference after timing adjustment exceeds the maximum transmission timing difference requirement, the UE may continue adjustment in this TAG. The UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal.

The reference timing shall be NTA+NTA offset×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms for SCS of UL signals smaller or equal to 120 kHz and 100 ms for SCS of uplink signals larger or equal to 480 kHz.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in table 7.1.2.1-1.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference point for each TAG for the PUCCH/PUSCH/SRS and PRACH transmission triggered by PDCCH order is defined in clause 7.1.1.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2-1 | 60 | K*64*Tc | 2.5*64*Tc |
|  | 120 | K*64*Tc | 2.5*64*Tc |
| 2-2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
|  | 480 | 0.8*64*Tc | 0.8*64*Tc |
|  | 960 | 0.8*64*Tc | 0.8*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5. |  |  |  |

<Change #2>

#### 7.5.2.1	Minimum requirements for inter-band synchronous EN-DC

The requirements in this clause apply as a reference for inter-band synchronous EN-DC.

The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell for inter-band synchronous EN-DC as shown in table 7.5.2.1-1 1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

If the uplink transmission timing difference between PCell and PSCell exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on the PSCell. The UE is allowed to transmit in the uplink PCell and PSCell provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in the PSCell.

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.

For E-UTRA FDD-NR FDD inter-band EN-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in table 7.5.2.1-1 provided that UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and not provided with nonCollocatedTypeMRDC-r18, and in table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16 or UE supporting requirementTypeIndication-r18 is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r18.

Table 7.5.2.1-1 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 | 35.21 |

<Change #3>

### 7.5.3	Minimum requirements for intra-band EN-DC

For intra-band EN-DC, only co-located deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2].

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in table 7.5.3-1 for E-UTRA TDD-NR TDD and E-UTRA FDD-NR FDD intra-band EN-DC provided the UE does not indicate that it is capable of asynchronous FDD-FDD EN-DC operation [16].

If the uplink transmission timing difference between PCell and PSCell exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on the PSCell. The UE is allowed to transmit in the uplink PCell and PSCell provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in the PSCell.

Table 7.5.3-1: Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 5.21Note1,Note 2 |
| 15 | 30 | 5.21Note 2 |
| 15 | 60 | 5.21 Note 2 |
| NOTE 1: This is not applicable for a UE which indicates the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for this UE. NOTE 2: If the transmission timing difference exceeds the cyclic prefix length of the UL Sub-carrier spacing for data in PSCell, NR UE Tx EVM degradation is expected for the symbol that is overlapping the LTE subframe boundary NOTE 3: The requirement in this table is also applicable for intra-band synchronous NE-DC. |  |  |

### 7.5.4	Minimum requirements for NR Carrier Aggregation

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or

-	configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode.

If the uplink transmission timing difference between pTAG and sTAG exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on the sTAG. The UE is allowed to transmit in the uplink pTAG and sTAG provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in the sTAG.

If the uplink transmission timing difference between the sTAGs exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on one sTAG. The UE is allowed to transmit in the uplink in both sTAGs provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in one sTAG.

The UE shall be capable of handling at least a relative transmission timing difference between subframe timing of all pairs of TAGs between FR1 and FR2-2 as shown in table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided.

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | 26.1 |
| NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

### 7.5.5	Minimum requirements for inter-band NE-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between PCell and E-UTRA PSCell as shown in table 7.5.5-1 for inter-band asynchronous NE-DC.

Table 7.5.5-1: Maximum uplink transmission timing difference requirement for inter-band asynchronous NE-DC

| Sub-carrier spacing in PCell (kHz) | UL Sub-carrier spacing for data in E-UTRA PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 30 | 15 | 250 |
| 60 | 15 | 125 |
| 120 | 15 | 62.5 |
| NOTE 1: Void |  |  |

Table 7.5.5-2 Void

#### 7.5.5.1	Minimum requirements for inter-band synchronous NE-DC

The requirements in this clause apply as a reference for inter-band synchronous NE-DC.

The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between PCell and E-UTRA PSCell for inter-band synchronous NE-DC as shown in table 7.5.5.1-1. The requirements for synchronous NE-DC are applicable for NR TDD- E-UTRA TDD, NR FDD- E-UTRA FDD, NR TDD- E-UTRA FDD and NR FDD- E-UTRA TDD inter-band NE-DC.

If the uplink transmission timing difference between PCell and PSCell exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on the PSCell. The UE is allowed to transmit in the uplink PCell and PSCell provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in the PSCell.

For NR FDD-E-UTRA FDD inter-band NE-DC with overlapping DL bands, only synchronized operation is assumed if UE does not indicate that it is capable of asynchronous FDD-FDD NE-DC operation. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between NR PCell and PSCell as shown in table 7.5.5.1-1 if UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16, and the same requirement as in table 7.5.3-1 apply provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r16.

Table 7.5.5.1-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NE-DC

| Sub-carrier spacing in PCell (kHz) | UL Sub-carrier spacing for data in E-UTRA PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 30 | 15 | 35.21 |
| 60 | 15 | 35.21 |
| 120 | 15 | 35.21 |

### 7.5.6	Minimum requirements for inter-band NR-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between PCell and PSCell as shown in table 7.5.6-1 provided that the UE indicates that it is capable of synchronous NR-DC only [14].

If the uplink transmission timing difference between PCell and PSCell exceeds the maximum value the UE can handle as specified in this paragraph, due to receiving a TA command, the UE may stop transmitting on the PSCell. The UE is allowed to transmit in the uplink PCell and PSCell provided that the UE meet all the transmit timing requirements defined in clause 7.1.2; otherwise, the UE shall not transmit any uplink signal in the PSCell.

Table 7.5.6-1: Maximum uplink transmission timing difference requirement for inter-band synchronous NR-DC

| Frequency Range |  | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| Cell in MCG | Cell in SCG |  |
| FR1 | FR1 | 34.6 |
| FR2-1 | FR2-1 | 8.5 |
| FR1 | FR2-1 | 34.1 |
| FR1 | FR2-2 | 34.1 |

The UE shall be capable of handling a maximum uplink transmission timing difference between PCell and PSCell as shown in table 7.5.6-2 provided that the UE indicates that it is capable of asynchronous NR-DC [14].

Table 7.5.6-2 Maximum uplink transmission timing difference requirement for inter-band asynchronous NR-DC

| Max {Sub-carrier spacing in PCell (kHz), Sub-carrier spacing in PSCell (kHz)} | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| 15 | 500 |
| 30 | 250 |
| 60 | 125 |
| 120 | 62.5 |
| 480 | 15.625 |
| 960 | 7.8125 |

<End Of Changes>