3GPP TSG-RAN WG4 Meeting #116	R4-2511157

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	(LTE_NR_DC_enh2-Core) Discussion on SCG activation.

Agenda item:	4.4.2

Document for:	Discussion

Introduction

Earlier in Rel-17, RAN4 has defined UE requirements for SCG activation and deactivation delay. These are captured in section 8.17. The current requirements are as such well-defined. However, during analysis of the feature one aspect was left out which would need to be discussed.

In this paper we look at the SCG activation delay requirements for RACH-based PSCell activation when PSCell is configured in deactivated state, with bfd-and-RLM with no RS in RadioLinkMonirotingConfig but TCI_ActivatedConfig.

Discussion

When looking at the RAN2 description of TCI_ActivatedConfig it states:

| TCI_ActivatedConfig | This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell. |
| --- | --- |

And:

tci-ActivatedConfig If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell. If configured for the PSCell when the SCG is indicated as deactivated in the containing message: - the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD. When this field is absent for the PSCell and the SCG is being deactivated: - the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

Hence, the PSCell can be configured in deactivated state with TCI_ActivatedConfig being configured as well. Once the network activates the PSCell the UE uses the provided TSI states (in the TCI_ActivatedConfig) if not provided at the time of the SCG activation.

If the UE is configured with bfd-and-RLM and TCI_ActivatedConfig is provided, the UE uses the TCI states provided in the TCI_ActivatedConfig – if no RS is configured in RadioLinkMonitoringConfig.

However, it seems that RAN4 is currently not covering the scenario where the UE is configured with a PSCell in deactivated state with TCI_ActivatedConfig when configured with bfd-and-RLM with no RS in RadioLinkMonitoringConfig.

Currently section 8.17.2 states:

-	For RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known, Tsearch = 0 ms. There are no requirements if TCI state is unknown.

And the definition of the TCI state being known:

If the UE is configured to perform bfd-and-RLM while the SCG is deactivated:

-	The TCI state is known if all the following conditions are met:

-	During the period from the reception of the RRC-based SCG deactivation command deactivating the PSCell to the first transmission towards PSCell on PUCCH or PUSCH:

-	UE has not detected beam failure

-	The side condition Ês/Iot ≥ -3 dB is fulfilled for the RSs configured for bfd-and-RLM for the PSCell being activated

-	The SSB measured remains detectable according to the cell identification conditions specified in clause 9.2.

-	Otherwise, the TCI state is unknown.

In the current requirement, this only cover when the UE receives a SCG deactivation command. Meanwhile it does not cover when the SCG/PSCell is configured in deactivated state with TCI_ActivationConfig and bfd-and-RLM configured.

To clarify the RACH-less based PSCell activation delay requirements with Tsearch = 0ms also applies when PSCell is activated after configuration, when the configuration requires the UE to perform bfd-and-RLM with no RS in RadioLinkMonitoringConfig, but with TCI_ActivatedConfig, we suggest following update, highlighted in green, to the existing requirements:

If the UE is configured to perform bfd-and-RLM while the SCG is deactivated:

-	The TCI state is known if all the following conditions are met:

-	During the period from the SCG configuration or reception of the RRC-based SCG deactivation command deactivating the PSCell to the first transmission towards PSCell on PUCCH or PUSCH:

-	UE has not detected beam failure

-	The side condition Ês/Iot ≥ -3 dB is fulfilled for the RSs configured for bfd-and-RLM for the PSCell being activated

-	The SSB measured remains detectable according to the cell identification conditions specified in clause 9.2.

-	Otherwise, the TCI state is unknown.

Update the RAN4 PSCell activation delay requirement capturing the missing scenario.

In [1] we provide a CR capturing the proposal.

Conclusion

In this paper we looked at the SCG activation delay requirements for RACH-based PSCell activation when PSCell is configured in deactivated state, with bfd-and-RLM with no RS in RadioLinkMonirotingConfig but TCI_ActivatedConfig.

- Update the RAN4 PSCell activation delay requirement capturing the missing scenario.

In [1] we provide a CR capturing the proposal.

- References

- R4-2511158, (LTE_NR_DC_enh2-Core) CR on SCG activation, Nokia