3GPP TSG-RAN WG4 Meeting #116	R4-2511196

Bengaluru, India, 25th August – 29th August 2025

Source:	Ericsson, Rogers

Title:	TP for TR 37.719-11-11 adding DC_5A_n7A-n66A

Agenda item:			6.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_5A_n7A-n66A as defined in WID [1].

# ---Start of changes---

## 7.x	DC_66_n5-n7

### 7.x.1	Configurations for DC

Table 7.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC Configuration (NOTE X) |
| --- | --- |
| DC_5A_n7A-n66A | DC_5A_n7A DC_5A_n66A |
| NOTE X: |  |

### 7.x.2	Co-existence analysis for DC

Table 7.x.2-1: Band 5 and Band n7 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2500 | 2570 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1651 | 1746 | 3324 | 3419 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 802 | 922 | 4151 | 4316 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4148 | 4268 | 5824 | 5989 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 98 | 47 | 6651 | 6886 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3302 | 3492 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4972 | 5117 | 8324 | 8559 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6648 | 6838 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9151 | 9456 | 726 | 896 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5802 | 6062 | 2453 | 2668 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10824 | 11129 | 5796 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9148 | 9408 | 7472 | 7687 |

Table 7.x.2-2: Band 5 and Band n66 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1710 | 1780 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 861 | 956 | 2534 | 2629 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 12 | 132 | 2571 | 2736 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3358 | 3478 | 4244 | 4409 |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 692 | 837 | 4281 | 4516 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1722 | 1912 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4182 | 4327 | 5954 | 6189 |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5068 | 5258 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5991 | 6296 | 1516 | 1686 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3432 | 3692 | 873 | 1088 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7664 | 7969 | 5006 | 5176 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6778 | 7038 | 5892 | 6107 |

Based on Table 7.x.2-1 there are 2nd and 3rd IMD products of band 5 and band n66 that may fall into Rx frequencies of band n7.

### 7.x.3	∆TIB and ∆RIB values

Values are derived from DC_5-7_n66.

Table 7.x.3-1: ΔTIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_5_n7-n66 | 0.3 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

Table 7.x.3-2: ΔRIB,c due to EN-DC

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| DC_5_n7-n66 | - | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |  |

### 7.x.4	Analysis of MSD requirements

Based on Table 7.x.2-1 there are 2nd and 3rd IMD products of band 66 and band n5 that may fall into Rx frequencies of band n7.

MSD values are reused from DC_5A-7A_n66A.

Table 7.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n7A-n66A | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n7 | N/A | 5 | N/A | 2624 | 29.0 | IMD21 |
|  | n66 | 1777.5 | 5 | 25 | 2177.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 	 RP-250966. Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia