3GPP TSG-RAN WG4 Meeting #116 	R4-2511211

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item: 7.3.4

Source: MediaTek inc.

Title: Discussion on low-band CA switching

Document for: Discussion

# Introduction

The low band carrier aggregation has been one of the network operators’ interests. The benefit on low band is mainly due to the large coverage compared to that using higher frequencies. Nevertheless, although RAN4 RF spec has already covered the low bands CA band combination, yet this has never been implemented in real field. This is mainly due to challenging task to implement low-band CA on handheld UE especially when there are some combinations has almost no frequency gap between UL transmission and DL reception channels. To overcome such obstacle and to better utilize the low band fragmented spectrums owned by operator, a new WI was approved in RANP#106 [1], in which, 25 companies agreed to introduce low-band CA via switching.

This contribution provides discussion on the low-band CA with the new carrier switching feature.

# Discussion

The main objectives of the work item is to introduce the physical procedures and requirements to enable low band CA via switching, detailed below:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

The PHY procedure as described in the WID objective is that a UE is configured with two low band carriers, where the UE operates on a single carrier at a time. The switching configuration is set by RRC signalling (semi-static). The procedure can be depicted as in Figure 1 below. It should be noted that the PCell of CA is within carrier 1, while the SCell is within carrier 2.

Figure 1: UE carrier switching procedure in low band CA.

RAN4 has already discussed several RRM aspects related to the low band CA operation and the agreements have been captured in the WF [2].

## Discussion on general issues

Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching

Recommended WF - Proposal 1: For defining RRM requirements for SDL SCell operation, it is assumed at least one SSB/TRS is available either during one switching pattern period or one SSB/TRS period depending on the periodicity of switching pattern and SSB/TRS. - Proposal 2: RAN4 to further investigate the RRM requirements impact with at least one of the following methods. - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching. - Proposal 3: No need to consider the extension solution at least for L3 measurement, it can not work under current switching pattern periodicity P = 40 approved by RAN1. If one SMTC occasion is missed, then all SMTC occasions would be missed since typical SMTC configuration contains one or multiple complete switching pattern periodicity. - Proposal 4: For an activated SDL SCell with LBCA-sw, for a serving cell which can be either FDD PCell or SDL SCell: - a. If all the SSB occasions of the serving are contained within the LBCA-sw pattern, no delay extension is allowed - b. If a fraction the SSB occasions of the serving are contained within the LBCA-sw pattern, the delay is extended in accordance to the fraction of available SSB occasions - c. If no SSB occasion of the serving cell is contained within the LBCA-sw pattern, the requirements do no apply - Proposal 5: - Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

In general from RRM spec, measurement period has been extended to compensate a missing sample. Hence, the same principle can be applied in here where extend the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

In addition, for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

- If all the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, no delay extension is allowed;

- If a fraction of the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions;

- If no SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

- Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

- RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if all the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, no delay extension is allowed.

- RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if a fraction of the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

- RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if no SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Issue 1-1-4: others for general part

- Proposal 1: The requirements for LB CA via switching is irrelevant to the UE capability of Rel-18 NES.

It is not clear what would be the spec impact if the above is agreed. Hence, this issue shall be deprioritized because it will not be captured in spec.

- RAN4 shall deprioritize this issue since it will not be captured in the spec.

## Discussion on switching delay issues

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered

RF agreements in RAN4#114: Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

This issue is out of the current WI scope. Also, RAN4 RF has concluded that the combination of more than two carriers is applicable for future releases only. Therefore, RAN4 RRM to close this issue, yet, if the case of more than two carriers is introduced in R19, RRM can revisit this issue.

- RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the R19 WID.

Issue 1-2-3: Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC

Agreement in RAN1 #120bis For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

RAN1 agreement has already considered the switching time, interruption, and scheduling restrictions. Therefore, there is no need for further requirements to be defined in the RRM session. Thus, this issue shall be closed.

- There is no need for further requirements to be defined for the switching time, interruption, and scheduling restrictions, hence, RAN4 RRM to close this issue.

## Discussion on SDL SCell related requirement

Based on agreements from the previous meetings, the summary of LB-CA operation regions can be given as:

- For the SCell which is deactivated: The switching pattern for SDL SCell is not applied.

- For the SCell which is in activation procedure: The switching pattern for SDL SCell is not applied.

- For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation): The switching pattern for SDL SCell is applied.

This can be demonstrated as in the figure below.

Figure 2: LB-CA operation regions, (i) Deactivated SCell, (ii) SCell activation period, and (iii) Activated SCell.

Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

If a fraction of the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are not available based on the LB-CA switching pattern, the delay is scaled in accordance with the fraction of available occasions defined as Kp_LB = Ntotal/Navailable.

- If case b in issue 1-1-3 is agreed, new Kp factor can be used for requirement design. Otherwise, new Kp is not needed.

Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption

Recommended WF - Discuss following points: - Whether to define the requirement for deactivated SDL SCell L3 measurement or not? - Option 1: No (QC) - Option 2: Yes for SSB based operation (OPPO, CATT, Apple, LGE, HW, Ericsson, vivo, MTK, Nokia) - If Yes for above, followings are FFS - Whether and how such measurement shall consider switching pattern and/or meaCycleSCell, e.g., only measure SDL SCC during the period of SDL SCell ON. - Whether to reuse existing corresponding requirement for deactivated SDL SCC measurement - Option 1: Yes (Apple, MTK) - Option 2: No - Whether and how to use MG for deactivated SDL SCC measurement (LGE, Nokia). - Whether and how to define the corresponding interruption requirement for this measurement. - Option1 (Apple, HW): If SSB-based SCell operation is assumed for SDL SCell measurement, for deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell. - Option 2 (Ericsson): interruption on PCell or activated SCell(s) are allowed with up to X % probability of missed ACK/NACK when measCycleSCell is below threshold 1 and Y% otherwise. Y can be 0.

Provided that this new feature is to enable CA band combo rather than enhancing existing SCell operation, then RAN4 shall define requirements for deactivated SDL SCell L3 measurement. In addition, provided that the UE only applies the pattern during activated SCell, hence, during deactivated SCell the UE behavior is based on activating RF chain to perform measurements and hence the UE is expected to cause interruption during the measurements. Thus, the existing requirements can be applied for interruption before and after measurements, yet additional scheduling restrictions should be applied during the measurement time. Furthermore, regarding whether to use measurement gaps, it should be noted that MG interrupts all carriers for the UE during the measurements. Although, the current scenario is focusing on two carriers only yet there is ongoing discussion to extend the case to three carriers. Now, given that MG interrupts all carriers then when the scenario includes three carriers then all the three carriers are interrupted. Therefore, our preference is to have this feature covered by interruption and scheduling restriction during SMTC/SSB period.

- RAN4 shall define requirements for deactivated SDL SCell L3 measurement.

- Existing interruption requirements based on meaCycleSCell can be applied with additional scheduling restrictions to cover the SSB/SMTC period.

- MG interrupts all CC, while the scenario that we have in here should interrupt PCell only. Hence, there is no need to extend using MG to this scenario.

Issue 1-3-7: SDL SCell activation/deactivation delay requirement

Recommended WF - Agreement: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation. - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern.

Although in legacy SCell activation requirements, a UE is required to report OOR CQI and lowest L1-RSRP, where such reporting is sent over PCell. Yet, for LB-CA the UE might not be able to send such reporting due to the collision between SSB/SMTC and the OOR CQI reports. To address this, the RRM group to make judgement based on the necessity of sending OOR CQI versus the SSB/SMTC occasions. Given that the UE needs to continue the measurements to activate SDL, then the UE should perform measurements and drop OOR CQI. Nevertheless, the prioritization during the collision scenario might result in additional UE complexities, therefore, we prefer to not consider the collision case, i.e. no requirements if colliding occurs.

- When CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation, RAN4 not consider the colliding case, i.e., no requirement if colliding occurs.

Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation

Recommended WF - Proposal 1 (OPPO, CATT(scheduling restriction), Nokia): - RAN4 to define the scheduling restriction or interruption caused by the measurements during SDL SCell activation procedure. - Proposal 2 (Apple): - the interruption pattern due to CC switching in SDL SCell activation shall follow SSB periodicity or SMTC periodicity used for SCell activation. - the interruption length for each CC switching in SDL SCell activation can be defined as {(switching duration for FDD-to-SDL) + SSB/SMTC duration + (switching time for FDD-to-SDL)} or reuse same interruption requirement from legacy FR1 intra-band SCell activation. - Proposal 3 (LGE): - RAN4 to specify SCell activation window which ensures SCell measurements of the deactivated SCell. - Proposal 4 (HW): - During SDL SCell activation, if the to-be-used RS resource is SSB: - Option 1: UE stays on SDL carrier until the SCell activation procedure is completed. The whole SCell activation delay would be specified as interruption or scheduling restriction. It shall be noted that UE shall also wait for one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - Option 2: UE switches to SDL carrier according to SSB occasions. The interruption or scheduling restriction would happen on SSB symbols and 1 symbol before each consecutive SSB symbols to be measured and 1 symbol after each consecutive SSB symbols to be measured. It shall be noted that UE shall also measure one CSI-RS resource for CQI. And a valid CQI will be transmitted on PCC. - FFS if the to-be-used RS is A-TRS or P-TRS. - Proposal 5 (ZTE): - RAN4 needs to consider the switching period/interruption during the SDL SCell activation procedure for the switching between PCell and SDL SCell L3/L1 measurement

As shown in Figure 2, the UE is expected to measure on SDL while operating on PCell. Given that the UE is can only operate on either PCell or SDL it is expected that the UE cause interruption on the main CC, i.e. PCell, while measuring on SDL. Interrupting a single CC in RRM requirements wording is given by scheduling restrictions. Therefore, scheduling restrictions requirements shall be defined for the measurements during SDL SCell activation procedure.

In addition, the scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation:

- the scheduling restriction length for each CC switching in SDL SCell activation can be defined as (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD).

  - Following RAN1 agreement, then:

  - PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD), will have scheduling restriction.

- RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

- The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation: the scheduling restriction length for each CC switching in SDL SCell activation can be defined as (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD). PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD), will have scheduling restriction.

Issue 1-3-10: BFD/CBD requirements for SDL SCell

Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

RAN4 discussed on whether to develop requirements for the scenarios of BFD/CBD, yet there was no consensus on whether to define requirements for BFD/CBD and hence the ‘IF’ letter in the agreement above. Given that this WI focusing on FR1 and mainly on sub 1GHz carriers, which means there is no beam forming mechanism, hence it is preferable to not define new requirements for BFD/CBD for LB-CA WI.

- RAN4 shall not define requirements for BFD/CBD for SDL SCell in LB CA via switching.

# Summary

In summary:

Proposal 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

Proposal 2: RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if all the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, no delay extension is allowed.

Proposal 3: RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if a fraction of the SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

Proposal 4: RAN4 RRM shall adjust the measurement requirements for gapless L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell that if no SSB occasions in SMTC window or CSI-RS for L3 measurement or SSB occasions or CSI-RS for L1 measurement are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 5: RAN4 shall deprioritize this issue since it will not be captured in the spec.

Proposal 6: RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the R19 WID.

Proposal 7: There is no need for further requirements to be defined for the switching time, interruption, and scheduling restrictions, hence, RAN4 RRM to close this issue.

Proposal 8: If case b in issue 1-1-3 is agreed, new Kp factor can be used for requirement design. Otherwise, new Kp is not needed.

Proposal 9:RAN4 shall define requirements for deactivated SDL SCell L3 measurement.

Proposal 10:Existing interruption requirements based on meaCycleSCell can be applied with additional scheduling restrictions to cover the SSB/SMTC period.

Proposal 11:MG interrupts all CC, while the scenario that we have in here should interrupt PCell only. Hence, there is no need to extend using MG to this scenario.

Proposal 12:When CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation, RAN4 not consider the colliding case, i.e., no requirement if colliding occurs.

Proposal 13:RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

Proposal 14: The scheduling restriction due to CC switching in SDL SCell activation shall follow RS periodicity or SMTC periodicity used for SCell activation: the scheduling restriction length for each CC switching in SDL SCell activation can be defined as (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD). PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot and PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of (symbol(s) for switching time of FDD-to-SDL)+ SSB/SMTC duration/CSI-RS + (symbol(s) for switching time of SDL-to-FDD), will have scheduling restriction

Proposal 15: RAN4 shall not define requirements for BFD/CBD for SDL SCell in LB CA via switching.

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2504895, “WF on RRM requirements for NR_LBCA_Sw”, Apple, Wuhan, China, April 2025.

- R4-2503614, “Topic summary for [114bis][204] NR_LBCA_Sw”, Apple, Wuhan, China, April 2025.