3GPP TSG-RAN WG4 Meeting #116 	R4-2511215

Bengaluru, IN, August 25th – 29th, 2025

Agenda Item: 7.21.5

Source: MediaTek inc.

Title: Discussion on SBFD Performance

Document for: Discussion

# Introduction

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the performance part issues captured in the WF.

# Discussion on performance part

# Discussion on accuracy requirements

Issue 3-2-1: L1-SRS-RSRP - side condition on time offset:

The discussion on time offset between DL timing and SRS arrival timing included two options, where the first option is use R16 requirements as baseline, i.e. with cell phase error. While Option 2 is based on removing the cell phase error, resulting in better accuracy requirements. It should be mentioned that Option 2 is only applicable for intra-cell CLI measurements, also, it is worth highlighting that the UE is not aware of whether this measurement is intra-cell CLI or inter-cell CLI.

Our preference is that RAN4 RRM to define a single set of test cases for a specific system parameter, i.e. not to define two sets of test cases for the same parameters, which are with and without cell phase error.

- RAN4 RRM to define test cases either with or without cell phase error but not for both for the same set of parameters.

Issue 3-2-2: L1-SRS-RSRP - side condition on SRS BW:

The discussion on frequency resources should follow the common deployment scenario where a given subband has a BW of approximately 20MHz, hence, the number of PRB should be 48PRBs.

- RAN4 RRM to define side condition on SRS BW to be 48PRBs, same as in R16.

Issue 3-2-3: L1-SRS-RSRP - side condition on SRS Es/Iot:

Regarding the side condition of SRS Es/IoT, reusing R16 CLI SRS RSRP assumption seems to be sufficient, which is ≥1 dB.

- RAN4 RRM to define side condition on SRS Es/Iot to be ≥1 dB, same as in R16.

Issue 3-2-4: L1-SRS-RSRP - side condition on accuracy requirements:

Follow the same conclusion of issue 3-2-1 on time offset, i.e. for a given set of system parameters the test cases to be defined for either intra-cell or inter-cell (not for both).

- For a given set of system parameters, RAN4 to define TCs for either intra-cell or inter-cell (not for both).

Issue 3-2-5: L1-SRS-RSSI - side condition on accuracy requirements:

- Recommended WF - Suggest to agree the following: - RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. - FFS on how to define the requirements - Option 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Option 2: other

Recommended WF from the previous meeting could be used as a starting point. Therefore, RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. Also, re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

- RAN4 to define accuracy requirements for L1-CLI-RSSI measurements.

- Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and type#2, as well as for Method#3.

# Discussion on test cases list

Issue: Test case list:

The following RRM core requirements are impacted due to the work on SBFD:

- L1 CLI measurement: test cases are needed to verify the new defined CLI measurements.

  - L1-SRS-RSRP measurements

  - L1-CLI-RSSI measurements

- RLM/BFD/CBD requirements: test cases are needed to verify the collision between measurement resources and UL transmission.

- L1 SINR and L1 RSRP measurements and reporting: test cases are needed to verify the collision between measurement resources and UL transmission.

Therefore, test cases should be defined for the following topics:

- L1-SRS-RSRP measurement (delay and accuracy)

- L1-CLI-RSSI measurement (delay and accuracy)

- CSI-RS RLM

- CSI-RS BFR

- CSI-RS L1-RSRP measurement (delay)

- RAN4 RRM to define test cases for the following topics: (i) L1-SRS-RSRP measurement (delay and accuracy), (ii) L1-CLI-RSSI measurement (delay and accuracy), (iii) CSI-RS RLM, (iv) CSI-RS BFR, and (v) CSI-RS L1-RSRP measurement (delay).

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM to define test cases either with or without cell phase error but not for both for the same set of parameters.

Proposal 2: RAN4 RRM to define side condition on SRS BW to be 48PRBs, same as in R16.

Proposal 3: RAN4 RRM to define side condition on SRS Es/Iot to be ≥1 dB, same as in R16.

Proposal 4: For a given set of system parameters, RAN4 to define TCs for either intra-cell or inter-cell (not for both).

Proposal 5: RAN4 to define accuracy requirements for L1-CLI-RSSI measurements.

Proposal 6: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and type#2, as well as for Method#3.

Proposal 7: RAN4 RRM to define test cases for the following topics: (i) L1-SRS-RSRP measurement (delay and accuracy), (ii) L1-CLI-RSSI measurement (delay and accuracy), (iii) CSI-RS RLM, (iv) CSI-RS BFR, and (v) CSI-RS L1-RSRP measurement (delay).

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.

- R4-2505568, “Topic summary for [115][219] NR_duplex_evo”, Huawei, HiSilicon, Malta, May 2025.