# 3GPP TSG-RAN WG4 Meeting #116	R4-2511227

# Bengaluru, India, 25th – 29th of August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS 36.102 Clause 6.4B - IoT_NTN_TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | THALES, Iridium Satellite LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add band 249 frequency error requirement as per approved IoT_NTN_TDD WID, RP-242415 (with revised WID approved in RP-243293). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.4B Transmit signal quality (and more precisely Clause 6.4B.1 Frequency error for UE category NB1 and NB2) for category NB1 and NB2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Frequency error requirement for band 249 will not be properly introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ----------------------------------

## 6.4B	Transmit signal quality for category NB1 and NB2

### 6.4B.1	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331 [6]), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 [8] clause 23.21.2.2).

The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.

When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 in which UE is allowed to perform pre-compensation at the beginning of the uplink burst of 8 consecutive transmitted subframes. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift corresponding to the estimated Doppler frequency at the beginning of the transmission.]

Table 6.4B.1-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

### 6.4B.2	Transmit modulation quality for Category NB1 and NB2

Transmit modulation quality requirements for Category NB1 and NB2 UEs for BPSK and QPSK modulation as specified in clause 6.5.2F of 36.101 [7] are applicable.

------------------------------ End of modified section -------------------------